# Chapter 8

# Observations and status of peatland greenhouse gas emissions in Europe

Matthias Drösler1*, Annette Freibauer2, Torben R. Christensen3 & Thomas Friborg4,

1Vegetation Ecology, Department of Ecology, Technical University of Munich, Freising, Germany

2Max-Planck-Institute for Biogeochemistry, Jena, Germany

3GeoBiosphere Science Centre, Physical Geography and Ecosystems Analysis, Lund University, Sweden

4Department of Geography and Geology, University of Copenhagen, Denmark.

8.1 Introduction

A peatland is a type of ecosystem where carbon along with nitrogen and several other elements has been accumulated as peat originating from the plant litter deposited on the site. A logical consequence of the above definition of peatlands is that they are ecosystems, which by way of nature are a sink for atmospheric carbon dioxide (CO2). This is the case because more carbon is accumulated through photosynthesis than is released through respiration. As a consequence of this organic matter accumulates as peat. The carbon accumulated in peatlands is equivalent to almost half the total atmospheric content and a hypothetical sudden release would result in an instantaneous 50% increase in atmospheric CO2. While this scenario is unrealistic it nevertheless highlights the central role of peatlands where huge amounts of CO2 have almost entirely been “consumed” since the last glacial maximum, but could respond differently as a result of future changes in climatic conditions. Peatlands have, hence, over the past 10000 years helped to remove significant amounts of CO2 from the atmosphere.

A complicating factor in this respect is that in terms of the major greenhouse gases (GHGs) peatlands are not only acting as a sink for CO2. The wet conditions that lead to the slow decomposition of organic material, and enable peat accumulation to occur, also cause significant amounts of the powerful greenhouse gas methane (CH4) to be formed. Indeed global wetlands (predominantly peatlands) are considered to be the largest single source of atmospheric CH4 also when considering all anthropogenic emissions. Peatlands are therefore also a key player in the atmospheric CH4 budget and as a result also influence the global climate.

## 8.2 Greenhouse gas fluxes in European peatlands

### 8.2.1 Peatland areas in Europe

Peatlands cover 5-6 % of the European continental land surface. They are concentrated in northern and temperate lowlands where water drainage is impeded, precipitation is high and/or low temperatures limit evapotranspiration. Joosten and Clarke (2002) give a good overview of the peatland distribution in Europe. The numbers used in this chapter are amended with national data where available. Russia contains half of the European peatlands. In EU-27, most peatlands are found in Scandinavia, UK, Ireland and the young pleistocene landscapes of Germany and Poland. Half of the peatlands on the European continent are subject to various sorts of land use, which is often associated with drainage. 20 % are drained for forestry, mainly in Scandinavia and UK, 16 % are drained for agriculture, mainly in Germany, Poland and the Netherlands, and 0.5 % is used for peat extraction. Restoration of abandoned peatlands is an emerging issue but still negligible in terms of areal coverage. The fractions of the various peatland uses greatly vary among countries and regions (Figure 1).

### 8.2.2 Peatland C stocks

Despite the small surface cover, peatlands hold large C stocks due to the high organic matter content, which often reaches down to several meters of peat accumulation. Estimates of the European peatlands C stocks are limited by unknown peat depth, peat bulk density and C fractions in the profiles. Byrne et al. (2004) estimated the C storage in European peatlands to 42 Pg C but with a high degree of uncertainty. The estimate relied on areal estimates by Joosten and Clarke (2002). The peat resources in volume or mass reported in Lappalainen (1996) were converted into C storage estimates based on the average dry bulk density of 91 g dm-3 (Mäkilä, 1994) and C concentration of 51.7% (Gorham, 1991). The depth estimates were based on Lappalainen (1996) where available. However, for most countries the average peat depths were not available and a conservative estimate of 1.75 m was used as the base of volume calculations.

Despite these uncertainties, peatlands are those European ecosystems with the highest carbon storage per area, which for example is on average two to three times higher than the ecosystem C stock in forests.

### 8.2.3 Typical greenhouse gas flux rates

In general, natural mires take up CO2, as part of photosynthesis, and release a part of the carbon as CO2 and CH4 to the atmosphere during microbial decomposition. Natural mires are neutral with respect to N2O. In contrast, drainage of peatlands results in the release of CO2 and sometimes N2O while CH4 emissions cease or are even changed to small CH4 uptake. Greenhouse gas (GHG) fluxes from peatlands are much more variable in space and time than GHG fluxes in upland ecosystems. Measured emission data are limited in number and rarely cover one or more full years. Nevertheless, a first attempt was made to classify peatlands and calculate typical annual GHG flux rates for the European peatlands. At the first level of classification, bogs (ombrogenic peatlands) were separated from fens (minerogenic peatlands). Bogs and fens were further distinguished according to the following categories:

- Forestry (drained)

- Fresh drainage

- Grassland (drained)

- Cropland (drained)

- Peat cut (drained)

- Abandoned (drained)

- Restored (rewetted)

- Mires (natural)

Drained and abandoned peat cut areas are only relevant for bogs.

A literature review was performed to gather all available information on measured GHG fluxes and on calculated regional average and typical GHG fluxes (Minkkinen and Laine 1998, Sundh et al. 2000, Kasimir-Klemedtsson et al. 1997, Nilsson et al. 2001). The full set of references is given in Byrne et al. (2004). The available data are strongly biased towards Finland, and unequally distributed through the categories listed above (Figure 2). Figure 2 demonstrates the shortcomings in the presently available data sets. CO2 and CH4 have been studied in some breadth, while N2O studies focused on managed fens. Few studies have measured the full GHG balance. Data about the C balance in arable, abandoned and restored peatlands and fen mires are particularly scarce. Therefore, the values shown in Figure 2 can only be seen as indicative orders of magnitude and may not necessarily be representative of European peatlands. In order to avoid bias produced by few exceptionally large measured fluxes within a category, the median of all available data was taken to represent typical GHG flux rates in Figure 2.

In Figure 2, the GHG flux rates for forestry were mainly derived from Finnish data sets, where drainage for forestry is moderate and water-tables are still close to natural conditions, in particular in young stands. Therefore, the calculated emission factors for peatlands drained for forestry may not be representative for more intensively managed forests. Grasslands dominate on more fertile peatlands and are sometimes deeply drained and fertilized. The wide range in literature values reflects a huge diversity in management practice in grasslands, from virtually no drainage in extensively grazed grasslands in Ireland and the UK to deep drainage in Germany. Croplands are the deepest drained, most intensively used, fertilized and ploughed, most disturbed, most fertile sites with strong peat degradation. Peat cut areas on bogs are small, but very variable, depending on treatment prior to and after peat extraction. The peat C loss from removal is not considered here, but only the flux rates on-site. Peatland restoration represents a new development due to peatland conservation programs. Still marginal areas are covered and very little data is available. While peatland restoration is an emerging area of GHG research, GHG fluxes on natural mires rely on many, however mostly seasonal, data from Northern bogs, oligotrophic and mesotrophic mires. There are no data for natural temperate fens.

Taking the three GHGs together, peatlands in the categories of natural mires, restoration and forestry are close to climate neutral while all other managed peatlands are significant sources of GHGs. As shown in Figure 3, the net climate effect calculated by the global warming potentials of the three gases used under the UNFCCC (GWP100, IPCC 1996) is closely related to the net C balance of peatlands. We followed this classical GWP-approach, as for policy related expression of the net-climate effect it is still the state of the art (Frolking et al., 2006). The peatland classes roughly follow the 1:1 line of C balance versus GWP. Deviations from this line originate from significant CH4 emissions in natural peatlands, which account for 30% of the NEE, and/or N2O emissions from drained agricultural peatlands. The GWP is typically stronger for fens than for bogs. All peatlands with a strong C source are also strong GWP sources, and all of them are managed (Figure 3, numbers 6-14). In contrast, natural peatlands and those only marginally drained for forestry are relatively balanced with regard to C and GWP (Figure 3, numbers 1-5), with a small exception of natural fens (Figure 3, number 5) where CH4 emissions dominate the GWP. Peatlands are not only hotspots of C storage, but also of GHG emissions, in particular when drained.

Figure 4 shows a conservative estimate for the peatland GHG budget of the top 10 peat countries in Europe based on typical GHG flux rates per land use class in Figure 2. Due to lack of adequate area statistics, recent drainage for forestry or peat cut, abandoned peat cut, and restoration could not be calculated. Whilst Finland, Sweden and UK turn out as CO2 sinks, all countries are net GHG emitters from peatlands.

The results of Figure 4 indicate some interesting features. While mires and forestry dominate the peatland areas (Figure 1), grasslands and croplands dominate the emissions. Russia contributes most to the European peatland GHG balance, which is equivalent to its share in peatland area. Germany turns out as the second-largest emitter (12% of European total) although it contains only 3.2% of the European peatlands. The reason is the intensive cropland and grassland use of most of the peatland area. Clearly, land use intensity disconnects the national GHG balance (Figure 4) from the peat area (Figure 1).

The results in Figure 4 are associated with large uncertainties, which cannot be adequately quantified yet. One of these is the distribution of land use types, particularly in Russia, the largest European peat nation. Sensitivity tests of some crucial assumptions give a rough order of magnitude of possibly higher or lower typical GHG emissions. The reality is likely to lie in between the extremes. The average GHG flux rates for forestry indicate that the peat forests have been drained only slightly, if at all. However, the area data indicate the areas of drained forest where drainage is likely to be repeated periodically. A first sensitivity test (Table 1) uses an alternative GHG flux rate for “drainage” (Figure 2). This alternative changes forestry from a small GHG sink to a significant GHG source and increases the overall peatland GHG balance in Europe by 25-50%. The fractional cover of cropland and grassland within agricultural use is highly uncertain. The default represents the best possible estimate by expert judgment. In two extreme scenarios, we assume 1) 100% cropland, and 2) 100% grassland. Consequently, GHG emissions from agricultural peatland change by 15-20%. This alters the European peatland GHG estimate by 5% only (Table 1). The uncertainty in the European peatland GHG balance resulting from the combined uncertainty in land use fractions and typical GHG flux rates is estimated between -10% and +50% of the default estimate of 52 Gg C-equivalents.

## 8.3 Processes of GHG sources and sinks in peatlands

The principal functioning of peatlands in their natural state is the accumulation of Carbon, with the consequence of a long-term growing peat-profile. Carbon accumulation results from the gap between gross primary production (GPP), ecosystem respiration (Reco), emissions of CH4 and losses via dissolved organic and inorganic carbon (DOC, DIC) and particulate organic carbon (POC). From the functional aspect of peat-growth, the Carbon-balance is the key factor. Classically the long term average Carbon balance is assessed via 14C dating of the profile together with the determination of the corresponding bulk density and the carbon content. Long term average carbon uptake rates (LORCA) in northern peatlands are around 25 g C m-2 a-1. However for the assessment of the climatic relevance of the peatlands the recent GHG-exchange is more important than the Carbon-balance. Specifically in peatlands not only the fluxes of CO2 but as well the fluxes of CH4 and N2O have to be assessed. All three gases together, multiplied with their individual GWP, determine the climatic forcing of the sites.

The dominating factor for the sensitive balance between uptake and release of Carbon and as well for the exchange of the three different gases within the system is the water-table: Undisturbed peatlands hold water-tables close to the surface as the specific vegetation-types together with the peat-characteristics together with a net surplus of water keep the water-level high. Therefore aerobic conditions in an undisturbed peat profile are limited to a shallow upper layer (acrotelm). Below the mean water-table, in the catotelm, the lack of oxygen creates anaerobic conditions. The thickness of the upper aerobic zone is of major importance for the gas-fluxes:

1.	The speed of respiration processes is an order of magnitude faster in aerobic than anaerobic conditions. The product of aerobic respiration in the acrotelm is CO2, produced by soil respiration and autotrophic respiration of the belowground plant parts. Anaerobic respiration by methanogenic bacteria on the other hand, takes place in the catotelm and leads to CH4-formation. Consequently the decomposition of the organic matter in the peat profile is slowed down, once the material enters the water saturated zone. Classically in an undisturbed peatland the decomposition is slower than the supply with fresh material through vegetation growth and decay. Therefore undisturbed peatlands sequester Carbon in the peat profile. However, if the water table drops down, two effects provoke rising respiration emissions: The zone in the peat-profile for aerobic respiration is thicker with the consequence of rising soil respiration and the vegetation at the surface, which requires normally high water-tables, starts to decay. Both effects together provoke rising Reco values, with the potential to shift the Carbon sink to a carbon source.

2.	Methane is formed in the anaerobic zone of the peat profile, specifically in the water saturated rooting zone of the wetland-plants. Here fresh carbohydrates and H2/ CO2 are available in the rhizosphere and the lack of oxygen creates the conditions for the methanogenic bacteria to produce CH4. However, the quantity of methane which leaves the ecosystem to the atmosphere depends on several factors, among which the aerated zone seems to be crucial: Methane entering the acrotelm is oxidized by methanotrophic bacteria to CO2. The efficiency of the oxidation of Methane is therefore mainly dependant on the thickness of the upper aerated layer (acrotelm). A mean water-table of -10 cm seems to be a critical level for switching on or of the methane emissions. This shallow aerated layer allows the methanotrophic bacteria to efficiently oxidize methane to CO2 (Nykänen et al. 1998, Christensen et al. 2003, Freibauer et al. in prep.). Together with diffusion, two other processes determine the amount of methane released from the peat profile, i.e. plant mediated transport and ebullition. Plant mediated transport takes place, once helophytes with aerenchyma are present and the water-table is higher than the rooting zone. Ebullition is predominantly a phenomenon of water-logged peatland-sites, and appears more stochastically than the other two emission pathways. It is difficult, however, to determine the relation of the different emission processes. But water-table plays for all of them a vital role as controlling variable.

3. 	The water-table close to the surface is, together with nutrients and pH-values, the key factor for the development and the maintenance of the typical peat-forming vegetation types. Vegetation composition reacts sensitively to water-level changes both by the shift of dominance structures of the different species as by the vitality of the vegetation layer.

### 8.3.1 Typical situations with CO2 sources and sinks

Natural peatlands are the only terrestrial ecosystems which act as continuous and long term CO2 sinks. Peat growth models (Clymo, 1984) predict a decrease of uptake rates till an equilibrium state between decomposition and sequestration because with rising peat depth decomposition takes place in rising volumes of the peat profile. However, recent studies of Carbon exchange (Lafleur 2003, Droesler, 2005) showed that even old growth mires with peat deposits of up to ten meters depth and development times since the last glaciations demonstrate active uptake rates, which are higher than the LORCA. The reason for this discrepancy may be, that the peat growth models underestimate the decrease of the decomposition rates with depth. Hence, gas-exchange studies hint to the crucial role of the activity of the top layer (acrotelm). Here again the water-table is the dominating factor, both for the activity and vitality of the vegetation as for the volume of peat where aerobic decomposition takes place. A dropping water-table can be non-permanent because of dry weather situations or permanent of human interference (drainage). Alm et al. (1999) showed that a peatland can lose in a single dry summer the amount of Carbon stored in five years of average accumulation. So the sensitive balance between uptake and release was disturbed with the consequence of a part-time shift to a CO2-source (see Figure 5). However, if the vegetation is not damaged, the system can switch back to a sink in the following year with favorable climate conditions. Severely drained peatlands are typically permanent CO2 sources because of the deep aerated zone together with the shift or removal of the vegetation cover on top (see Figure 5) . Even if very active grasses or crops are grown after drainage, they can (normally) not hold against the rising soil respiration, thus converting the system to a CO2 source. Restoration via elevating the water-table has short term effects on the reduction of the soil respiration and mid to long term effects in creating the appropriate site conditions for the development of typical peatland vegetation types. Hence almost immediately after the elevation of the water-table, the CO2-balance changes due to a decreasing heterotrophic respiration (Rh). However, the direction and magnitude of the CO2-exchange depends principally on the GPP-part of the fluxes (see Figure 5). The time till a new equilibrium in CO2-exchange depends on the development speed of the vegetation and can take several years to decades. This development is triggered by the water-table (high or flooded), the type of start-vegetation before restoration and potential engineering practices, like the introduction of matrix or target species. Principally restoration leads instantly to reduced CO2-emissions (lower source strength) in comparison to drained sites. The time till the sink-function will be restored is up to now not generally predictable, as only very few studies are available on that topic. First results from fen-peatlands in the Netherlands (Hendriks et al. submitted) confirmed a strong CO2-sink (uptake of 270 g CO2-C m-2 a-1) 10 years after restoration of a peatland-meadow. Similarly restoration in the Donauried-fen (southern Germany; Bergmann et al. in prep.) lead after 25 years to CO2-sinks (uptake of 109 – 142 g CO2-C m-2 a-1). In a southern German bog however, 12 years after restoration of former peat cut areas, the sites were still acting as CO2-sources (release of 62 – 126 g CO2-C m-2 a-1; Droesler 2005).

### 8.3.2 Typical situations when high CH4 emissions occur

Based on the outlined processes for methane formation and emission, Figure 5 visualizes typical situations when high CH4 emissions occur, in relation to the water- table position. Natural sites as well as restored ones normally act as significant CH4-sources. Most problematic, according to recent studies seem to be flooded sites with emission rates up to 205 g CH4-C in a restored north-eastern German fen-peatland (Augustin et al. in prep.). Both the lack of an aerobic layer for methane oxidation as the decay of the former grassland-vegetation under anaerobic conditions lead to these huge CH4 emissions. So restoration without flooding, oriented at the critical mean water-level of around -10 cm provoke much lesser methane emissions (see Figure 5). In the southern German Donauried e.g., the difference of -1.5 vs. -6.5 cm of water-table was reflected in significantly different methane emissions of 77 vs. 13 g CH4-C m-2 a-1 (Freibauer et al. in prep.). Very important for the methane emission seems to be as well the period of flooding, when temperature is rising in the year course and the plants are entering in the growing season. Here rising soil temperatures and the supply of fresh substrates in the rhizosphere, create favorable conditions for methanogenesis. NEE seems therefore to be a good predictor for CH4 emission as exemplarily shown for NE Greenland (Joabsson & Christensen 2001) and southern Germany (Droesler 2005). But as the wetland plants with high NEE rates are normally sedges with aerenchyma, the combination of the by-pass effect with the fresh substrate supply may be responsible for the high methane emission rates.

Methane uptake, however, is a phenomenon limited to drained peatland soils (see Figure 5). In peatlands, the importance for the Carbon balance of this small uptake pathway (limited to ~ 0.3 g CH4-C; Jungkunst, unpubl.) is limited, because these sites are big emitters for CO2, with rates several orders of magnitude higher than the uptake of methane.

### 8.3.3 Typical situations when high N2O emissions occur

N2O is formed via Nitrification and Denitrification. Principally, bog-peatlands show very small emissions because of their low nutrient contents. Fens, however, can have significant N2O emissions under conditions, which are not too wet (natural or restored) and not too dry (severely drained sites). In case of N2O, Figure 5 shows therefore the behavior for a fen peatland.

### 8.4 Emission patterns

8.4.1. Spatial emission patterns

Peatlands are hotspots for GHG-emissions in the landscape. The European continental average GHG flux rate is 1 g C-equ m-2 y-1. Russia has average GHG flux rates of 0.23 g C-equ m-2 y-1, but Germany has 3.1 g C-equ m-2 y-1. The estimated net CO2-emissions (3.2-5.1 Tg C a-1) from German peatlands under current landuse compensate for the recent Carbon sink (5.2 Tg C a-1) in German forests (Benndorf pers. comm. 2005), while German Peatlands hold with 16.520 km2 just 15 % of the forest area.

At a smaller scale, hotspots can be identified within the peatlands, according to the spatial representation of the different driving factors. As shown before, the parameters water-table, peat characteristics and vegetation-type and –cover dominate the control over the GHG-emissions in peatlands. These parameters are partly inter-correlated (like water-table and vegetation type). However, the overlay of these parameters creates small scale patterns of changing site conditions. This small scale variability of site conditions is not only a phenomenon of disturbed peatlands. Even in natural peatlands, depending on the peatland type, microsite variability is high. Natural bogs e.g. show normally complex patterns of hummocks, lawns and hollows, all of them colonized by a specific plant composition, sensitively reacting to the water-table level and fluctuation. Therefore Bubier et al. (1995) was successful to extrapolate methane emission rates from the representation of distinct Sphagnum species as indicators for the mean water-table.

In peatlands with a former landuse history, the variability of the site factors can even be more pronounced. Drainage and especially peat cutting activities lead to severe changes in the site conditions, which create mostly sharp gradients and small scale patterns of site factors. Site management like restoration via water-table manipulation creates even more pronounced small scale site differentiation, depending on the former microtopography and the magnitude of restoration works.

So the small scale variability in peatlands, mostly driven by very small differences in the mean water-table and the vegetation composition, creates an individual small scale pattern of GHG-exchange behavior at scales of m2. Hotspots can be direct neighbors of balanced sites. The sites of the schematic Figure 5 can all be represented within a small area of less than one ha. This implies the necessity, to be able to distinctively measure the individual gas-exchange of the different sites, applying techniques with sufficient spatial resolution.

8.4.2. Temporal emission patterns

In terms of temporal emission patterns different sites show individual reactions to the driving variables. For the temporal variability over whole year cycles, Droesler (2005) showed e.g. in southern German bogs that the daily NEE of natural bogs is negative (uptake to the system) over almost the whole growing period, whereas degraded former peat cut sites where sinks just on single days over the same period and demonstrated much higher winter fluxes. Restored sites behave in between, with uptake to the system in spring but a shift back to sources during mid and late summer. These differences could be attributed to the dynamic of the water-table and to the vegetation cover.

Temporal emission pattern in CH4 is mostly reflecting the temperature dynamics, as shown e.g. in Bellisario et al. (1999). Therefore these emissions follow generally a seasonal curve. However, the functional link between NEE and Methane-Emission hints to additional underlying processes, which at the natural sites go parallel with the temperature curve through the year. Here the detection of the dominating factor is not possible in field studies.

Emission dynamics in N2O, however, do not follow a simple seasonal temperature or radiation driven curve, like e.g. in methane emissions or in Reco or GPP. Notable amounts of N2O are leaving the system in occasions of frost and thaw or after fertilizer applications. Therefore, the modeling of these fluxes is not a simple topic. Reth et al (2005) found in forest and grassland ecosystems the time since the last rain event as a critical parameter for the prediction of the emissions. The applicability in peatlands is still to be tested, in view of the importance of water-table fluctuations for the N2O emission rates.

As weather is changing from year to year, the interannual dynamics can be significant. This is especially true for CO2, because NEE is just the small difference between the big terms of GPP and Reco. Lafleur et al. (2003) showed the span of yearly NEE-sums for consecutive measurement years in a Canadian bog ecosystem. Single year studies should therefore be interpreted with caution in their absolute numbers, but can definitely allow for the comparison of the behavior of different sites, measured within the same year.

## 8.5 State-of-the-art and current capabilities of GHG measurements in peatlands

Carbon stocks in peatlands are very big in relation to annual changes of the carbon-content. Therefore for the assessment of the climatic relevance, the measurement of the GHG-gas exchange is the only viable approach. Especially in peatlands all three climatic relevant trace-gases have to be monitored to get full GHG-balances of the sites. Together with the above outlined topics of temporal and spatial variability, the requirements for the applied measurement techniques are:

- high spatial resolution to cope with small scale mosaics of site conditions and hence spatial emission patterns (specifically needed for the detection of hotspots and the influence of management upon the fluxes!)

- potential to be applied under all climate conditions for the development of full year balances

- high temporal resolution to cover the sensitive times for the different gases

- in remote conditions (like peatlands normally are) independency from power line connection

A single technique is not available which can support all of the above mentioned requirements. Therefore the combination of different techniques seems to be appropriate to fully cover the specific needs for GHG-exchange measurements in peatlands. A general comparison of the characteristics of the different established methods can help to choose the best combination according to the specific needs in a given environment (see Table 2).

The combination of eddy covariance for CO2 and manual chambers for CH4 exchange monitoring is currently successfully applied e.g. in a restored peatland meadow in the Netherlands (Hendriks et al. submitted). Eddy covariance provides the overall integrated signal of the different underlying small scale ecosystems (dry land, saturated land and ditches). With the chambers, the individual contribution of these subsystems for methane emissions is assessed. Ebullition of methane from water bodies, however, is difficult to measure with chambers, as these fluxes appear stochastically and may need a continuous measurement system to be detected realistically.

In typical peatlands small scale mosaics of sites, it is of interest, to differentiate the CO2-exchange for the individual sites (combination of vegetation and management, water-table and peat-characteristics) to better understand the underlying processes of NEE. Climate controlled chambers can help out and allow, e.g. to directly link methane emissions to NEE, once the fluxes are measured at the same plot. Those climate controlled transparent NEE-chambers, e.g. as used in Finland by Tuittila et al. (1999) or in Germany by Droesler (2005), can successfully be applied to support NEE-models for entire year courses, once the parameterization of the models is done frequently (e.g. biweekly) under all climate conditions over the year. In peatlands according to current topographical settings very often stable boundary layers develop during nighttime. CO2 concentrations can rise to 1000 ppm directly above the vegetation. Eddy covariance may have difficulties to realistically track the CO2-exchange under these non-turbulent conditions. Automated chambers could be used for the direct high frequent measurement of NEE even under those conditions. However, up to now, no climate controlled automated chambers are available, which would be the best match between climate controlled manual chambers and Eddy covariance systems.

## 8.6. Strategy towards a pan-European GHG (continental scale) peatlands monitoring

### 8.6.1 Observation network

A continental scale GHG monitoring must take the large, highly variable GHG fluxes on peatlands into account. Observation sites should include LORCA, the present ecosystem C balance and fluxes of all GHG species. In the following minimum requirements for peatland observation sites are recommended.

For LORCA, at minimum, peat depth, peat dry bulk density profile, C content profiles and 14C at the peat bottom are needed. All of these measurements are scarce in Europe. Finland may be an exception with a long history in peat profiling by the Finish forestry service. The first three variables are particularly relevant as they establish the C stocks in the peatland.

The ecosystem C balance can only be established if all important (>1% of C flux) components of the C exchange in the peatland are observed, i.e., CO2, CH4, C losses to the ground- and surface water via DOC, DIC and POC, and C import and export by land use and management.

For measuring the GHG budget comprehensive, year-round, multi-year greenhouse gas flux measurements (CO2, CH4, and N2O), are required. Frequent measurements in periods of high flux variability and likely extreme bursts of CH4 and N2O are as critical as long-term measurements to cover the interannual variability and sensitivity of GHG fluxes to climate variability and trends. The thawing period is particularly important for CH4 and N2O while for CO2, the entire non-growing season can significantly contribute to the annual C balance. Each study site should comprise measurements in all relevant sub-units (e.g. combination of individual vegetation types, water-table situations, peat-characteristics). A minimum core set of ancillary variables is needed for interpretation of the measurements and for scaling the site level measurements to the European peatlands. Scaling usually relies on models based on relationships between climatic, edaphic and biotic drivers and observed GHG fluxes. The most critical core variables are

- meteorological data, in particular temperature (air and soil temperature profiles) and radiation (PAR)

- water-table

- vegetation type, fractional cover and biomass of plant functional types (peat-forming mosses, other mosses, vascular plants: aerenchymal plants, others to be specified if appropriate).

- C/N ratio of peat

- pH, electrical conductivity in soil water

- detailed documentation of land use history and present management

### 8.6.2 Upscaling

First results of the combination of emission factors with land-cover types underline the relevance of peatlands as hotspots for the European GHG-budget. However, the assessment and monitoring of this GHG-exchange needs major efforts for upscaling methodologies. Apart from site level GHG flux data, ancillary scaling variables and suitable models, a major constraint to the European peatland GHG budget is set by the poor information about the geographic location and extent of peatlands, their properties (bog versus fen, nutrient status, water-table) and, in particular, their use. Peat resource data are often outdated as peatlands have been degraded so much that a significant fraction of the former peat area is turning to shallow peaty soils, which are, by definition, no peatlands any more. Detailed spatial information about dominant vegetation types, water levels and management is a prerequisite for reducing the uncertainty in the European peatland budget to levels below 50%.

At present, data gaps about C stocks, GHG fluxes, spatial coverage and management are most problematic for regions outside EU-15, where vast peatland areas have been subject to significant changes in land use after 1990. There is particular need for more data about GHG budgets of peatlands under particular land uses: a) bog: grassland, cropland, land abandoned after peat cut, restoration, forest chronosequences, N2O fluxes in general. b) fen: abandoned after harvest, restoration, CO2 fluxes in general.

Hotspots from small scales to regions and typical management regimes appear to be very important for overall European GHG emissions from peatlands. Large emissions isolated in time and space have for example been observed associated with organic agricultural soils for N2O and with permafrost melting and subsequent changes in northern natural mires for CH4. These hot spots will be crucial to capture in a pan-European GHG monitoring effort.

To support upscaling activities, there is a specific need to

-	represent much better the GHG-exchange of the span of different peatland-ecosystems (measurements)

-	to update the landcover and landuse information of the peatlands (remote sensing)

-	to update the actual extent and depth of the peatlands (combination of remote sensing and inventories)

### 8.6.3  Future directions for enhanced measurements

Automation of chamber measurements with climate control systems and remote control to minimize labour requirements and disturbance during measurements is a prerequisite for operationalization of GHG measurements in peatlands. Measurements of the net C balance including C losses to the groundwater need to be included in the core set of observations. The methodology is established, but still laborious. For integration of site level observations into a landscape scale C budget the fate of DOC, DIC and POC in the adjacent waters and streams needs to be traced.

The recommendation made above can be implemented stepwise towards a systematic, operational modelling /monitoring strategy as follows:

- define peat regions with similar sensitivity of GHG fluxes to climate and management, with similar site conditions

- identify regional hotspots with particular vulnerability to climate and land use changes, which may merit additional attention (e.g. permafrost regions, lowland areas with saltwater intrusion from sealevel rise, areas with drastically changing water-tables, e.g. through restoration or drainage)

- identify most important land use types in terms of areas and hotspots

- set-up core set of long-term observation sites in each peat region, each covering a range of sub-sites to represent the typical small-scale mosaic

- characterize spatially explicit peatland area, peat types, water-tables and land use

- improve models to represent the GHG exchange of all major peatland uses

## 8.7 Conclusion

## Peatlands are hotspots for GHG-emissions. Even in an industrialized country like Germany the estimated peatland GHG-exchange account for 2.3 – 4.5 % of anthropogenic emissions. Peatlands can on the other hand be managed more wisely with the potential to contribute significantly to GHG mitigation. Raising the water table to more natural levels will reduce CO2 losses and hence improve the net C balance of peatlands. Improper water management may, however, increase CH4 emissions and  reduce the overall GHG mitigation efficiency in some cases. The optimal effect can be determined by careful monitoring of CO2 and CH4.

## To fully cover all major peatland-regions, -types and -ecosystems as well as the ongoing or future management practices a specific monitoring program is necessary. The key requirements of such a program are outlined above. This program should be geographically differentiated according to the peatland regions to fully cover the ongoing driving forces (e.g. permafrost-melting, rising sea-water level, peat-cutting, afforestation, restoration, conservation) in EU-peatlands, with impact on the GHG-exchange. In depth assessments of the vulnerability, the hotspot activity and future management strategies of EU-peatlands need a pan-European monitoring program of GHG-exchange in peatlands.

## References

Alm, J., Schulman, L.,  Walden, J., Nykanen, H.,  Martikainen P.J.,  Silvola, J., 1999. Carbon balance of a boreal bog during a year within an exceptionally dry summer. Ecology: Vol. 80, No. 1, pp. 161–174.

Augustin et al. (pers. com.).

Bellisario LM, Bubier JL, Moore TR, Chanton JP (1999) Controls on CH4 emissions from a northern peatland. Global Biogeochemical Cycles, Vol. 13, No. 1, pp. 81-91.

Bergmann L., Drösler, M., Freibauer, A., Schultz, R., Höll, B., Jungunst, H. & Fiedler, S. (in prep.): Net ecosystem exchange of restored and agriculturally used fen-peatlands, Donauried, southern Germany.

Bubier, JL, Moore, T, Juggins, S (1995) Predicting methane emission from bryophyte distribution in northern canadian peatlands. Ecology, 76(3), 677-693.

Byrne, K.A., Chojnicki, B., Christensen, T.R., Drösler, M., Freibauer, A. Friborg, T.,  et al., 2004. EU Peatlands: Current Carbon Stocks and Trace Gas Fluxes,. ISSN 1723-2236. http://gaia.agraria.unitus.it/ceuroghg/ReportSS4.pdf, Viterbo, Italy.

Christensen TR, Ekberg A, Strom L, Mastepanov M, Panikov N, Oquist M, Svensson BH, Nykanen H, Martikainen PJ, Oskarsson H (2003) Factors controlling large scale variations in methane emissions from wetlands Geophysical Research Letter, 30, 1414.

Clymo RS (1984) The limits to peat bog growth. Philos. Trans. Of the Royal Soc. of London, B 303, 605-654.

Drösler, M,. 2005. Trace gas exchange and climatic relevance of bog ecosystems, Southern Germany, Dissertation TUM, 179 p. Online publication: urn:nbn:de:bvb:91-diss20050901-1249431017

Freibauer A., Höll, B.S., Drösler M., Fiedler S., Jungkunst H., (in prep.): Methane and nitrous oxide budgets in drained and restored calcaric fens.

Frolking S., Roulet N., Fuglestvedt J., 2006: How northern peatlands influence the Earth’s radiative budget: Sustained methane emission versus sustained carbon sequestration, JOURNAL OF GEOPHYSICAL RESEARCH, VOL. 111, G01008, doi:10.1029/2005JG000091

Gorham, E., 1991. Northern peatlands: role in the carbon cycle and probable responses to climatic warming. Ecological Applications, 1: 182-195.

Hendriks D. M. D., van Huissteden J., Dolman A. J., van der Molen M. K. 2007: Biogeosciences, 277-316. SRef-ID: 1810-6285/bgd/2007-4-277

IPCC, 1996. Global Change 1995 - The Science of Climate Change. Contribution of Working Group I to the Second Assessment Report of the Intergovernmental Panel on Climate Change

Joabsson A, Christensen TR (2001) Methane emsissions from wetlands and their relationship with vascular plants: An Arctic example. Global Change Biol. 7, 919-932.

Joosten, H. & D. Clarke (eds.), 2002. Wise use of mires and peatlands, background and principles including a framework for decision-making, pp. 1-304.

Kasimir Klemedtsson, Å., Klemedtsson, L., Berglund, K., Martikainen, P-J., Silvola, J. & O. Oenema, 1997. Greenhouse gas emissions from farmed organic soils: A review. Soil Use and Management, 13: 245-250.

Lappalainen, E. (ed.), 1996. Global Peat Resources, International Peat Society and Geological Survey of Finland: 57- 162.

Lafleur, P.M., N.T. Roulet, J.L. Bubier, T.R. Moore, and S. Frolking, 2003. Interannual variability in the peatland- atmosphere carbon dioxide exchange at an ombrotrophic bog. Global Biogeochemical Cycles, 17(2), 5.1-5.13, 1036, doi:10.1029/2002GB001983.

Minkkinen, K. & J. Laine, 1998. Long-term effect of forest drainage on peat carbon stores of pine mires in Finland. Canadian Journal of Forest Research, 28: 1267-1275.

Mäkilä, M., 1994. Suon energiasisällön laskeminen turpeen ominaisuuksien avulla. Summary: Calculation of the energy content of mires on the basis of peat properties. Report of Investigation 121, Geological Survey of Finland: 1-73.

Nilsson, M., Mikkela, C., Sundh, I., Granberg, G., Svensson, B. H., & B. Ranneby, 2001. Methane emission from Swedish mires: National and regional budgets and dependence on mire vegetation. Journal of Geophysical Research-Atmospheres, 106: 20847-20860.

Nykänen H, Alm J, Silvola J, Tolonen K, Martikainen PJ (1998) Methane flux on boreal peatlands of different of different fertility and the effect of long-term experimental lowering of the water table on flux rates. Global Biogeochemical Cycles, 12, 53-69.

Reth, S., Hentschel, K., Drösler, M., Falge, E., 2005: DenNit - Experimental analysis and modelling of soil N2O efflux in response on changes of soil water content, soil temperature, soil pH, nutrient availability and the time after rain event. Plant-and-Soil. 2005; 272(1-2): 349-363

Sundh, I., Nilsson, M., Mikkelä, C., Granberg, G. & B. H. Svensson, 2000. Fluxes of Methane and Carbon Dioxide on Peat-mining Areas in Sweden. Ambio, 29 (8): 499-503.

Tuittila ES, Komulainen VM, Vasander H, Laine J, 1999. Restored cut-away peatland as a sink for atmospheric CO2. Oecologia, 120, 563-574.

World Energy Council (2001) 2001 Survey of Energy Resources. http://www.worldenergy.org/wec-geis/publications/reports/ser/peat/peat

Figure captions chapter X

Figure 1 Peatland area and use in the top 10 peat countries of Europe. Mires are undrained natural peatlands. The categories forestry, grassland, cropland and peat cut are peatland use with drainage. The unknown area results from inconsistencies in the data sources and incomplete information and is considered to be undrained. Data from Joosten and Clarke (2002), Lappalainen (1996), World Energy Council (2001) and national sources.

Figure 2 Typical GHG flux rates in European peatland categories. Black: bogs, grey: fens. The numbers n indicate the number of data sets available for bogs and fens. The bars represent the median GHG flux rates, error bars indicate the maximum and minimum values found in the literature.

Figure 3 Carbon balance versus net climate effect of peatlands. Positive values indicate C losses and net climate warming. The net climate effect was calculated from typical flux rates in Figure 2 using the global warming potentials (GWP) used under the UNFCCC (GWP100, IPCC 1996): 1 kg CH4 = 21 kg CO2; 1 kg N2O = 310 kg CO2. Carbon balance = Net Ecosystem Exchange (NEE) – C removal + CH4-C.

Figure 4: Conservative estimate for the peatland GHG budget of the top 10 peat area countries in Europe

Figure 5 Schematic comparison of flux directions and relative magnitudes of Net-CO2 (dark grey), CH4 (white) and N2O (light grey) fluxes in peatlands. Dotted areas symbolize the water saturated part of the profile. Waves in last panel: Flooded.

Table captions chapter X

Table 1: Sensitivity of the European peatland GHG balance to assumptions about land use and drainage intensity [Gg C-equivalents].

Table 2: General comparative characteristics of GHG-measurement techniques.

Chapter  X Figure 1

Chapter X Figure 2

Chapter X Figure 3

Chapter X Figure 4

Figure 5:

natural

slight drainage or drought

severe drainage

restoration:

high water-table

restoration:

flooding

transient over time?

Chapter X Table 1

|  |  | Continental Europe | EU-15 | EU-25 |
| --- | --- | --- | --- | --- |
| Peatland | Default | 51660 | 15291 | 21468 |
| Peatland | Maximum | 67512 | 24377 | 32019 |
| Peatland | Minimum | 49057 | 14325 | 20324 |
| Forest | Default | -362 | -212 | -225 |
| Forest | Deeper drainage | 9666 | 7133 | 7726 |
| Agriculture | Default | 40036 | 11925 | 17471 |
| Agriculture | 100% crop | 45860 | 13666 | 20071 |
| Agriculture | 100% grass | 37433 | 10959 | 16327 |

Chapter X Table 2

| methods | Eddy covariance | automated chambers | manual chambers |
| --- | --- | --- | --- |
| characteristics |  |  |  |
| undisturbed gas-exchange | ++ | +/- (cross-checks!) | +/- (cross-checks!) |
| integration over spatial variability | ++ | - (quantity of chambers vs. mosaic) | - (quantity of chambers vs. mosaic) |
| direct measurement of small scale spatial variability and management | -- | + | ++ |
| tracking temporal variability | ++ | ++ | - (campaigns as basis for modelling) |
| Costs | -- | -- | ++ |
| workload | ++ | + | -- |
| performance under all climate conditions | +/- | +/- | ++ |