Chapter 17

ISSUES IN ESTABLISHING IN SITU GREENHOUSE GAS MONITORING NETWORKS IN EUROPE AND IN REGIONS OF INTEREST TO EUROPE

Euan Nisbet1, Phillip O’Brien2, C. Mary R. Fowler1, Aodhagan Roddy2

1Dept. of Geology, Royal Holloway, Egham TW20 0EX, UK.

2 Dept. of Physics, Galway University, Galway, Ireland.

17.1  Introduction: General problems of in situ greenhouse gas monitoring

The atmospheric concentration of greenhouse gases can be measured in situ to great precision.  However, can the emission and uptake fluxes of these greenhouse gases be inferred from these measurements? Just as a wolf sniffs the wind, so sources of emissions can be measured at every scale from local to global and quantified by modelling. However, as monitoring equipment is usually static, measurements only apply to air masses which have passed through the station. Several problems emerge:

- Emission sources can be numerous and hard to distinguish. Seasonality may be muted. Thus very high standards of precision are needed at the in situ stations accompanied by very careful intercomparison between stations.

- At the risk of pushing the wolf analogy too far, pack hunters have advantages over solitary predators. A network of strategically placed in situ sampling stations can monitor and assess emissions within a region far more effectively than a whole series of independent stations acting alone.

In building an effective network of complementary in situ monitoring stations, the first problem is to ensure that the data from all the stations are intercompared. This is not easy even for a single integrated network, but has been achieved by a multi-national network in Europe with careful round-robin programs (e.g. Meth-MonitEUr 2005 for methane).

Local and specific emissions within access ('sniffing range') of stations can be determined by various techniques. Under certain conditions, the local natural ground source emission of 222Rn can be used as a proxy for assessing footprint and inversion height – the atmospheric concentration of 222Rn can, in effect, provide a measure of the volume into which the emission is diluted. Similarly on a local scale, “ratios of mixing ratios” can be used very effectively (e.g. Levin et al., 1999, Lowry et al., 2001). If CO2 emissions inventories are relatively well-known in a city (e.g. from petrol sales and heating emissions), then the CH4: CO2 ratio gives the CH4 emission (which may be harder to inventory) and N2O: CO2 can provide an independent check.  The underlying assumption in both 222Rn and CO2 studies is that there is adequate knowledge of the emissions rate of the proxy parameter.

Regional studies must be able to observe changes as measured against that changing background. Although the basic background information must come from global networks, such as the US NOAA program, these have limitations since their coverage of the tropical landmass is poor. Strong meteorological control on natural and anthropogenic sources, with meteorologically-linked feedbacks, require continued campaigns of process studies on both regional and local scales.

17.1.1 Key Issues

The difficulties of establishing Greenhouse Gas monitoring networks are frequently discussed at specialist workshops and meetings (e.g. Worthy and Huang, 2005) but rarely mentioned in the scientific literature. Logistical problems such as shipping gas cylinders or maintaining high-quality data sets in remote locations are central problems of monitoring, yet tend to be taken as trivial by those using results.

Some key issues relevant to the establishment of a European network are:

- Purpose of monitoring

- Choice of concentrations and isotopes to be measured

- How to measure

- Site location.

Then there are the data quality issues:

- Calibration of stations

- Inter-comparison around the network.

Next come the deeper issues:

- Usefulness: types of result

- Robustness of science

- Robustness of financial and political support.

Arguably the robustness issues are the most important. The technology and methodology must be strong so that the network delivers good dependable measurements. Financial support for a network must be steady and sustainable – better not to attempt too much than to imperil long-term time series by a programme that is initially too ambitious.

Long term monitoring of environmental systems is not glamorous and may not immediately deliver first-author research publications. But, it is vital if we are to understand and assist the health of our planet.  It is impossible to go back and take measurements in hindsight. Change can only be observed if long-term monitoring time series are protected.

17.2  PURPOSE OF MONITORING

17.2.1  Why monitor?

17.2.1.1  To understand the global biosphere.

The global biosphere cannot be understood scientifically unless the carbon transfers can be measured. Methane has a lifetime of roughly a decade and CO2 emissions are in the air for centuries. Thus carbon gases have a 'topography' across the planet that is comparable to the distribution of temperature across the planet (although with the difference that there is a strong N-S gradient). Although carbon gases are well-mixed on annual or decadal scales, there is strong seasonal and latitudinal variation from which much information can be derived.

At present this can only be achieved with in situ monitoring that provides high quality regional and global datasets from a number of carefully intercompared sites. Satellite data (e.g. OCO, Sciamachy) are becoming a vital adjunct to in situ measurment but for many years satellite measurements will remain both less precise and less accurate than ground-based measurements. Moreover, satellite measurements need to be “ground-truthed”: in situ and satellite work are mutually supporting.

In the late 1950s the Mauna Loa record first began show the 'breathing' of the planet (e.g. Keeling and Whorf, 2005). Now fifty years later, total global carbon-gas budgets are reasonably well known, though surprises still emerge and major sources and sinks remain poorly quantified. Studies are beginning to assess regional scale budgets. On a local scale carbon can be tracked isotopically from anthropogenic sources (e.g. urban power stations) and distinguished from other sources. For example, European landfill emissions of methane can be constrained by isotopic monitoring. This is a powerful method - grass-burning in Southern Africa can be detected by sampling methane in New Zealand (Lowe et al., 1994).

With an intercompared in situ monitoring record from a modest European and global network, coupled with the wide geographic scope of satellite monitoring, and supported by detailed local, regional, and global modelling, it should be possible to track and to quantify by sources and seasons the fluxes of carbon gases from major natural sources and sinks, not only in the wealthy northern lands but also in the tropics and oceans. Single sites are very useful (e.g. Biraud et al., 2000, 2002; Ryall et al., 2001), but an integrated network is needed to support regional inversion.

17.2.1.2    To verify emissions declarations

CO2 and CH4 emissions are declared under the UN Framework Convention for Climate Change (UNFCCC) and the Kyoto Treaty. Presently, carbon emissions are self-declared. This is done by the emitters at almost all levels, from farmer to European Union, by collecting ‘bottom-up” statistics (e.g. numbers of eructating cows, tons of coal burnt).   Such a self-declaration system is analogous to income tax being declared and paid with no checking mechanism.  If the Kyoto process is to succeed, there is an urgent need for an auditing process that is independent of information provided by the emitters. That auditing process is atmospheric monitoring.

“Trust but verify”, said Ronald Reagan, citing an old Russian saying.  Verification ensured the success of the Nuclear Test Ban Treaty, which set up the World-Wide Standardised Seismograph Network to detect nuclear tests (and was unexpectedly a significant factor in the understanding of Plate Tectonics). UNFCCC and Kyoto lack verification: there is no independent top-down (atmospheric) validation of emissions declarations. The aim of verification should be to quantify national emissions by source type, season and location, and within nations to provide independent checks on the veracity of national inventory declarations of emissions.

17.2.2 What to monitor?  What proportion of the effort should go to each species?  What should be the balance between in situ and satellite monitoring?

The purpose of monitoring dictates the species selected.  CFCs are already well monitored under Montreal protocol rules. CFC sources are now largely controlled.  What should be monitored in situ under a Kyoto-focussed program? CO2 and CH4 mixing ratios are obvious targets, but beyond them, the choices are complex given the limited finance available. How much support should be given to isotopic monitoring? Does monitoring tropospheric ozone need support, or is it sufficient to study methane. What about hydrogen, which is an indirect greenhouse gas?  What about N2O, which is a 'low-hanging fruit' for emission controls but is badly neglected by monitoring?

Perhaps the best immediate answer is that the effort (defined as financial budget) should be apportioned between gases according to total radiative impact. On a short-term or present-day scale, methane has roughly half or more of the effective direct and indirect impact of carbon dioxide (Hansen and Sato, 2001) and is much more easily reduced. If so, then methane monitoring should have half the budget allocated to CO2. The next question concerns isotopic measurements – how much effort should go on these? That question is less easily answered. Isotopes offer insight into sources, so that not only can fluxes of gas be measured but the specific types of source (e.g. methane gas leak or landfill) can be quantified. But isotopic work is expensive.

The best division of resources between in situ programmes and satellite programmes is also hard to decide. Many programmes have a rough rule-of-thumb of 50:50 allocation, which seems sensible. Europe has an ad hoc Call-by-Call approach. Europe has a bigger population than the U.S.A. and far fewer military projects. It has excellent satellite launching abilities which has led to strong support for satellite work and also for techniques such as upward-looking FTIR, that are used to ground-truth satellite data. However, this has had the effect of terminating or suppressing in situ work, even though that in situ work may be more accurate by a factor of 100. For example, sunlight-using satellite infra-red observations cannot assess the Arctic in winter, nor Europe by night. Also, they do not have the precision to observe absolutely the seasonal and diurnal methane cycles – only in situ chemical analysis is able to do this.

It is therefore essential that satellite and in situ observation work in tandem – both satellite and in situ studies need the support of the other.

Historically, Europe has been unable or unwilling to support an in situ network comparable to the US NOAA CMDL system. Moreover, Europe has recently shut down the painstakingly inter-compared Meth-MonitEUr methane network. In many locations, European carbon gas observation is left to nation states. Some nations (e.g. France, Finland) are making major contributions, but others (e.g. the UK) have no national support for CO2 monitoring.

The lack of European support for monitoring has the global impact that the U.S.A. has to bear the burden of in situ monitoring in many locations, even in European territory. US work ranges from monitoring at Ireland’s Mace Head (NASA AGAGE) to flask collection in the Baltic, Romania and even on the UK’s Ascension Island (NOAA).  In view of Europe’s strong stance on Kyoto this leaves the European Union open to a charge of hypocrisy.

17.2.3 Where to monitor? Global or regional or local?

There is presently one major global monitoring network, the US NOAA network. NOAA has a good basic global network, better in the Pacific hemisphere than the Old World. Australia and France help on a semi-global scale while other national programs (e.g. Germany, Japan, New Zealand) have long reach outside their own nations. There are important national programs (e.g. Canada, China, Finland) but as a whole Europe is struggling to create integrated regional networks. The NW Europe marine/coastal boundary layer is relatively well studied, as is the French sector of the Indian Ocean, but the Mediterranean and former Soviet Europe are not.

Monitoring networks need both rigorous calibration and intercomparison between sites since without these the data are useless. Badly-located sites and badly-measured data are worse than useless since using poorly intercompared or simply imprecise measurements in models make the models worse, not better, at capturing reality.

The  NOAA monitoring program very sensibly chose to set up a few continuous monitoring sites, mostly of marine boundary layer air and then supplemented this with flask collection. Mauna Loa (Scripps Inst.) is a site that samples free troposphere, but elsewhere the free troposphere can be monitored from aircraft, as in the CARIBIC programme.

Much of the globe's land area is not monitored. The Middle East and North Africa, major sources and consumers of oil and gas, have virtually no monitoring. In particular the giant gasfields of the Gulf and Iran are little sampled for methane emission. The Indian subcontinent, potentially one of the largest coal-burning economies, has little monitoring apart from French efforts, and the huge source regions of South-East Asia are essentially unmonitored.

Continuous monitoring in Africa is at the excellent station on Cape Point (South Africa). This is superbly located for observing the Southern Ocean, but less well for sampling African air.  It rarely if ever experiences air from the Congo (which is as far from Cape Town as Uganda is from Cyprus). There are only a few flask sampling programs in sub-Saharan Africa, apart from measurment on Mt. Kenya. Much of Latin America is unmonitored. Generally, regional and local studies are badly needed in the tropics. In general, monitoring observations are not yet good enough to support regional modelling in many areas of emission.

Good local information and local meteorological knowledge are vital if in situ data are to be used in modelling. Global wind fields are now well known. However, in large parts of Africa and other tropical regions, local meteorological data are not especially accurate and in many places, long records have now been discontinued.  Moreover, unmapped local sources may exist (e.g. new local landfills). Thus, interpreting both integrated and spot flask samples from these locations can be problematic.

17. 2.4 Hotspots

In addition to collecting data from background sites for global and regional studies there is also a need to monitor local high-emission ‘hot-spots’ (Levin et al., 1999; Lowry et al., 2001; Zinchenko et al., 2002). These have been much neglected in past work. For example, the commonly cited global average values for CO2 and CH4 mixing ratios are for marine background settings. If the landmasses are included, global values are higher.

Heavily populated areas are major source areas, where both point sources (e.g. power stations) and disseminated sources (e.g. cars) are significant.  In such areas, either tall towers can be used to collect from various heights in the boundary layer, or carefully located well-characterised sites. Location also needs care to ensure a well-distributed network that samples all the major source regions nearby: this means siting according to the wind spectrum.

In major cities simple inexpensive monitoring (e.g. a single station running continuous real-time carbon gas and isotope measurements and 222Rn) can, with good modelling, potentially assess emissions in real-time from an area with a footprint of say 50-100km, containing millions of people. The data are especially useful if assessed in conjunction with emissions inventories. Alternately, they can be used to test inventories (Baggott et al., 2005). Emissions inventories may be quoted to extremely high precision (for example, London's inventory is quoted to 9 significant figures, or the nearest 100 kg (a single truck's diesel tank) (Buckingham et al., 1998)).  However, they can at the same time be very inaccurate.

In poor-nation mega-cities, emissions inventories are rare or may be absent. Measurement may be the simplest way of assessing emissions, but a potential major problem arises in many areas of major emissions, as knowledge of the height of the mixed layer is frequently not good enough to support reliable emissions studies. Here 222Rn can be very useful.

17.2.5 When to monitor?

The obvious answer is frequently as possible, but this is not realistic since not every site can have a continuous instrument, and there will always be severe limitations on funding. Society may well prefer to support a medical clinic than a Greenhouse Gas station. Thus any monitoring program needs to be tailored very cautiously to sustainable long-term budgets. This is a key point and will be returned to later below – it is better to run a sustainable and reliable long-term SmartCar time series than to use erratic funding for stop-go Rolls-Royce programs.

17.3. CRITERIA FOR LOCATING MONITORING SITES

Scientifically, the key problem is to locate monitoring sites in the best places to support modelling. How can the errors in representing actual greenhouse gases in the air be reduced in models?  However, financial constraints often overwhelm scientific priorities. Given that financial support will only permit a certain number of sites, where are the places from which datasets will maximise the quality of the model and minimise differences with actual greenhouse gas distributions and sources?

There are both intuitive and quantitative answers to this.

Intuitive answers would include classifying the air masses into broad categories – marine background air; continental interior background air; polluted hot-spots, etc.  However the problem is more complex: specific sources will need to be identified if reduction programs are to be assessed and this means that major source regions need to be studied with more care, either from within the region itself or downwind from it.

Quantitative answers come from the use of models to choose locations. Simple back-trajectory studies are particularly useful in assessing locations best sited to sample air from different source regions and background regions. Trajectory modelling can also be used to search for likely trajectories that cross two or more sampling points, and hence can directly sample more than one key emissions area. Ideally, from new continuous sites, both major source locations and background air streams need to be accessible.

In order to determine which are the key locations for measuring particular sources, mesoscale forward models can be used, to search for the ability, from knowledge of carbon gases at surrounding points where measurement is already occurring, to reproduce greenhouse gas mixing ratios and isotopes at potential study sites. If the models are successful there may be little benefit in sampling at a potential study site: the new data will add little to understanding.

Realistic sites are however not perfectly distributed and individual sites may appear poor in both intuitive and quantitative tests. The key need is for long-term sustained high quality measurement. In practical terms this cannot easily be delivered unless there is a strong long-term commitment to measurement. This will depend on managerial and political factors, not science. Thus the U.S.A., with a very long-term commitment to NOAA, can maintain a number of very high quality baseline sites, while other nations may maintain one or two such sites and a few flask sites.

Scientifically this determines location of global sites. It may be desirable to support a long-term site in North Korea or Somalia, but practically this will not occur unless support can be found: it may be better to settle for a less good site in South Korea or Kenya. Such considerations are also (on a lesser scale) important within Europe: different nations have different long-term commitment to sustain long-term observations. For example, the North Atlantic CO2 background on the Irish coast is monitored by France and the U.S.A.. This is because Ireland does not support measurement and in the UK CO2 monitoring is very low on the priority list, not seen to be in the remits of either the research council or government department.

A long-term European network needs to be built on a regional/continental scale. The European dimension extends to the gas fields providing Europe’s gas from NW Siberia (a major methane source). Europe also has responsibility for sharing in African monitoring.  The sampling strategy must reflect this extent: in other words the network design must support modelling but yet not be so expensive it cannot be financially supported for the long-term.

Future societal needs may need additional local scale studies. This will require new stations. New methods will continuously appear, each demanding new monitoring strategies. It is therefore vital for a sustainable network design to contain an integrated research component that will deliver continuous network improvement.

17.4.   PRESENT SITE LOCATION: EUROPEAN AND GLOBAL NETWORKS

While network sites should ideally be chosen on purely meteorological grounds, in practice this is unrealistic. The main site selection for U.S.A. and northern European programs was made long ago and the best future policy is to build on the existing network of stations. The equipment and skill investment should exploit the valuable heritage of time-series already accrued (Figs 1, 2).

Although these stations may not seem at first glance optimally sited in terms of atmospheric need, in pragmatic terms they are indeed optimally sited. This is because they have grown up naturally and have support over the long-term. Each station has sustaining factors. Some are close to key universities or institutes, or in locations of national interest. Many of these sites have a multi-year record of observation, representing a long-term financial investment. The records from the stations may not be well intercompared or even well calibrated, but data exists. Hence there is a need, as far as possible, to back-integrate these datasets by intercomparison of archived air.

Local design of stations clearly depends on purpose. While tall towers on ‘typical’ flat land are ideal for characterising reasonably large footprints, major emissions often occur in port or river cities and locations with complex topographies (e.g. oilfields).

Microwave aerial design for cellphone telephony depends heavily on detailed local topographic modelling. This is a useful parallel. In assessing local emissions, there is a need for more thought about linking similarly sophisticated local topographic data into meteorological models of monitoring, so that footprint areas around continuous sites can be identified and fluxes assessed on a real-time basis.

The present array of nationally and internationally supported stations (Figs 1, 3) is strong in NW continental Europe, and has Atlantic stations at Mace Head, Ireland, Ny Alesund, Svalbard, and Izana (Tenerife), but is weak in southern and eastern Europe, the UK, and Iberia.

The far eastern Mediterranean, proximal to the Middle East oil and gas industry, is a major gap, as is the southern Mediterranean, near the large North African gas industry that supplies many parts of Europe. In particular, the network lacks continuous stations in many key regions.  Much is currently being done to improve this position. The Chiotto Tall Towers (part of Carbo-Europe) are adding important new geographic input (e.g. Bialystok, Angus). For methane, there is also now better integration between AGAGE and NOAA-linked stations. Flask sampling is being used to extend geographic spread (e.g Portugal and a new Cyprus station in Geomon).

17. 5. DEVELOPING A EUROPEAN NETWORK - NEW SITE SELECTION

Gaps in the network need to be filled to permit a full European understanding of regional methane budgets at a level that allows quantitative assessment of European sources. New sites should be co-sited with other gas and meteorological measurements. As mentioned above, there is a particular need for new stations in the Mediterranean and in Russia (Fig. 3).

17. 5.1 Trajectory studies with the HYSPLIT 4 model

Relatively simple conclusions can be reached about site locations using the HYSPLIT 4 (Hybrid Single Particle Lagrangian Integrated Transport) model, which was developed at the Atmospheric Research Laboratories, Maryland and designed to support a wide range of simulations related to the long-range transport, dispersion, and deposition of pollutants (Draxler, 1992; Draxler and Rolph, 2003; Rolph, 2003).  The calculation method is a hybrid between Eulerian and Lagrangian approaches: advection and diffusion calculations are made in a Lagrangian framework while concentrations are calculated on a fixed grid.

The network of existing methane sites has been assessed in terms of the mutual or shared back trajectories which allows assessment of the degree to which a particular station might contribute to a modelling programme for estimating regional emissions.  When two (or more) sites share a back trajectory, then measurements from the first station can initialise the model and represent the air mass conditions prior to sampling at the second station (Figs, 3, 4). Any differences between the sites are then ascribed to emissions, sinks and other processes that have occurred as the air travelled between sites.

Sites on the western fringe of Europe have relatively few shared back trajectories arriving from other sites due to the prevailing westerly airflows in the North Atlantic region. However, they commonly share back trajectories with other stations further east, thereby the western stations can often act as the background or initialisation, from which to investigate the emissions in regions between stations. This is especially important as measurements made at western marine or coastal sampling stations tend to be representative of a larger region than measurements taken inland. This shared trajectory perspective shows the gaps in the coverage of the present network: the Mediterranean region and northern and southern Europe are poorly covered (Fig. 4).

Certain criteria can be identified to assist in selecting sites for new stations within the network.

- Prospective sites should be chosen on the basis that they should already have some limited air-sampling programme, or manned synoptic meteorological observations. A good example might be that the site is already part of the NOAA CMDL flask-sampling programme. The sites in Areiro (Portugal) and on the island of Lampedusa are good examples (Fig. 2).

- New sites should provide additional coverage to the present overall network.  The sites of Areiro and Lampedusa would effectively add coverage of Iberia and southern France. Other potential sites in the eastern Mediterranean (e.g. Crete, Cyprus) appear largely independent of the larger network; typically, with air masses in this region neither originating nor finishing in Western Europe. Thus they would extend the network significantly eastwards but not add to the knowledge of the present regions.

- As already mentioned, the site should not be already redundant – i.e. the site must add new information, not already supplied within the network. New sites should target regions where at present coverage is sparse.  However this does not automatically exclude new sites which are geographically close to others, as topography has a profound influence on the movement of air masses: proximity of sites to the Alps does not automatically make them good sites for monitoring northern Italy.

One purpose of the European monitoring network may be to provide data useful in predicting chemical weather in Europe. Figures 3 and 4 show that the present network does not allow for any robust statistics on how regional fluxes in eastern and southern Europe change with time. Moreover, with the exception of the heavily populated area in NW Europe many locations may only be overpassed by one trajectory during an entire year (Fig. 3). For station-to-station trajectories during the entire year the region touched by more than one trajectory is yet smaller (Fig. 4). However, although rare, such individual trajectory studies are useful for case studies, especially if isotopic data are available.

###### 17.6   SOME SPECIFIC ISSUES IN NETWORK DESIGN

It is assumed that long-term in situ stations should measure continuously for CO2, CH4, CO and (ideally) N2O and H2 mixing ratios. In addition, stations should gather local weather data (especially local winds in anticyclonic periods of nearly-still air).  To be useful, all measurements should attain UN WMO Global Atmosphere Watch quality standards, with regular intercomparison with other stations. If stations do not supply these measurements, there is little purpose in monitoring except for specific campaign studies.

17.6.1. Source identification – trajectory studies

Trajectory or transport models allow source identification in individual packets of air, in specific episodes. This is useful both in Kyoto-related studies (testing emissions inventories and their seasonality), and also in biosphere studies. In testing inventories, such work is best at testing ratios of declared species rather than quantitatively determining fluxes, because of the uncertainties in local meteorological modelling and flux distribution. Individual studies collate to give semi-quantitative information about source characteristics and fluxes if multiple long-term trajectory records are gathered. The observational network thus needs enough stations to cover major sources and transport directions, as well as a few ‘background’ stations, especially marine background inputs, and stations peripheral to the region of interest.

17. 6.2 Isotopic Measurements

Isotopic measurements need precision that is adequate to differentiate between specific remote sources, and also to measure seasonality in the long-term record.  Remote sources are studied by high volume, high-precision spot sample studies (e.g. for methane, circa 30-100L of air, better than 0.03‰ precision in 13CCH4). This high precision work necessarily has infrequent sampling, if collected in flasks. Integrated sampling over 2-week periods remains valuable for characterising isotopic ratios in ambient air over large remote areas (e.g. Arctic).

Quasi-continuous isotopic measurement capability has recently been developed (Fisher et al., 2006). It is likely that continuous isotopic monitoring for CO2 and CH4 will soon be invaluable in quantifying emissions from major source regions.  Low-volume intermediate-precision work (e.g. <100 mL, 0.05‰ precision in 13CCH4) can be run automatically every 30 mins. However, these intermediate precision isotopic studies do not yet have adequate precision to monitor tropical seasonality for CH4, nor to spot remote sources by back-trajectory study.

The issues of cost and reliability will be problematic if automatic isotopic monitoring instruments are placed at remote monitoring stations. Short-run continuous data sets have the same interpretive problems as the single station concentration data.  However, in principle, we can create a relatively inexpensive isotopic network acquiring robust average gradients over time from a relatively sparse (e.g. several hundred kilometres between stations) array of stations appropriately sited around the European region.

Although continuous measurements will be useful in areas of major sources such as urban centres, collection of integrated isotope samples (over say 2 weeks), remains necessary to determine regional gradients in a monitoring network. Integrated (large bag) sampling techniques have become “unfashionable” but have clear assets in cost effectiveness when taking into account the limits of interpretive capacity the meteorology imposes.

For flask samples (event sampling) to be useful, they must be frequent. If collected sporadically (e.g. the NOAA/CMDL network one per week), data are not sufficient for rapid determination of gradient shifts in European weather systems. In a region with large variability on time-scales shorter than the sampling frequency there will be a large element of randomness built into the dataset. The flask sampling programs are, nevertheless, extremely important both for giving global average pictures and most significantly to provide a completely independent verification of the long-term monitoring instruments performance.

17.6.3. Vertical measurement

Greenhouse gases are emitted on the surface. Vertical measurement (e.g. Fourier-Transform Infra-Red, FTIR) is essential but the core of the Kyoto problem is on the ground. The circulation of the mid and upper troposphere around the planet in mid-latitudes takes a few weeks. This means that relatively sparse sampling at altitude is enough to define the fast-moving troposphere: more sampling will not necessarily provide much more information about sources. However, upward mixing from sources is rather slower – thus in order to ground-truth satellite data, regular from-ground upward-looking measurements will be needed. Such upward-looking vertical measurement, made from a few selected meteorologically simple sites, is vital for characterising the free troposphere.

The key problem with satellite studies is that they measure total column while the 'action' in greenhouse gases is in the mixed layer and low troposphere. At present the usefulness of satellite measurements remains limited and there are significant problems in converting vertical column densities to mixing ratios. In particular, cloud parameterisation remains a problem, especially in the tropics. However (e.g. Frankenberg et al., 2006) the methodology is fast improving, the technique will be much more powerful when OCO is operating, and will be useful in sustaining global inversions. There is thus also a need for upward looking FTIR ground-truthing in difficult sites in the cloudy tropics, co-located with good in situ conventional measurement of Carbon gases.

Satellite-mounted instruments, looking downwards, must have a high precision and also accuracy to be capable of detecting and discriminating between emissions sources. The retrieval of information is improving rapidly, but remains unable to validate emissions inventories. It is very difficult to determine diurnal and seasonal emission changes by total column observation. For methane, satellite measurements have a precision roughly two orders of magnitude poorer than ground measurements and can barely detect diurnal variation.

There are thus severe inherent problems with satellite studies. Infra-red based satellite work depends on sunlight and is thus not possible at night or during the polar winter. The Arctic is a key region of winter anthropogenic methane emission from the major gasfields, and is arguably the location of the greatest 'abrupt change' global climate risk (clathrate degassing). But it is dark in winter. In heavily populated areas, major emissions (e.g. heating) occur at night, compromising the usefulness of light-based sensing in the nocturnal hemisphere.

Summing up, in the near future, some satellite work (e.g. OCO) will indeed become very useful, dependent on better cloud parameterisation. Rigorous ground-truthing is essential, with a need for upward-looking FTIR instruments.Thus in situ monitoring will be essential for the foreseeable future. Clearly satellite data and vertical profiles are useful where available, but the backbone of a greenhouse gas sampling network must be built on surface monitoring stations unless new breakthroughs in vertical sampling technology or remote sensing techniques occur.

17.7. CASE STUDY – THE SOUTH ATLANTIC

The South Atlantic is a region where better carbon gas monitoring is urgently needed. Le Quere et al. (in press) showed that the Southern Ocean sink of CO2 has decreased between 1981 and 2004 by 0.03 PgC/y per decade (5-30% per decade), in spite of the large increase in atmospheric CO2. They comment that data are "particularly sparse in the Southern Ocean, where the magnitude of the CO2 sink is heavily disputed, its interannual variability is unknown, and its control on atmospheric CO2 during glaciations is firmly established but still not understood nor quantified." There are very few stations in this region. The main data stream is from a single high quality continuous station at Cape Point, supplemented by NOAA flask sampling from Halley Bay, Antarctica and Ascension Island.

This region is clearly a target for much better monitoring and is a European responsibility. The main islands of the South Atlantic (Ascension, St. Helena, Falklands, S, Georgia, Tristan, Gough, Bouvet) are all territories of European nations (UK and Norway), although UK-owned Gough Island is under South African administration. The major South Atlantic power is the UK, but given UK lack of support for monitoring, any initiatives will likely have to come from EU sources.

Three sites were chosen for investigation – Ascension, Falklands and South Georgia. Other sites considered were St. Helena, Tristan and Gough Island. Each of these has merits, but in the context of the excellent Cape Point observatory, the study focussed on Ascension and the islands on the west of the ocean.

Ascension Island is located at 8oS, close to the submarine Mid-Atlantic Ridge. The island is a relict volcano, with a modest peak. The prevailing winds are the SE Trades, which are almost invariant. Only in El Nino conditions do occasional thunderstorms occur when the Inter-Tropical Convergence Zone drifts over the island. Figure 5 shows representative trajectories. It is clear that the low-level air sampled on Ascension is representative of the wider South Atlantic. Ascension Island is therefore an excellent site for monitoring the ambient S. Atlantic, as far south as say 25-30o.  There is however a steady flow of mid-troposphere (above 3km) air from the east, including plumes from the biomass burning areas of Angola and Zambia/Congo.  Upward-looking FTIR based on Ascension would monitor this, and may be useful in ground-truthing OCO. This air is also accessible via sampling on regular scheduled aircraft (RAF charter) flights.

The Falklands Islands (Stanley 51oS) are a continental plateau. Figure 6 shows typical trajectories for air arriving at the Falklands. While much of this air arrives in the southwest to northwest sector, air also comes from the southern Pacific straight through the Drake Passage, directly from the south (Antarctica) and from the north (SW Atlantic). This is an excellent site for sampling air from southern South America, from the Drake Passage to the Anarctic penninsula region, from those parts of the S Atlantic to the north and east of the Falkland Islands and from the southern Pacific across towards New Zealand.

South Georgia (54oS) is a remarkable ridge (up to 2km high) straddling the westerly flow round Antarctica.  Figure 7 shows typical trajectories. Much of the air comes through the Drake Strait between South America and the Antarctic Peninsula. However, the site also has access to air from eastern Argentina and air that has looped round the local Southern Ocean.

Each of these potential sites has specific advantages for improving South Atlantic monitoring. Logistics are particularly important in this region: there is a regular flight from Oxford-Ascension-Falklands, but in contrast, communication to South Georgia is less regular. Given the presence of the current Cape Point station and the flask sampling at Halley Bay, the prioritisation might be Ascension and Falklands first, followed by South Georgia later.

17.8. THE NEED FOR CONTINUITY

European funding is on the short-term: typically 3-4 years. Fortunately within Europe, national governments are in many cases (UK excepted) prepared to take on long-term monitoring, so that centralised European funding only sustains the interlinking of measurements. Worldwide the picture is not reassuring. The key requirements for tropical stations are for multi-decade funding. However, many national governments may have little real interest in support (even in nations like India, hostile to greenhouse gas controls). Thus there is a failure to provide support (Nisbet 2005).   Europe may carry out campaigns and process studies in the tropics but not routine monitoring. Unless the U.S.A. expands its excellent NOAA program, there is little likelihood of this failure being rectified.

Long-term time series are needed to understand the great variability in the emissions and uptake budgets. Continuity is essential. Carbon gas emissions come from many unexpected sources, and CO2 sinks vary greatly year-on-year, while methane sinks (mainly OH) are more easily calculated. All these, sources and sinks, are closely related to climate and local meteorology. A cold spring can greatly delay carbon uptake over the whole of northern Eurasia, noticeably biassing the global pattern for a year. Anthropogenic sources and sinks are also meteorologically biassed, both for heating and for human activities. Reservoirs of carbon can also respond to change – for example peat carbon sequestration over the last 10,000 years has been immense but can be given back to the air if peat decays under global warming and drying. Similarly, clathrate storage of methane in the Arctic is very vulnerable to warming. Soil carbon stores in the tropics are large: these too are not only meteorologically dependent but also very directly controlled by human activity (e.g deliberate fires). many such stores can be released catastophically (e,g peat fires; large unstoppable underground coal fires in China). All these can only be tracked if long-term records are held.

Long-term change in major regions with large known carbon inventories (e.g. clathrate hydrate regions, forests; peatlands; soil carbon in the tropics) can be studied by sustained measurement of regional fluxes and by assessing inventory changes. For example, back-trajectory studies from Ireland can assess the gas and isotopic mixes of Canadian forest fire outputs in Quebec. Very sudden change is probably easier to measure than slower but cumulatively larger change – thus emissions from a forest fire can be quantified more easily than a 10 year forest die-back. Fossil fuel burning is in principle probably one of the easier sources to quantify – isotopic ratios are known as are geographic locations.

Assessing the response of all these sources to climate change may be difficult. For example, quantifying future Arctic methane clathrate release may depend on comparative past-history control studies in areas where such release is not now occurring: but how to identify such areas? Present terrestrial inventory studies for emissions of the main greenhouse gases (CO2, CH4, N2O) as well as CO and H2 may still be very poorly defined.

###### 17.9.   CONCLUSION: REQUIREMENTS FOR A MONITORING SYSTEM

To be sustainable and cost-effective, a scientifically and socially useful European monitoring system needs:

  - Mixing ratios measured by continuous high-precision ground-based monitoring from a network of stations with adequate spacing to cover the region

  - High-precision isotopic measurements from the same sites, either continuously or as integrated or very-frequent spot samples

  - An associated long-term modelling and interpretation program which would also routinely calculate total column estimates

  - High analytical quality, to WMO/IAEA GAW Expert Panel standards, the quality being audited and controlled via rigorous and regular intercomparsion programs

  - In addition to the continuous sites, flask sample programs from distant sites (e.g. Africa), subject to the same QA/QC

  - Vertical profiling by upward-looking high-resolution FTIR, routinely assessed against the modelling results

  - Routine satellite ground-truthing against total column models and FTIR results

  - Regular aircraft monitoring to assess the free troposphere, plus regular but infrequent stratospheric sampling (e.g. Arctic, where the stratosphere is more easily accessible)

  - Monitoring programs in key areas (e.g. major cities, major gasfields or coalfields) for continuous assessment of major local sources

Most important of all is long-term financial support, so that once started, monitoring time-series are maintained and not abandoned. Presently this is not possible with European Union funding programs and is a key reason why monitoring of Europe by the U.S.A. is essential.

ACKNOWLEDGEMENTS

The authors gratefully acknowledge the NOAA Air Resources Laboratory (ARL) for the provision of the HYSPLIT transport and dispersion model and READY website (http://www.arl.noaa.gov/ready.html) used in this publication. We are grateful to Tuula Aalto for assistance with Figure 2.

##### REFERENCES

Baggott, S.L., Brown, L., Milne, R.,  Murrells, T.P., Passant, N., Thistlethwaite, G. and Watterson, J.D., 2005.  UK Greenhouse Gas Inventory, 1990-2003. AEA Technology/ Department for Environment, Food and Rural Affairs. ISBN 0-9547136-5-6.

- Biraud, S., Ciais, P., Ramonet, M., Simmonds, P., Kazan, V., Monfray, P., O'Doherty, S., Spain, T. G. and Jennings, S. G., 2002. Sources of Carbon Dioxide, Methane, Nitrous Oxide, and Chloroform over Ireland inferred from Radon 222 and Radon 220 at Mace-Head, Tellus B, Vol. 54, 41-60.

- Biraud S., Ciais, P., Ramonet, M., Simmonds, P., Kazan, V., Monfray, P., O'Doherty, S., Spain, T. G. and Jennings, S. G., 2000. European greenhouse gas emissions estimated from continuous atmospheric measurements and Radon 222 at Mace Head, Ireland, Journal of Geophysical Research, 105, 1351-1366.

Buckingham, C., Clewly, L., Hutchinson, D., Sadler, L., and Shah, S., 1998. London Atmospheric Emissions Inventory. London Research Centre.

Draxler, R.R., 1992. Hybrid single-particle Lagrangian integrated trajectories (HY-SPLIT): Version 3.0 - User’s guide and model description. NOAA Tech. Memo. ERL ARL-195, 26 pp. and Appendices. [Available from National Technical Information Service, 5285 Port Royal Road, Springfield, VA 22161].

Draxler, R.R. and Rolph, G.D., 2003. HYSPLIT (HYbrid Single-Particle Lagrangian Integrated Trajectory) Model access via NOAA ARL READY Website (http://www.arl.noaa.gov/ready/hysplit4.html). NOAA Air Resources Laboratory, Silver Spring, MD.

Frankenberg, C., Meirink, J.F., Bergamaschi, P., Goede, A.P.H., Heimann, M., Koerner, S., Platt, U., van Weele, M. and Wagner, T., 2006. Satellite chartography of atmospheric methane from SCIAMACHY on board ENVISAT: analysis of he years 2003 and 2004. Journal of Geophyscial Research 111, D07303, doi:10.1029/2005JD006235.

Hansen, J.E.  and Sato, M., 2001. Trends of measured climate forcing agents. Proc. Nat. Acad. Sci. USA, 98 14778-83, www.pnas.org/cgi/doi/10.1073/pnas.261553698

Keeling, C.D. and T.P. Whorf. 2005. Atmospheric CO2 records from sites in the SIO air sampling network. In Trends: A Compendium of Data on Global Change. Carbon Dioxide Information Analysis Center, Oak Ridge National Laboratory, U.S. Department of Energy, Oak Ridge, Tenn., U.S.A..

Le Quere, C., Rödenbeck, C., Buitenhuis , E.T., Conway, T.J.,  Ray Langenfelds, R., Gomez A., Labuschagne C., Ramonet, M., Nakazawa T., Metzl, M., Gillett, N.,  and Heimann, M., 2007. Recent decrease in the Southern Ocean sink of CO2  Science, in press.

Levin, I., H. Glatzel-Mattheier, T. Marik, M. Cuntz, M. Schmidt and D.E: Worthy, 1999. Verification of German methane emission inventories and their recent changes based on atmospheric observations. J. Geophys. Res.,104, D3, 3447 - 3456.

Lowe, D.C., Brenninkmeijer, C.A.M., Brailsford, G.W., Lassey, K.R., Gomez, A.J., and Nisbet, E.G., 1994. Concentration and 13C records of atmospheric methane in New Zealand and Antarctica: evidence for changes in methane sources. Journal of Geophysical Research, 99, 16913-16925.

Lowry, D., Holmes, C.W., Rata, N.D., Nisbet, E.G. and O'Brien, P., 2001. London Methane Emissions: use of Diurnal Changes in Concentration and 13C to Identify Sources and Verify Inventories, J. Geophys. Res., 106 (D), 7427-7448.

Meth-Moniteur  2005. Final Report Section 6. Methane Monitoring in the European Union and Russia. European Union. http://www.gl.rhul.ac.uk/METH/MonitEUr/

Nisbet, E.G., 2005. Emissions control needs atmospheric verification. Nature, 433, 683.

Rolph, G.D., 2003. Real-time Environmental Applications and Display sYstem (READY) Website (http://www.arl.noaa.gov/ready/hysplit4.html). NOAA Air Resources Laboratory, Silver Spring, MD.

Ryall, D.B., Derwent, R.G., Manning, A.J., Simmonds, P.G. and O'Doherty, S., 2001. Estimating source strengths of European emissions of trace gases from observations at Mace Head. Atmospheric Environment 35, 2507–2523.

Worthy, D. and Huang, L., 2005. 12th WMO/IAEA meeting of experts on carbon dioxide concentration and related tracers measurement techniques. WMO Global Atmosphere Watch, Geneva: World Meteorological Organisation WMO TD 1275 (GAW 161).

Zinchenko, A.V., Paramonova,N.N., Privalov,V.I. and Reshetnikov, A.I., 2002. Estimation of methane emissions in the St. Petersburg, Russia, region: An atmospheric nocturnal boundary layer budget approach, J. Geophys. Res., 107, D20, 4416, doi:10.1029/2001JD001369.

FIGURE CAPTIONS

Figure 1 	Some existing CH4 and 13C-CH4 measurement sites in Europe, Siberia and the Arctic (Meth-MonitEUr 2005)

Figure 2 	Sites of present and likely future carbon gas monitoring in Europe: circles, already in operation; crosses, proposed/under establishment.

Figure 3  	A composite of daily 12.00 GMT 5-day back trajectories for 2002 indicating density of European coverage. Sites in Network are: Heidelberg, Kasprowy, Mace Head, Ny-Ålesund, Pallas, Paris, Puy de Dome, RHUL, Schauinsland, St. Petersburg. Red represents the highest density and yellow the lowest. (From NUI Galway, report to Meth-MonitEUr).

Figure 4 	Segments of trajectories as shown in Fig. 3 where prior to arrival at the sampling site, the air mass has passed within two degrees of another site in the network. (From NUI Galway, report to Meth-MonitEUr).

Figure 5	Representative 5-day HYSPLIT back-trajectories for air arriving at Ascension Island on 16/01/2006 (top left), 16/03/2006 (top right), 16/06/2006 (lower left), 16/10/2006 (lower right). The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

Figure 6	Representative 5-day HYSPLIT back-trajectories for air arriving at the Falkland Islands on 16/03/2006 (top left), 18/03/2006 (top right), 16/05/2006 (lower left), 16/07/2006 (lower right). The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

Figure 7	Representative 5-day HYSPLIT back-trajectories for air arriving at South Georgia on 16/02/2006 (top left), 16/06/2006 (top right), 16/07/2006 (lower left), 16/11/2006 (lower right). The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

FIGURE CAPTIONS

Figure 1 	Some existing CH4 and 13C-CH4 measurement sites in Europe, Siberia and the Arctic (Meth-MonitEUr 2005)

Figure 2 	Sites of present and likely future carbon gas monitoring in Europe: circles, already in operation; crosses, proposed/under establishment.

Figure 3  	A composite of daily 12.00 GMT back trajectories indicating density of European coverage.  Sites in Network are: Heidelberg, Kasprowy, Mace Head, Ny-Ålesund, Pallas, Paris, Puy de Dome, RHUL, Schauinsland, St. Petersburg. Red represents the highest density and yellow the lowest.  (From NUI Galway, report to Meth-MonitEUr).

Figure 4 	Segments of trajectories where prior to arrival at the sampling site, the air mass has passed within two degrees of another site in the network. (From NUI Galway, report to Meth-MonitEUr).

Figure 5	Representative 5-day HYSPLIT back-trajectories for air arriving at Ascension Island on 16/01/2006 (top left), 16/03/2006 (top right), 16/06/2006 (lower left), 16/10/2006 (lower right).  The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

Figure 6	Representative 5-day HYSPLIT back-trajectories for air arriving at the Falkland Islands on 16/03/2006 (top left), 18/03/2006 (top right), 16/05/2006 (lower left), 16/07/2006 (lower right).. The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

Figure 7	Representative 5-day HYSPLIT back-trajectories for air arriving at South Georgia on 16/02/2006 (top left), 16/06/2006 (top right), 16/07/2006 (lower left), 16/11/2006 (lower right).. The three trajectories show air arriving at heights of 100m, 500m, 1000m (light grey).

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7