Chapter 7

Flux tower sites, state of the art and network design.

Han Dolman1, Riccardo Valentini2, Margriet Groenendijk1 & Dimmie Hendriks1

1Vrije Universiteit,  Department of Hydrology and Geo-environmental sciences

Boelelaan 1085 1081 HV Amsterdam

han.dolman@geo.falw.vu.nl

2 University of Tuscia,  Viterbo, Italy

## 7.1  Introduction

Direct observations of the exchange fluxes of water, energy, carbon dioxide, trace gases and momentum have been possible since the 1960-ies. Most of this early work comprised campaigns of short duration and has contributed substantially to our process understanding of surface atmosphere exchange. The data obtained in these studies were fundamental in assessing the validity of Monin-Obhukov similarity over natural surfaces, in testing resistance analogue formulations (e.g. Monteith, 1965) and were also used to calibrate flux models against single site data. The seminal two volume work by J.L. Monteith “Vegetation and the atmosphere” provides an excellent summary of the micrometeorology of natural vegetation of this period (Monteith, 1976). It was also during this period that climate modelers became increasingly aware of the important role of the biosphere in climate. In the 1980-ies and 1990-ies the early observational work was extended to study variability along climatic or other transects. The analysis of Valentini et al. (2000) describing the variation of Net Ecosystem Exchange of CO2 along a North-South gradient in Europe typifies this stage. New developments in technology (e.g. Moncrieff et al. 1998), such as the availability of “off the shelf” fast response instrumentation, gave opportunity to the development of the eddy co-variance technique. The “la Thuile workshop” in 1994 which resulted in a special issue of Global Change Biology (Baldocchi et al., 1996) was fundamental in bringing together micrometeorologists and ecologists in studying exchange fluxes, not only of water and energy, but also of carbon. At the political level, the Kyoto process provided extra impetus to these studies, particularly for carbon, although the direct relevance of flux measurements to the Kyoto process remains somewhat controversial (Körner, 2003, Dolman et al., 2003). The data coverage (Figure 1) of this FLUXNET network (Baldocchi et al., 2001) is now near global, from Siberian sites (Dolman et al., 2004) to tropical rainforest (Araújo et al., 2002). Since around 2000 the analysis has begun to take advantage of this near global coverage of sites, and studies have emerged of inter-annual variability, data assimilation and continent scale estimation of carbon uptake (Knorr and Kattge, 2005; Papale and Valentini, 2003).

This chapter will briefly describe the technical details of modern eddy covariance systems, and will describe some of the important developments in our understanding of the magnitude of fluxes of different biomes and their spatial variability. It will then concentrate on the inter-annual variability of the fluxes and the related questions of vulnerability of the terrestrial carbon cycle. As an example of a vulnerability study, the analysis of the European heat wave and associated drought in 2003 (Ciais, et al., 2005) is shown. That analysis clearly shows the value of a dense network in picking up these synoptic scale disturbances in the carbon cycle. The chapter ends with an assessment of the representativity of the current European flux sites and how a future system could be designed to be able to address as many questions as possible to contribute to improved understanding of the continental scale carbon cycle.

[INSERT FIGURE 1 HERE]

7.2. State of the art

7.2.1 The eddy covariance technique

Figure 2 shows a schematic of the application of the eddy covariance technique based on Aubinet et al al. (2000). The technique is based on the fact that the flux of a quantity like CO2, FCO2 can be expressed as the co-variance (the average of the product of the fluctuating, turbulent quantities) of the vertical wind speed, w with that quantity, e.g. the mixing ratio of CO2, c.

Fc=+w'ρc'

This requires fast sensors that are able to observe the fluctuation up to 20 Hz. Instruments that achieve this are the sonic anemometer and closed or open path infrared sensors for H2O and CO2. This principle is well established and commercial instruments that can perform these measurements with adequate accuracy (Aubinet et al., 2000) are available since the early 1990-ies. Once these measurements are obtained, a set of data quality checks and assessment are required. These are briefly described below.

For instance, it appears notoriously difficult to distinguish the real vertical velocity from pseudo-vertical wind velocities that occur due to miss-alignment of the anemometer or distortion in the flow due to topography. These need to be corrected by rotating the calculated wind speeds in such a way that the mean vertical wind speed is zero. In practice this implies that the mean vertical windspeed is set perpendicular to the mean flowline. The real vertical wind speed fluctuations can be then estimated (Kaimal and Finnigan 1994, Finnigan et al., 2003).

[INSERT FIGURE 2. HERE]

One of the fundamental hypotheses that underlies the equivalence between the eddy covariance flux and the net ecosystem exchange of carbon is the existence and stationarity of the turbulent process. In practice environmental conditions change continuously during the day, for instance due to the diurnally varying solar input and associated temperature changes, and this assumption is thus easily violated. The non-stationarity test is a means to detect whether the non stationarity is such that basic assumptions are violated (Wichura and Foken, 1995).

In the past, largely because analog processing was used, running means were applied to calculate the fluctuating parts. Defining the average of such an operation is not trivial and may strongly affect the calculated final covariance . The simplest and most widely used and recommended technique today is the block averaging (Finnigan et al., 2003). One has to take care in defining the length of the block period, as low frequency contributions to the flux may become more important at tall vegetation (forests) than at low vegetation (e.g. grasslands) (Finnigan et al., 2003). Once the averaging period is determined, raw fluxes or covariances can be computed.

Eddies advected past a sonic anemometer do so at various angles. Because the sonic anemometer has a fixed shape, at some angles shading occurs and the measurements are affected. Gash and Dolman (2003) postulated that this angle would depend on the roughness length of the vegetation and that the neglect of this error could be responsible for the observed bad energy balance closure over tall vegetation. They showed furthermore that, although large angles of attack do not occur frequently, a large portion of the turbulent fluxes is carried by eddies under those large angles. This flux angle distribution can be shown to depend on surface (vegetation) roughness, measuring height and stability regime. With such large parts of the fluxes carried by eddies with angles of attack outside of the manufacturer's specifications, the potential for errors to affect the measurements becomes large. It can be shown that outside the recommended angles, the response of a widely used anemometer is bad. This, in combination with the fact that most of the flux is carried at large angles, suggests that the turbulent fluxes measured with these anemometers can be seriously underestimated. Wind tunnel experiments can determine the errors and a so called angle of attack dependent calibration was developed by van der Molen et al., (2004).

The all important step in Figure 2 is the step from uncorrected to corrected fluxes, and complete textbooks have been devoted to this subject (e.g. Lee et al., 2004). Key eddy covariance corrections (e.g. Lee et al., 2004, Aubinet et al., 2000) include the Webb correction for the existence of an apparent vertical velocity induced by the sensible heat flux from the surface and frequency loss corrections. Measuring the turbulent fluctuations requires fast sensors that are able to sense all the fluctuations in the frequency range (spectrum) in which turbulent transport takes place.  Optimum measurement conditions are hard to achieve in the real world due to limitations of the sensor speed, the delays that occur during data acquisition, averaging over a path rather than a point and physical separation between the sensors. It is possible for each of these situations to derive analytical expressions (transfer functions) about the magnitude of the individual effects for an ideal system. As mentioned before, the spectrum shows which frequencies of transport really contribute to the flux. By deriving the ratio of the frequency correction for a particular measurement setup and the universal spectra for a particular (co)variance one can calculate the frequency loss of that setup and afterwards add it to the observed flux (Aubinet et al., 2000). Several other smaller corrections need also be applied before the final flux is recovered from the raw turbulent signals.

The various correction factors result in substantial changes to the raw flux. In particular the Webb correction can be substantial in dry hot environments. Angle of attack and frequency response corrections normally change the flux by up to 20%. Since the sign of the correction is not always the same, the net effect is of the order 5-10% depending on the flux of interest.

Once the flux is calculated for typically half hourly values, periods for which no data is available need to be gap filled. Various methodologies exist for this purpose and range from simple linear interpolation, using functional relationships between fluxes and environmental data (radiation, C-uptake, Temperature- C-respiration etc.) neural networks, looking for flux data under similar environmental conditions, to the use of look up tables and detailed process modeling (e.g. Falge et al., 2001, Reichstein et al., 2005). It appears that those gap filling techniques that use neural networks generally perform well, surprisingly sometimes better than those based on process modeling.

7.2.2 Key issues

7.2.2.1 Separation of respiration and assimilation

Because the eddy-covariance technique measures only net exchange fluxes, some methodology is required to separate the Net Ecosystem Exchange (NEE) into its two main components, Gross Ecosystem Production (GEP) and Ecosystem Respiration (Reco). This is non-trivial. Separation not only is required for process understanding, but a good gap-filling methodology obviously depends on adequate calculation of the component fluxes. Existing flux partitioning algorithms can be classified in those that use only night-time data for the estimation of ecosystem respiration and those that use daytime data or both day- and night-time data using light response curves (see Reichstein et al., 2005 for a review). The two approaches have been compared by Falge et al. (2001), resulting in generally good agreement between the two methods, except in ecosystems where large soil carbon pools exist. Under those conditions, the light-curves derived from daytime data may not well represent the night-time respiratory processes.

The standard light-response curve method only allows the estimation of the daily average respiration without estimation of a temperature-dependent diurnal course. A regression model, where GEP and Reco are described in one equation, is often used where Reco is explicitly dependent on air or soil temperature. Once regression parameters are fitted, GEP and Reco can be computed separately on a half-hourly time step. While this is an elegant approach, it suffers from a number of problems (Reichstein et al., 2005). First, similar model predictions of NEE, can be obtained by several parameter settings, as long as Reco and GEP are calibrated in opposite directions. In particular, the estimation of the temperature response of Reco is perturbed by the response of GEP to water vapour pressure deficit (VPD), which can lead to a bias in the estimation of Reco. An afternoon drop of NEE can be caused by a VPD related drop of GEP or by a temperature related increase in Reco. If a regression model only ascribes the effect of Reco to the drop of NEE, this will lead directly to an overestimation of Reco (e.g Figure 3). Secondly, with this method, both Reco and GEP are modeled and based on certain assumptions (e.g. hyperbolic light response of GEP). If the flux-partitioned data is then used to evaluate models, one is essentially comparing two models, which can lead to circular arguments (e.g. a model with the same hyperbolic assumption will be more likely validated than other models). This is a rather fundamental problem and unfortunately virtually excludes using this method when the aim of the flux-partitioning is model evaluation (Reichstein et al., 2005). Using separate estimates of soil and plant respiration is unfortunately the only objective, but costly and complicated, method to circumvent this issue. This involves taking measurements with dark flux chambers at the eddy-correlation site. Combining these flux data, which only comprise of ecosystem respiration fluxes, with soil temperatures an Arrhenius relation with a functional dependence on temperature (Q10) can easily be determined (Dore et al. , 2002; Hendriks et al., 2006)

Lastly, CO2 fluxes near sunrise and sunset are rapidly changing; involve non-stationarity problems and often storage changes occur. If these are not corrected for in the data, incorrect attributions to either respiration or GEP may occur. A final problem arises in the poor availability of soil moisture observations at flux sites, which often makes it virtually impossible to derive meaningful relationships between drought and respiration reduction.

Reichstein et al (2005) suggest that the VPD specific problem can be tackled by including a VPD response of GEP to the regression model.  They develop a flux-partitioning algorithm that first estimates the temperature sensitivity from short-term periods, and then applies this short-term temperature sensitivity to extrapolate the ecosystem respiration from night- to daytime. In this case, one can introduce seasonally varying temperature sensitivity or apply site-specific constant temperature sensitivity.

[INSERT FIGURE 3. HERE]

Another elegant fundamental analysis of temperature sensitivity of respiration can be found in Davidson and Janssens, (2006). They show the complexity of unraveling the “true” temperature sensitivity of respiration from apparent sensitivities that are confounded by various climatic and edaphic factors. In general there appears not yet to be a “best” separation algorithm, but when comparing sites a general applicable methodology may be found in Papale et al. (2006), but also see van der Molen et al (2007).

7.2.2.2 Nighttime fluxes and advection

A particular complex problem is caused by the nighttime build up of CO2 under conditions of low windspeed and low turbulence. This leads potentially to a build up of CO2 near the ground and the possibility of subsequent lateral drainage in terrain with small (1-5% slope) topography. This “night time flux loss” is neither seen by the eddy covariance system at the top of the tower nor by any concentration gradient measurements system on the tower.  Correction techniques are hard to define and generally great care has to be taken when using eddy covariance techniques to estimate night time fluxes. Indications of the potential size of errors vary from 20-100% (e.g. Kruijt et al., 2004, Aubinet et al., 2005, Araújo et al., 2006).  The night time flux loss is a systematic error, and a neglect of say 1 mol C m-2 s-1 over a 12-hour night every night per year leads to a yearly off set of 189 gC m-2,   that is comparable to the net average annual uptake of ecosystems! Standard practice so far has been to eliminate the fluxes observed under low turbulence conditions (established by a friction velocity threshold) and replace these with estimates of night-time respiration based on observations under high turbulence.

More insight into this problem can be gained by writing the conservation equation for a scalar quantity like CO2 in a two-dimensional flow situation (x, y horizontal direction, v=0) as:

I	    II	   III	        IV		  V

where the term denoted I represents the scalar source/sink term that corresponds to the net ecosystem exchange of CO2. Term II represents the turbulent (eddy) flux at height h (the flux which is measured by eddy covariance systems) with w the vertical velocity and the prime denoting the fluctuating part, term III represents the storage of the scalar below the measurement height and IV and V represent the fluxes by horizontal and vertical advection. Under conditions of atmospheric stationarity (no major changes, no trends) and horizontal homogeneity, the last three terms of the right hand side can be ignored, and the eddy flux equals the source/sink term. In practice these conditions are often not met, and the two advection terms become important. For vertical advection, term V, corrections have been suggested by Lee (1998), but these were later criticized by for lumping all errors, including the horizontal advection, into a single term.

Aubinet et al. (2005) analyzed data from several sites with significant topography. They found that the relative importance of advection and vertical storage was not the same for all sites. At steeply sloping sites, advection appeared to be dominant over storage, but at sites with intermediate topography, both processes were equally important. They introduce a new criterion, the advective velocity that combines, amongst other, net radiation (as a proxy for radiative, night-time cooling) and elevation angle, to determine the relative importance of storage versus horizontal advection. Still, the night time flux loss problem is not fully resolved until more precise measurements of horizontal and vertical gradients and wind speed and CO2 concentration become available for more sites. This means that sites prone to advection need additional measurements, and it seems difficult to know a priori which site is prone to which complicating circumstances  Replacement of poor turbulence values with estimated respiration rates still remains the best option for flat sites with low estimated advective velocities, and for sites with high advective velocities additional observations on drainage may be required.

7.3 Magnitude of fluxes

This section gives a brief overview of the magnitude of the fluxes of three main land use types in Europe: forest, grasslands and Northern wetlands (peat lands). Only data obtained by eddy covariance techniques is used. Data based on other methods and further comparison can be found in the respective chapters of this book.

7.3.1 European forests

Van Dijk et al.  (2005) used the results of eddy covariance measurements of 15 sites in Europe to study the spatial variability of forest NEE. In contrast to an earlier study (Valentini et al., 2000), they used more data and a consistent methodology to separate the observed NEE into Gross Primary Production and Respiration. The methodology to split NEE into respiration and assimilation used night time flux data only (Van Dijk and Dolman 2004). We used the current data and reevaluated that work using the Reichstein et al (2005) splitting algorithm for GEP and Reco. On average European forests appear to be a sink of 212 g C m-2 yr-1. The range around this average value is however considerable with a South Italian beech forest taking up 570 g C m-2 yr-1 with Northern forest being small sources of 50 g C m-2 yr-1. The average GEP of European forests is 736 g C m-2 yr-1 and the average respiration 530 g C m-2 yr-1. The average rates given here are not weighted averages taking into account the area covered with a forest species and age structure, but a simple average of the fifteen measurement locations. Direct comparison with the forest inventory measurements is therefore problematic, but an average value of 212 g C m-2 yr-1 for the Net Ecosystem Production of European forest derived from eddy covariance data is reasonable. This value is about twice as high as that obtained from forest inventory data. Indeed Janssens et al (2003) used a factor of 0.47 to scale eddy covariance data to forest inventory data. This factor takes into account that eddy covariance measurements only measure net ecosystem fluxes without disturbance effects such as harvest, forest fires, and wind throws.

The original data published by Valentini et al. (2000) suggested an increasing NEE with decreasing latitude. Subsequent analysis (Valentini, 2003, Janssen et al., 2001, van Dijk and Dolman, 2004) consistently showed the same latitudinal trend.  In Figure 4..a, the mean annual totals of NEE, GEP and R are plotted against latitude as in van Dijk and Dolman (2004). While net uptake tends to decrease towards the pole as found by Valentini et al. (2000), particularly above 600 N, van Dijk and Dolman (2004) concluded that this is not because respiration increases, but because GPP decreases faster than respiration does.

[insert Figure 4. here]

A more detailed analysis of the same data set led van Dijk et al. (2005) to conclude that radiation, temperature and leaf area were the main drivers of the observed variation between sites in GEP, R and NEE.  They also indicated that these drivers were responsible for much of the large scale inter annual variability of the fluxes. Supporting evidence comes from Reichstein et al. (2003) who concluded that respiration could be adequately simulated by a model that contained information on temperature, precipitation and Leaf Area Index.

7.3.2 European grasslands

Soussana et al. (2004, and this book) present first results of the GREENGRASS project concerning the balance of CO2, N2O and CH4 exchanges between grasslands and the atmosphere at nine contrasted European sites. These cover a major climatic gradient over Europe and include four grassland types (sown grass, sown grass-clover, intensive permanent grassland and semi-natural grassland), three types of herbage use (rotational cattle grazing, continuous cattle grazing and mowing) and contrasted N fertilizer supplies. At each site, CO2 fluxes were monitored close to the ground by open and closed path eddy covariance systems. The results show that for CO2 all the grasslands of Europe are a sink in terms of NEE, excluding C losses by harvest and grazing. However, when the emission of other GHGs such as N2O and CH4 is taken into account some of the sites become a smaller sink, but all are able to maintain an overall negative GHG balance. Using data from the Europe, the US and Asia, a close relation between GPP and total ecosystem respiration with respect to the total annual precipitation has been found, while no significant relationship with temperature was obtained.

For managed grasslands in Europe, the annual C exports are approximately of the same magnitude as the NEE observed by eddy covariance, and the uncertainties on the amounts of harvested C and of organic C supplied by manure thus play a significant role in the total uncertainty of the net C storage (i.e. NBP). This implies that the annual NEE cannot be compared directly among sites with contrasted managements (e.g. grazed vs. cut sites) and that in most cases modelling is required to derive a full carbon balance. Chapter (Ciais) gives more details on the amounts of carbon involved in lateral transport.

7.3.3 European peat lands and northern wetlands

There is little data available from eddy covariance measurements of peat land. Most data appear to be related to (sub) arctic ecosystems, and are summarized by Aurela  et al. (2002). In northern areas the length of the growing season puts constraints on the capacity to take up carbon through photosynthesis. Peak uptake is during a very short growing. Table 1 shows the partitioning of NEE into its component fluxes as a function of season for a sub arctic fen in Northern Finland (Aurela et al., 2002). Clearly most of the uptake takes place in the summer. Noticeable are the very substantial

[ INSERT TABLE 1 HERE]

losses of carbon in the winter period of 105 g C m-2. Winter CO2 losses are a significant major gap in the understanding of C losses from these and other arctic ecosystems. Although winter CO2 losses have long been recognized, more recent research indicates that these losses are greater than was previously thought and may be the product of significant respiratory activity during the winter (Aurela et al., 2002), when recently fixed C is respired. The snow cover acts as a very good thermal insulator and provides an environment for respiratory bacteria to continue decomposing organic material, well after the air temperatures have dropped substantially below zero.

In contrast to arctic fens, temperate fens show a more prolonged growing season, resulting in a larger uptake. For a Dutch fen meadow site in the centre of the Netherlands, Hendriks et al. (2007) found that the growing season in 2004, defined as the period when NEE is negative, lasted from 7 April to 14 October and has a net uptake of 543 gC m-2. The NEE for 2004 was cumulated, resulting in a net annual CO2 uptake of 446 +/- 83 gC m-2. The growing season in 2005 lasts from 25 March to 8 September and has a net uptake of 327 gC m-2, while the annual NEE amounts 311 +/- 58 gC m-2. The growing season in 2006 lasts from 25 April to 2 September and has a net uptake of 283 gC m-2, while the annual NEE amounts 232 +/- 58 gC m-2.  Table 2 shows the partitioning of NEE into its component fluxes as a function of season for a temperate fen meadow in the Netherlands (Hendriks et al., 2007).

[ INSERT TABLE 2 HERE]

These numbers are of similar size as those of the forest in the same climate region, but cannot be directly compared as C uptake. A third of the NEE in the peatland is lost as CH4, another fraction lost belowground by water movement. As in the case of grasslands, care must be taken to interpret thee results as suggesting that peat lands act to sequester carbon. In particular methane emissions may act as GHG sources (Chapter ??), and turn the ecosystem into a overall GHG source rather than sink.

7.3.4 Croplands

Croplands play an important part in the carbon cycle, but few direct measurements are available of cropland NEE. Croplands in Europe occupy 126.5 Mha (EU25+ Norway and Switzerland). Modelling studies suggest that European croplands are sources of carbon (Vleeshouwers and Verhagen, 2002, Janssens, 2003). In the past some efforts at measurements of fluxes by eddy covariance were made, but these served largely to estimate water balances at short periods, or when they were executed also for carbon, important losses through soil and residue respiration were not taken into account (Baldocchi et al., 1994).   Particularly for cropland, it is important to cover several full rotation cycles. Using a combination of modeling, yield data and direct eddy covariance estimates Anthoni et al. (2004) were able to determine the Net Ecosystem Exchange and Net Biome Production of a winter wheat crop in East Germany. At 185-245 and -45 +/- 11 gCm-2 yr-1 respectively. Three quarters of the source of carbon was due to soil carbon losses. This latter finding agrees with other estimates that suggest the European agricultural soils to be a source of carbon (e.g. Janssens et al., 2003).  Given the potential to use agricultural practices to mitigate climate change under the Kyoto protocol, it is still surprising how little effort there is put into obtaining reliable eddy covariance dat in combination with yield and soil respiration measurements within Europe.

7.4.  Variability of fluxes

Climate is a main forcing agent for the exchange fluxes and consequently the fluxes show considerable variation from year to year. For example, NEE  in a 60-to 80-year-old deciduous forest in the northeastern USA ranged from 1.2 to 2.5 Mg C ha–1 year–1 during the period from 1993 to 2000 (Barford, 2000). The Inter Annual Variation in NEE is an omnipresent phenomenon observed at almost all of the flux sites across the world (Baldocchi et al. 2001). Understanding the causes of interannual variation may improve our understanding and future predictions of global carbon cycling. Observed interannual variation in NEE at flux sites has been related to several sources (e.g. Hui et al., 2003):

- changes in climatic variables per se such as temperature, cloud cover, summer drought, winter snow depth and the time of snowmelt;

- changes in physiological and ecological processes such as growing season length, natural or artificial changes in stand structure, the timing of leaf  emergence, and coupling between carbon and nutrient cycling with time delay; and

- altered balance between canopy photosynthesis and ecosystem respiration. Empirical evidence suggests that climatic variability among years is among the factors causing interannual variation in NEE, the latter being also strongly influenced by climate-induced changes in ecosystem physiological parameters.

As an example we show in Figure 5 the annual courses of NEE for evergreen coniferous and deciduous forests in Europe. The data show averaged annual trends in NEE together with the standard deviation. The data is based on multi year values of 14 coniferous forest and 9 deciduous forest sites, and includes for evergreen coniferous forest north 26 site years, for temperate 35 site years and southern coniferous forest 19 site years. For deciduous broadleaf northern forest 7 site years, temperate 18 site years and southern broadleaf also 18 site years. The standard deviation around the mean suggests periods of the year when the variability is high, and that will drive the changes in NEE between years. A few issues stand out. The variation in NEE in conifers as indicated by the broken lines, is less than deciduous forest. Particular for deciduous forest, there is a shift in variation from early spring in Northern forests to late summer in Southern European forest. This suggests that in Southern, Mediterranean forests the summer period with erratic rainfall is the most variable (vulnerable) in NEE. A similar trend is seen in the coniferous forest, albeit that the Northern forests do not really show a marked variation around the spring period. In central European, temperate forests we observe a larger variation at the beginning of the growing season than during summer and autumn. Overall Figure 5 suggests that care must be taken in generalizing interannual variation controls in forests. Depending on the climate and forest type, different responses of NEE to interannual variation in climate can be observed.

[ INSERT FIGURE 5 HERE]

The Net ecosystem Exchange response is confounded by the two large fluxes in and out of the ecosystem, the Gross Ecosystem Production and Respiration. We used the Reichstein et al. (2005) algorithm to separate the fluxes of the same data as used in figure 5 to plot the GEP and Respiration (Re) separately in Figure 6 and 7. It appears that for GEP of coniferous forests, the largest variation is found in the southern forest in the summer. We suggest that drought responses to variable rainfall are the main cause for this. Surprising is the low variation of GEP in spring for Northern forests. Deciduous Northern forests show very large variation during the spring leaf budding period. The variable timing of the start of the growing season with associated large changes in phenology finds expression in the large GEP variation around spring. Both deciduous and coniferous temperate forests show little variation throughout the year. There is some additional variability around the late summer, autumns for southern deciduous forest, that, as for coniferous forest, might be associated with periodic drought conditions. A further noteworthy phenomenon is that the maximum average fluxes for deciduous forest are always

[ INSERT FIGURE 6 HERE]

[INSERT FIGURE 7. HERE]

larger than those for coniferous forest. This holds in all climate zones and only for GEP and NEE, suggesting that variation in NEE of European forest is dominated by changes in GEP.

The variation in respiration fluxes is generally large and of similar order as the average flux. The largest variation is found in summer for the coniferous forest, suggesting temperature related effects in the north and drought effects in the south. Of the deciduous forests the variation in respiration in the Northern forests is very large, with apparently surprisingly little variation in the southern deciduous forests. For the latter forest the total respiration flux is however relatively small compared to the temperate and northern forests. In fact, the variation in the summer and late autumn is still up to 50% of the mean value and thus in relative terms quite large. For all southern forests drought responses to respiration would seem a likely explanation. Differences in seasonal respiration between deciduous and coniferous forest might be related to changes in the balance between heterotrophic, soil and autotrophic respiration, but this remains a speculation in the absence of full carbon balance observations.

7.5. Vulnerability

7.5.1 Vulnerability from eddy covariance flux variability

Modeling results (Cramer et al., 2001, Cox et al., 2000, Friedlingstein et al., 2001, 2006) suggest that the sink strength of the biosphere may deteriorate and change from a sink to a source. This may have potentially dramatic impacts on the CO2 content of the future atmosphere. This potential for positive feedback is based on the different temperature sensitivity of assimilation and respiration. This hypothesis is contested by both short and long term experiments that show acclimation to higher temperatures. Knorr et al. (2005) present an analysis of the temperature sensitivity. They conclude that the hypothesis of a positive feedback of increased C-losses from soils due to rising temperature is fully consistent with the current experimental evidence.

Eddy covariance data can be helpful in two aspects: showing where typical vulnerabilities of NEE lie, and to check and validate models that are used to predict future vulnerabilities. Table 3 shows the sensitivity of the net ecosystem exchange of carbon of European ecosystems to changes in fire frequency, drought, length of growing season, temperature, vapor pressure deficit and snow coverage. This table is based on eddy covariance data of the FLUXNET network (e.g. Baldocchi and Valentini (2004) and this chapter).

The main sensitivity of temperate forest appears to be in the response to the length of the growing season. An early spring adds quickly a number of extra number of day of carbon uptake to the season, amplifying the sink strength of these forests. The IAV at this period of time suggest also that this is important (Figure 4 and see also Figure 10.). Luyssaert et al (2006) suggest however that these extra events hardly ever dominate the interannual variability.On the negative side increasing drought stress in summer season may decrease the uptake capacity (Southern forest in Figure 6.). The Scandinavian boreal forests are very sensitive to small changes in temperature and fire frequency, whereas Mediterranean forest and woodland are very sensitive to drought and fire stress. Fires in the boreal regions however, have a larger effect on the carbon emissions as the boreal forests contain much more biomass than the Mediterranean woodlands. Wetlands appear to be mostly sensitive to temperature change, although changes in the hydrological cycle may enhance such sensitivities. In general these vulnerabilities coincide with the areas where the fluxes show their largest variation.

[ INSERT TABLE 3 HERE]

7.5.2 The value of a network, the summer heatwave of 2003 in Central Europe.

Europe experienced a particularly extreme climate anomaly during 2003, with July temperatures up to 6 °C above long-term means, and annual precipitation deficits up to 300 mm yr-1, 50% below the average. The presence of an extensive network of eddy covariance sites with continuous records of CO2, water, and energy fluxes allowed Ciais et al. (2005) to assess the impact of such extreme events on the carbon balance. They analysed CO2 fluxes from 14 forest sites and one grassland site for 2002–2003. Hourly fluxes of photosynthesis (gross primary productivity, GPP) and total ecosystem respiration (Reco) were separated from the observed NEE  fluxes using the same method for each site. In general a spatially uniform response of GPP to the abnormal conditions of 2003 would not be expected, given the inter-site differences in drought duration and intensity, soil characteristics, vegetation state, and species-specific responses to climate variation. Yet, as Figure 8 clearly shows that nearly all sites in the affected area experienced a significant GPP reduction in 2003 compared to 2002. The GPP drop coincides with reduced evapotranspiration and soil drying due to the rainfall deficit. Generally, below a threshold of 0.4 in relative extractable water, water stress occurs, causing both GPP and transpiration to decrease in response to stomatal closure . Particularly large reductions in GPP were found in temperate deciduous beech and northern Mediterranean forests (Hesse, Roccarespampani, San Rossore), together with reductions in canopy conductance (in 2003, conductance reached only 15% of its 2002 value at Hesse). These productive temperate and Mediterranean forests were greatly affected by extreme drought and/or heat. Moreover, the GPP did not entirely recover from the summer stress during the remainder of the growing season. Southern evergreen forest sites El Saler (pine) and Puéchabon (oak) also experienced a reduction in GPP during the heatwave. Southern forest sites with an herbaceous layer that normally dries in summer were less affected by the heatwave and drought (Pianosa). It it noteworthy that forest sites in the Netherlands and Denmark showed higher NEE and GPP than in 2002, because the

[INSERT FIGURE 8. HERE]

temperature stimulated GPP more than Reco in these sites, where drought appeared less important. These results reinforce our conclusions above from the analysis of interannual variability, that show high variability in summer/autumn for Southern forests. They also suggest that the 2003 response is a mix in direct response to drought that altered the partitioning between TER and GPP.

7.6. Representativity and network design

7.6.1 Representativity

The key issue that stands out is the assessment of the representativeness of the network. How good is the current FLUXNET network distributed along biomes and does it capture the climatic (interannual) variability within a biome, and perhaps more importantly, does it capture the intra biome variability? There is little known about the optimum structure of a (the FLUXNET) network and it is likely that what defines an optimum depends on the type of questions asked. The vulnerability study of Ciais et al. (2005) showed the value of a dense network to pick up synoptic scale disturbances. An assessment of the Ameriflux network suggested that coverage of the important climate zones and ecosystems was achieved. Globally this is unlikely to be the case, given the different densities of the network in parts of the world other than Europe and the US (see Figure 1). The current network is not well distributed, there is severe overlapping in Western Europe, US, and there is a dominance of forest sites that are in the high growth rate part of their life cycle.

The CarboEurope-IP plan gives an indication of the European distribution, both in number of sites and when weighted with NBP estimates. For this example the distribution is good, although for a larger EU, the grassland and cropland sites would be underrepresented. Note also that this comparison only indicates that fractional coverage is correct, i.e. the percentage of forest sites equals the percentage forest area in Europe. It tells us nothing of how dense such a network should be to pick up synoptic scale disturbances, changes in age of forest, management etc. or about the number of sites and their spatial distribution required to sample a single land use type.

[INSERT FIGURE 9. HERE]

Globally most measurement sites are located in the deciduous forest type, and this type seems to be overrepresented with more than 40% of all sites, compared to a global coverage of the biome of only 16%. These numbers can also depend considerably on the land use map used to derive the statistics. When the productivity of the different vegetation types is taken into account, forests (with most sites) have the highest Net Primary Production (NPP). Viewed in this context, the high percentage of forest sites in the FLUXNET is less of a problem. Also the variation of the fluxes between the forests sites is high. The distribution furthermore covers quite reasonably the global climate space, as defined by annual rainfall and temperature. Most of the sites are located in the temperate climate region with latitudes of 15 to 50 degrees (Central Europe and Northern America). There are almost no sites located in the more extreme cold, or warm, wet regions.

An important question is whether there is more variability between biomes, or biomes in different climates than within a single biome.  If there is more or similar variation within a single biome or climate zone, than we may need to deploy more resources in obtaining insight into this variability than trying to cover the full climate space, as generally is noted in discussion on the design of the flux network. We may investigate this issue by looking at the variability of annual estimates of flux sites. If the variability between sites in a given climate band is larger than that between climate band, we need to conclude that issues like land use history and management play a more important role than clime and vegetation type per se. The implication for network design follows immediately.

For 63 FLUXNET sites there is enough data available to calculate the average annual Net Ecosystem Exchange (NEE) and the annual evaporation flux. We plotted these against latitude as a proxy for the combined effect of the annual average temperature and the annual precipitation (Figure 10). From this figure it is evident that there is as much variation between sites in a single latitude band as between latitude bands. When separating NEE into Gross Primary Production and respiration a single picture emerges. Factors, such as land use history and management and soil type, are thus likely to play a more important role in determining annual NEE, GPP and R than climate. Analysis of two towers in Brazil 30 km from each other apart, suggest that flux sites will never be able to capture this topographically induced local variation (Harris et al., 2004).

[INSERT FIGURE 10. HERE]

The interannual variation at a single site is also often much larger or of similar size as the variation between sites. This suggests that within biome variability is not yet adequately captured by the network. High spatial resolution satellite observation would be required to correctly interpret the causes of this variation. It also suggests that long term observations are key to understanding the variability of the global carbon cycle.

7.6.2 Network design

We can conclude that the coverage of FLUXNET sites seems appropriate in terms of climate space  (P,T) and appears generally appropriate in terms of fractional coverage of land use types, be it with an over representation of mid-latitude forest sites. The current data however also suggest that there is more variability between sites at climate bands than between adjacent climates. Although various studies attribute key controlling factors to the NEE such as radiation and temperature and moisture (e.g. Van Dijk et al., 2005), these appear to be true only in a general sense. When such data or parameters are used to derive fluxes at similar sites, differences will occur that are related to small scale differences in land use history, management and climate. Particularly in Europe, land use history and small scale climate differences are important. Information on management practice, previous land use history is essential to interpret carbon fluxes. This may imply that the current setup needs to redesigned in such a way, that the network still captures the climatic variability, but that is also samples within supersites, different forms of land management, history, age class of the forests. Rather than expanding the network into new areas, an intensification of the observation in the vicinity of a few single sites may be appropriate.

To make any further progress in designing the network further, synthetic type data studies need to be performed. For instance, studies could be performed on the effect of loosing information in a data assimilation network. This can be done using more sites within a single biome in a data assimilation study, and showing whether there is redundancy or under-representation in the network. Even with relatively simple data assimilation techniques such as the neural networks used by Papale and Valentini et al (2003), these studies could be performed.

7.7. Conclusions and recommendations

In the United States and Europe, the flux measurements have long started to form an integral part of the experimental programs that are based around the multiple constraint approach to close the carbon balance. The key rationale behind these efforts is that, ultimately, fluxes from the land, both the slow and the fast ones, affect atmospheric concentrations of CO2 at a regional scale and that the slow and fast fluxes ideally should match. Currently, the results of eddy-correlation measurements are considered reliable, when they meet the standards of Carboeurope or FLUXNET minimum requirements. These requirements that have been developed in the past decades comprise of necessary corrections on the measurements and gap filling strategies.

Flux towers yield a consistent picture of the controls on NEE at local scale and have improved our process understanding tremendously. They do not in isolation contribute to the Kyoto goals of monitoring, as is often erroneously claimed. On the contrary, as correctly pointed out by Körner (2003), their strength is in the combined use as a network or in a multiple constrains approach. Eddy covariance networks have substantially improved our understanding of biosphere-atmosphere exchange of carbon, water and energy. Additionally, flux towers play an extra role of importance in measurement programs, because they provide data for model validation and comparison (Van Dijk et al., 2005) as well as vulnerability analysis (Ciais, et al. 2005). Various methods have been developed for the separation of main flux components as well as for the calculating nighttime fluxes, although this last topic subject is still not fully solved. Especially in mountainous areas and areas with strong advection more effort should be put in interpreting the nighttime data. Magnitude and variation of fluxes is well described for a large part of Europe. The existence and quality of the flux network plus the databases reflect the evolvement of the networks and maturity of the field. None the less, improvements can be made on the representativity of the network design (land use types and geographical spread) as well as descriptions of land use history of the sites.

Despite the complexity of the issue, it is possible to set a number of criteria for future new site level studies:

- They should be operated as multi tower sites to sample more than one land use (history) type, say peat lands and different types of crops within a single climate area.

- They should adhere to the full Carboeurope, FLUXNET minimum requirements for measurements, calculations and corrections. This pertains particular to the execution of soil moisture and biomass measurements.

- More sites could be established in the former East European countries as well as the Balkans and Greece.

- They should run al least for a minimum of 5 years to capture interannual variability in climate.

- An effort should be put into generating a realistic (carbon) land use history for all existing sites as well as future sites.

- Data submission to a central data base should be timely and consistently.

- Measurements in adjacent ecosystems to cover the full development cycle or management range of ecosystems (e.g. forest age classes, major crop types).

The eddy covariance network can provide uniform datasets for sites now and in the future. But possible problems here are the predictability of the future status of sites and thus the continuity of the current network. For most sites there is no long-term plan for measurements. Most sites are used for several years within a project framework and are either kept operational with new funding in another project, are kept partly operational, or are abandoned. International organizations (GCOS, IGCO, GCP) are actively seeking support to keep an optimum number of sites operational, well after the first Kyoto commitment period.

References

Anthoni, P.M., Freibauer, A., Kolle, O.  and Schulze, E-D., 2004. Winter wheat carbon exchange in Thuringia, Germany, Agricultural and Forest Meteorology, Volume 121, 55-67.

Araújo A. C., A. D. Nobre, B. Kruijt, J. A. Elbers, R. Dallarosa, P. Stefani,

C. von Randow, A. O. Manzi, A. D. Culf, J. H. C. Gash, R. Valentini and P. Kabat, 2002. Comparative measurements of carbon dioxide fluxes from two nearby towers in a central Amazonian rainforest: The Manaus LBA site, J. Geophys. Res., 107 (D20), 8090, doi:10.1029/2001JD000676.

Araújo, A.C., Kruijt, B, Nobre, A.D., Dolman, A.J., Waterloo, M.J.,  Moors, E.J., de Souza, J.S., 2006. CO2 underneath a tropical forest canopy and nocturnal accumulation in a topographical gradient. Ecol. Applic. Submitted.

Aubinet, M., A. Grelle, A. Ibrom, Ü. Rannik, J. Moncrieff, T. Foken, A. S. Kowalski, P. H. Martin, P. Berbigier, Ch. Bernhofer, R. Clement, J. Elbers, A. Granier, T. Grünwald, K. Morgenstern, K. Pilegaard, C. Rebmann, W. Snijders, R. Valentini, and T. Vesala, 2000, Estimates of the annual net carbon and water exchange of forests: The EuroFlux methodology, Advances in Ecological Research, 30, 113−176.

Aubinet, M.; Berbigier, P.; Bernhofer, Ch.; Cescatti, A.; Feigenwinter, C.; Granier, A.; Grünwald, Th.; Havrankova, K.; Heinesch, B.; Longdoz, B.; Marcolla, B.; Montagnani, L.; Sedlak, P., 2005 Comparing CO2 Storage and Advection Conditions at Night at Different Carboeuroflux Sites. Bound. Lay. Meteorol. 116 pp. 63-93(31).

Aurela M., T. Laurila, and, J.-P. Tuovinen, 2002. Annual CO2 balance of a subarctic fen in northern  Europe: Importance of the wintertime efflux, J. Geophys. Res., 107 (D21), 4607, doi:10.1029/2002JD002055.

Baldocchi, D. 1994. A comparative-study of mass and energy-exchange rates over a closed C-3 (wheat) and an open C-4 (corn) crop. 2. CO2 exchange and water-use efficiency. Agric. For. Meteorol. 67,  291–321.

Baldocchi, D.  and Valentini, R., 2004. Geographic and Temporal Variation of Carbon Exchange by Ecosystems and Their Sensitivity to Environmental Perturbations In : Field, C. B. and Raupach, M. (Eds.) The Global Carbon Cycle, Island Press, Washington, 2004.

Baldocchi, D.D., R.Valentini, S.R. Running, W. Oechel and R. Dahlman. 1996. Strategies for measuring and modelling CO2 and water vapor fluxes over terrestrial ecosystems. Global Change Biology.2: 159-168.

Baldocchi, DD, Falge, E, Gu, L., R. Olson, D. Hollinger, S. Running, P. Anthoni, Ch. Bernhofer, K. Davis, J. Fuentes, A. Goldstein, G. Katul, B. Law, X. Lee, Y. Malhi, T. Meyers, J.W. Munger, W. Oechel, K. Pilegaard, H.P. Schmid, R. Valentini, S. Verma, T. Vesala, K. Wilson and S. Wofsy. 2001. FLUXNET: A New Tool to Study the Temporal and Spatial Variability of Ecosystem-Scale Carbon Dioxide, Water Vapor and Energy Flux Densities. Bulletin of the American Meteorological Society 82: 2415-2435.

Barford, C. C., Wofsy, S. C., Goulden, M. L., Munger, J. W., Pyle, E. H., Urbanski, S. P., Hutyra, L., Saleska, S. R., Fitzjarrald, D., and Moore, K, 2001. Factors controlling long- and short-term sequestration of atmospheric CO2 in a mid-latitude forest, Science, 294, 1688–1691.

Ciais, Ph., M. Reichstein, N. Viovy, A. Granier, J. Og|[eacute]|e, V. Allard, M. Aubinet, N. Buchmann, Chr. Bernhofer, A. Carrara, F. Chevallier, N. De Noblet, A. D. Friend, P. Friedlingstein, T. Gr|[uuml]|nwald, B. Heinesch, P. Keronen, A. Knohl, G. Krinner, D. Loustau, G. Manca, G. Matteucci, F. Miglietta, J. M. Ourcival, D. Papale, K. Pilegaard, S. Rambal, G. Seufert, J. F. Soussana, M. J. Sanz, E. D. Schulze, T. Vesala and R. Valentini, 2005. Europe-wide reduction in primary productivity caused by the heat and drought in 2003. Nature 437, 529-533.

Cox, P. M., Betts, R. A., Jones, C. D., Spall, S. A. and Totterdell, I. J., 2000. : Acceleration of global warming due to carbon-cycle feedbacks in a coupled climate model, Nature, 408, 184-187.

Cramer, Wolfgang, Bondeau, Alberte, Woodward, F. Ian, Prentice, I. Colin, Betts, Richard A., Brovkin, Victor, Cox, Peter M., Fisher, Veronica, Foley, Jonathan A., Friend, Andrew D., Kucharik, Chris, Lomas, Mark R., Ramankutty, Navin, Sitch, Stephen, Smith, Benjamin, White, Andrew & Young-Molling, Christine, 2001. Global response of terrestrial ecosystem structure and function to CO2 and climate change: results from six dynamic global vegetation models. Global Change Biology 7 (4), 357-373. doi: 10.1046/j.1365-2486.2001.00383.x.

Davidson E.A., and I.A. Janssens. 2006. Temperature sensitivity of soil carbon decomposition and  feedbacks to climate change. Nature 440(7081):165-173.

Dolman, A. J. , Maximov, T. C., Moors, E. J., Maximov, A. P., Elbers, J. A., Kononov, A. V., Waterloo, M. J. and van der Molen, M. K., 2004 Net ecosystem exchange of carbon dioxide and water of far eastern Siberian Larch (Larix cajanderii) on permafrost. Biogeosciences, Vol. 1, pp 133-146.

Dolman, A.J., E.-D. Schulze, R. Valentini, 2003. Analyzing Carbon Flux Measurements Science, 301: 916-917.

Dore, S., G. J. Hymus, et al. (2003). "Cross validation of open-top chamber and eddy covariance measurements of ecosystem CO2 exchange in a Florida scrub-oak ecosystem." Global Change Biology 9(1): 84-95.

Finnigan, JJ, Clement, R, Mahli, Y, Leuning, R, and Cleugh, H., 2003. A re-evaluation of long-term ux measurement techniques: Part 1. Averaging and coordinate rotation. Boundary-Layer Meteorology107: 1-48.

Friedlingstein, P., L. Bopp, P. Ciais, J.-L. Dufresne, L. Fairhead, H. LeTreut, P. Monfray, J. Orr, 2001. Positive feedback between future climate change and the carbon cycle, Geophys. Res. Lett., 28(8), 1543-1546, 10.1029/2000GL012015.

Friedlingstein, P., et al., 2006: Climate-carbon cycle feedback analysis: results from the C4MIP model intercomparison. J. Clim., 19, 3337–3353.

Hendriks, D.M.D., J. van Huissteden, A.J. Dolman, M.K. van der Molen, 2006 The full greenhouse gas balance of an abandoned peat meadow. BiogeoSciences Discussion 277-316. in: Greenhouse gases in the northern hemisphere Editor(s): A. Freibauer, R. Valentini, A.J.. Dolman, and I. Janssens.

Harris,P. P., Huntingford,C., Gash,J. H. C., Hodnett,M. G., Cox,P. M., Malhi,Y., Araújo,A. C., Calibration of a land-surface model using data from primary forest sites in Amazonia, 2004. Theoretical and Applied Climatology, 78, 27-45, http://dx.doi.org/10.1007/s00704-004-0042-y, DOI 10.1007/s00704-004-0042-y.

Hui, D, Luo, Y. and Katul, G., 2003. Partitioning interannual variability in net ecosystem exchange between climatic variability and functional change. Tree Physiology 23, 433–442

Gash, J.H.C. and  Dolman, A.J., 2003. Sonic anemometer (co)sine response and flux measurement I. The potential for (co)sine error to affect sonic anemometer-based flux measurements. Agricultural and Forest.

Janssens, I. A., H. Lankreijer, G. Mateucci, A. S. Kowalski, N. Buchmann, D. Epron, K. Pilegaard, W. Kutsch, B. Longdoz, T. Grünwald, L. Montagnani, S. Dore, C. Rebmann, E. J. Moors, A. Grelle, Ü. Rannik, K. Morgenstern, S. Oltchev, R. Clement, J. Guðmundsson, S. Minerbi, P. Berbigier, A. Ibrom, J. Moncrieff, M. Aubinet, Ch. Bernhofer, N. O. Jensen, T. Vesala , A. Granier, E.-D. Schulze, A. Lindroth, A. J. Dolman, P. G. Jarvis, R. Ceulemans, and R. Valentini, 2001. Productivity and disturbance overshadow temperature in determining soil and ecosystem respiration across European forests, Global Change Biology, 7, 269–278.

Janssens, I.A., Freibauer, A., Ciais, P., Smith, P., Nabuurs, G.-J., Folberth, G., Schlamadinger, B., Hutjes, R.W.A., Ceulemans, R., Schulze, E.-D., Valentini, R., Dolman, H. 2003. Europe’s biosphere absorbs 7-12% of anthrogogenic carbon emissions. Science 300, 1538-1542.

Kaimal, J.C. and Finnigan, J.J., 1994. Atmospheric Boundary Layer Flows: Their Structure and Measurement. Oxford University Press, New York, 289 pp.

Knorr, W., Prentice, I.C., House, J.I., and Holland, E.A., 2005. Long term sensitivity of soil carbon turnover to global warming. Nature 433: 298-301.

Knorr, W., Jens Kattge, J., 2005. Inversion of terrestrial ecosystem model parameter values against eddy covariance measurements by Monte Carlo sampling. Global Change Biology  11:8 1333.

Körner, C. 2003.: Slow in, rapid out – carbon flux studies and Kyoto targets, Science, 300, 1242-1243.

Kruijt, B., J. A. Elbers, C. von Randow, A. C. Araújo, P. J. Oliveira, A. Culf, A. O. Manzi, A. D. Nobre, P. Kabat, and E. J. Moors. 2004. The robustness of eddy covariance fluxes for Amazon rain forest conditions. Ecological Applications 14:S101–S113.

Lee, X., 1998. On micrometeorological observations of surface-air exchange over tall vegetation. Agricultural And Forest Meteorology, 91(1-2): 39-49.

Lee, X. Massmann, W.B. and Law, B., 2004. Handbook of Micrometeorology: A Guide for Surface Flux Measurement and Analysis. Kluwer Ac. Press, 250 pp.

Luyssaert, S., I.A. Janssens, M. Sulkava, D. Papale, A.J. Dolman, M. Reichstein, T. Suni, J. Hollmén, T. Vesala, D. Lousteau, B. Law and E.J. Moors, 2006.  Photosynthesis drives interannual variability in net carbon-exchange of pine forests at different latitudes Global change Biology (in review).

Moncrieff,J.B., J. M. Massheder, H. de Bruin, J. Elbers, T. Friborg, B. Heusinkveld, P. Kabat, S. Scott, H. Soegaard and A. Verhoef, 1998. A system to measure surface fluxes of momentum, sensible heat, water vapour and carbon dioxide, Journal of Hydrology, Volumes 188-189, 589-611.

Monteith, 1965. Evaporation and environment. In Fogg, G.E. (Ed). The state and movement of water in living organisms. Cambrig. Univ. Press, Cambridge, UK. Pp 205-234.

Monteith, J.L., 1976 (Ed). Vegetation and the Atmosphere - Volume 1: Principles. Ac. Press. London, NY.

Papale, D. and Valentini, R., 2003. A new assessment of European forests carbon exchanges by eddy fluxes and artificial neural network spatialization, Global Change Biol., 9, 525–535.

Papale, D., M. Reichstein, M. Aubinet, E. Canfora, C. Bernhofer, W. Kutsch, B. Longdoz, S. Rambal, R. Valentini, T. Vesala and D. Yakir, 2006. Towards a standardized processing of Net Ecosystem Exchange measured with eddy covariance technique: algorithms and uncertainty estimation. Biogeosciences, 3, 571–583.

Reichstein, M., Rey, A., Freibauer, A., Tenhunen, J., Valentini, R., Banza, J., Casals, P., Cheng, Y., Grünzweig, J., Irvine, J., Joffre, R., Law, B., Loustau, D., Miglietta, F., Oechel, W., Ourcival, J.-M., Pereira, J., Peressotti, A., Ponti, F., Qi, Y., Rambal, S., Rayment, M., Romanya, J., Rossi, F., Tedeschi, V., Tirone, G., Xu, M., Yakir, D. (2003) Modelling temporal and large-scale spatial variability of soil respiration from soil water availability, temperature and vegetation productivity indices. Global Biogeochemical Cycles, 17, 1104, doi:10.1029/2003GB002035.

Reichstein, M., Falge, E., Baldocchi, D., Papale, D., Aubinet, M., Berbigier, P., Bernhofer, C., Buchmann, N., Gilmanov, T., Granier, A., Gr ¨unwald, T., Havr ´ankov´ a, K., Ilvesniemi, H., 30 Janous, D., Knohl, A., Laurila, T., Lohila, A., Loustau, D., Matteucci, G., Meyers, T., Miglietta, F., Ourcival, J.-M., Pumpanen, J., Rambal, S., Rotenberg, E., Sanz, M., Tenhunen, J., Seufert, G., Vaccari, F., Vesala, T., Yakir, D., and Valentini, R., 2005. On the separation of net ecosystem exchange into assimilation and  cosystem respiration: review and improved algorithm, Global Change Biol., 11, 1424–1439.

Soussana J-F et al., 2004. Greenhouse gas emissions from European grasslands. Report 4/2004 CarboEurope-GHG Concerted action. 93 pp.

Valentini, R. (Ed.), 2003. Fluxes of Carbon, Water and Energy of European Forests. Ecological Studies Series 163, Springer-Verlag, Heidelberg.

Valentini, R., Matteucci, G., Dolman, A.J., et al., 2000. Respiration as the main determinant of carbon balance in European forests. Nature 404: 861-865.

Van der Molen, M.K., J.H.C. Gash, J.A. Elbers (2004). Sonic Anemometer cosine response and flux measurement: II The effect of introducing an angle of attack dependent calibration. Agr. For. Meteorol. 122, 95-109.

Van der Molen et al. (2007) The carbon balance of the Boreal Eurasia consolidated with

eddy covariance observations. Submitted Glob. Chang. Biology.

Van Dijk, Albert I. J. M. & Dolman, A. J. (2004) Estimates of CO2 uptake and release among European forests based on eddy covariance data. Global Change Biology 10 (9), 1445-1459.doi: 10.1111/j.1365-2486.2004.00831.x.

Van Dijk A. I. J. M., A. J. Dolman, E.-D. Schulze (2005), Radiation, temperature, and leaf area explain ecosystem carbon fluxes in boreal and temperate European forests, Global Biogeochem. Cycles, 19, GB2029, doi:10.1029/2004GB002417.

Vleeshouwers, L. M  and Verhagen A. 2002. Carbon emission and sequestration by

agricultural land use: a model study for Europe. Global Change Biology  8:6 519

Wichura B, Foken T., 1995. Anwendung integraler Turbulenzcharakteristiken zur Bestimmung von Beimengungen in der Bodenschicht der Atmosphäre. DWD,Abteilung Forschung,Arbeitsergebnisse no 29, 52 pp.

FIGURE CAPTIONS:

Figure 1.	Locations of the 266 FLUXNET measurement sites available on the 1st of January 2005.

Figure 2. 	The long winding road of eddy covariance in practice, showing the sequence of data

aquisition, treatment and control. Note that to arrive at  30 min data several corrections need to be applied. To progress towards annual sums, gap filling of missing data becomes an issue (from Aubinet al et., 2000).

Figure 3.. 	Schematic representation of the various confounding effects introduced into a temperature dependence of ecosystem respiration derived from annual data. Triangles and squares are hypothetical data for summer active and summer passive ecosystems, respectively. The true middle curve is the sensitivity of respiration to short-term variations; the other two are based on long-term data input as confounded by other seasonally varying factors, resulting in an overestimation of the temperature sensitivity in summer active and underestimation in summer passive ecosystems (from Reichstein et al., 2005).

Figure 4. .	Net Ecosystem Exchange and respiration and Assimilation versus latitude for 15 European forests (4a) and respiration (4b) as a function of gross ecosystem production of the same set of 15 forests.

Figure 5.	Mean annual trends in NEE for European forests, shown are geographical averages for deciduous and coniferous forests (solid line) and standard deviation around the average (broken lines).

Figure 6.	Mean annual trends in GEP for European forests, shown are geographical averages for deciduous and coniferous forests.  Data from ….

Figure 7..	Mean annual trends in respiration, Re for European forests, shown are geographical averages for deciduous and coniferous forests.  Data from ….

Figure 8.. 	Eddy covariance forest site measurements of GPP, Reco and NEP from July to September in 2002 and in 2003 (from Ciais et al., 2005) . Circles, NEP, squares GPP, triangles Reco, open values refer to 2002,

Figure 9.. 	Distribution of Flux sites in the CarboEurope-IP and representativeness according to biome and NBP distribution in the former EU-15 (from CarboEurope-IP, Annex of work).

Figure 10. . Distribution of GEP. NEE and LE fluxes (annual average values) with latitude for Fluxnet sites

(dots, error bars indicate standard deviations of single sites for multiple years

TABLE CAPTIONS

Table 1.. 	Carbon uptake (g C m-2) for several periods of the year for a subarctic fen (after Aurela, 2002)

Table 2. 	CO2 fluxes (g C m-2) for growing and non-growing periods of the 2004, 2005 and 2006 for a temperate fen meadow (after Hendriks et al., 2007)

Table 3.. 	Sensitivity of net ecosystem exchange to environmental change. The plusses indicate weak or strong sensitivity; the minus sign indicates no sensitivity (Note that sensitivity to fires indicates primarily a sensitivity of the Net Biome Exchange, and less so of NEE).* sensitivity of natural peat land to fires is small (only drained peatlands burn, the others are too wet, or only burn in the top few centimeters)

FIGURES

Figure 1.



Figure 2.


Figure 3.

Figure 4.
Figure 5.


Figure 6.


Figure 7.

Figure 8.

Figure 9.


Figure 10.

Table 1

|  | Thaw | Preleaf | Summer | Autumn | Winter | Year |
| --- | --- | --- | --- | --- | --- | --- |
| Length of period | 23 | 45 | 71 | 40 | 186 | 365 |
| DOY of period | 112-134 | 135-179 | 180-250 | 251-290 | 291-111 | 112-111 |
| NEE | 17.1 | 31.5 | -273.8 | 51.8 | 105.2 | -68.1 |
| GEP | -7.9 | -84.6 | -786.1 | -31.4 | 0 | -910 |
| R | 26.7 | 118.9 | 515.8 | 81.9 | 109.5 | 852.8 |

.

Table 2

Table 3

|  | Annual NEE (g C m-2 year-1) | Max. daily uptake (g C m-2 day-1) | Sensitivity to environment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vegetation type |  |  | Fire | Drought | Length of growing season | Tempe-rature | Vapor pressure deficit | Snow |
| Temperate broadleaf | 100-600 | 6 | + | + | ++ | + | + | + |
| Temperate needle leaf | 300-400 | 4 | + | ++ | ++ | + | ++ | - |
| Boreal needle leaf | 100-200 | 3 | +++ | + | ++ | +++ | +++ | ++ |
| Mediterranean woodland | 300 | 0.7 | +++ | +++ | + | - | ++ | - |
| Northern wetlands | 100 | 0.8 | -/+ | - /+ | ++ | +++ | - | ++ |
| Agriculture | -100 | 4 | n/a | ++ | + | +/- | - | - |
| Peat lands | 250 | 4 | n/a* | ++ | + | +/- | - | +/- |
| Grassland | 100 | 4 | + | ++ | + | +/- | - | +/- |