# Chapter 15

# Designing an observation strategy for N2O

Annette Freibauer

Max-Planck-Institute for Biogeochemistry

P.O. Box 100164

07701 Jena

Germany

E-mail: afreib@bgc-jena.mpg.de

## 15.1 Introduction

Nitrous oxide (N2O) is a powerful greenhouse gas (GHG) with a global warming potential (GWP) of 296 in relation to CO2 (IPCC 2001). In the context of the United Nations Framework Convention on Climate Change (UNFCCC) a GWP value of 310 is used (IPCC 1996). For easy comparison, we express N2O fluxes in units of nitrogen (N) and in CO2-carbon equivalents (Ceq), applying the politically relevant GWP of 310. One kg N2O-N is equivalent to 133 kg Ceq. Nitrous oxide is also involved in the depletion of the stratospheric ozone layer.

The atmospheric concentration of N2O has increased from 270 ppb to more than 320 ppb today (IPCC 2001). The lifetime of N2O in the atmosphere is about 114 years (IPCC 2001). Globally, the microbial processes of nitrification and denitrification in soils constitute the largest source of N2O. Global anthropogenic N2O emissions are estimated to be 6.9 Tg N yr-1 (0.87 Gt Ceq yr-1, average in 1990s), more than half of which result from fertilizer application to soils. Human activities contribute about 40 % to the total global N2O emissions. N2O accounts for 6 % of the global anthropogenic GHG emissions (IPCC 2001).

N2O is photochemically destroyed in the stratosphere through reaction with excited oxygen atoms. There is evidence that a large proportion of the current N2O emissions is removed in the stratosphere, but the global N2O sources have not yet been fully identified (IPCC 2001).

This chapter will give an overview of the present N2O emissions in Europe, and then focus on the major source, the soilborne N2O emissions. The spatio-temporal variation of soil N2O fluxes and underlying biogoechemical processes will be described, as well as approaches to observe and quantify N2O fluxes, associated uncertainties and ways towards a systematic operational N2O monitoring. In the latter part, N2O emissions from other sources than soils will be included.

## 15. 2 N2O emissions in Europe

According to the National Communications under the United Nations Framework Convention on Climate Change (UNFCCC, http://ghg.unfccc.int/index.html) the European Community (EU-25) emitted 0.83 Tg N as anthropogenic N2O in 2004 (Table 1). Emissions tended to decrease during the past 15 years. The N2O emission in 2004 is equivalent to 13% of the global anthropogenic N2O emissions and 7% of Europe’s direct GHG emissions. The national communications contain all N2O emissions from industry, a conservative estimate of N2O emissions from road traffic and additional N2O emissions from soil that can be directly attributed to human N input, including enhanced N2O emissions in natural ecosystems due to atmospheric N deposition. For comparison, estimates of the total soilborne N2O emissions are shown in Table 1. The so-called “natural background N2O emissions” from agricultural soils are excluded from this calculation. The reported N2O emissions are slightly lower than independent model-based estimates of the N2O emissions from agricultural soils and forests (Table 1), but uncertainties in both estimates are high.

The most important sources of N2O in Europe are agricultural soils, followed by industrial nylon and nitric acid production (Table 1). The anthropogenic emissions of N2O from soils are caused by fertiliser use, nitrogen fixation by agricultural crops, and atmospheric nitrogen deposition (Nevison and Holland 1997). Soils contribute by far the largest uncertainty to anthropogenic N2O emission estimates (Table 1). In total, N2O flux rates are associated with much higher uncertainty than CO2 exchange in terrestrial ecosystems.

(Table 1)

## 15.3 Regulation of N2O emissions in the terrestrial biosphere

### N2O formation in soils

N2O is a by-product of nitrification, an aerobic microbial process converting ammonium to nitrate, and of denitrification, an anaerobic microbial process converting nitrate in two steps to N2O, and then N2O to N2. At the micro-scale, process level control of N2O production by nitrification and denitrification is well known (Mosier et al. 1996; Skiba et al. 1998; Smith et al. 1998). Both processes are highly sensitive to temperature, substrate availability, pH and oxygen, the latter being controlled by soil moisture status and microbial activity. In agricultural soils, N2O emissions are additionally controlled by management factors, among which crop type, fertilizer type and N application rates seem to be most relevant (Stehfest and Bouwman 2006). Figure 1 shows the dependence of N2O production on soil temperature and moisture in a schematic way. It is obvious that the relationships are highly non-linear and complex when considering that temperature and soil moisture interact with each other and that nitrification and denitrification can occur simultaneously. Environmental factors are in most cases unknown at the micro-scale, but only measured at the site scale. It is also the ecosystem scale at which model input parameters are determined (Mosier et al. 1996). Temperature, precipitation/soil moisture and management activities regulate N2O emissions in a complex non-linear way at the daily time scale (Freibauer and Kaltschmitt 2003; Smith et al. 1998). However, at annual time scales, the explanatory power of each of the environmental and management factors at the ecosystem scale is usually low (Freibauer and Kaltschmitt 2003).

(Figure 1)

Natural abundance of 15N-N2O and 18O-N2O may be a suitable proxy for attribution of tropospheric N2O to agricultural soils (Kaiser et al. 2004; Kim and Craig 1993; Perez et al. 2001; Rockmann et al. 2003) although the signal is partly disturbed by ocean and in-situ atmospheric processes, and the level of depletion of N2O from agricultural soils in 15N and 18O is not constant (Tilsner et al. 2003; Yamulki et al. 2001). At ecosystem level, N2O forming processes of nitrification and denitrification can be distinguished by isotopic studies of natural abundance 15N-N2O and 18O-N2O or labelling experiments in conjunction with laboratory incubation studies (Tilsner et al. 2003; Yamulki et al. 2001). This distinction between processes provides valuable insights for validation of process-based models.

### Spatio-temporal emission patterns of soilborne N2O emissions

Brumme et al. (1999) introduce a valuable concept of controls on seasonal and annual N2O fluxes for forest ecosystems, which can be expanded to other land use types (Figure 2).

(Figure 2)

Long-term controls of N2O fluxes (ecosystem and climatic properties): Background emission patterns show no correlation with long-term controls of N2O fluxes and are characterized by low fluxes throughout the year. Seasonal emission patterns are explained by the combined effect of high annual precipitation, broadleaved trees, amount and structure of soil litter layer, high mineral bulk densities, and plant community (Brumme et al. 1999). They roughly follow the seasonal cycle of temperature.

Short-term controls of N2O fluxes (weather, management): Short-term bursts of N2O occur on top of these seasonal patterns. Unlike CO2, N2O emissions from soils can have a strong episodic nature. Extreme events lasting a few hours to days can contribute up to 80% of the annual N2O emissions.

N2O emissions typically exhibit a high degree of spatial variability (75-100%; reviewed by Freibauer and Kaltschmitt, 2003). Managed, in particular fertilized, soils tend to show a wider site-to-site variation in the magnitude of annual N2O emissions than unmanaged ones. In contrast, "indirect" N2O emissions from diffuse pollution of aquifers is probably smaller than originally expected (Nevison 2000).

Effects of fertilizer type, timing, and quantity, and of crop species and crop rotation have been commonly studied by factorial designs. They have provided an extensive dataset for deriving emission factors, parametrization and validation of process-based models and the attribution of emissions to recent human interference versus natural/land use history-based emissions (Bouwman et al. 2002a; Stehfest and Bouwman 2006). Long-term studies with factorial design are the key source of information for N2O monitoring.

Adequate information of key drivers of N2O emissions at ecosystem scale is essential for bottom-up flux estimates. Whilst general information about vegetation type, tree species and agricultural crops is available in reasonable quality, major gaps in knowledge still exist with regard to spatially explicit soil characteristics (organic C content, moisture) and agricultural management. The latter can be overcome by coupling farm-economic with ecological models (Neufeldt et al. 2004) and better exploitation of agricultural census data.

## 15.4 Current monitoring capabilities

N2O poses much greater challenges than CO2 to an adequate observational design and protocol at ecosystem level, and to quantitative modelling and upscaling. However, considerable progress has been made in the bottom-up quantification of N2O emissions over the last decade.

### Flux measurements at ecosystem level

Annual or longer N2O flux measurements have been performed at more than 100 sites in Europe covering a wide range of ecosystems. On agricultural sites, measurements often encompass various fertilizer treatments or crop types. Most researchers have applied manual static chambers combined with gas chromatography and electron capture detector (GC-ECD) or infrared gas analysis (IRGA) and measure in time steps of days to weeks, so the necessary temporal interpolation between measurements introduces an additional uncertainty to annual N2O flux estimates. A comparison with continuous measurements demonstrated that a combination of weekly with event-driven sampling can produce robust annual N2O flux estimates with an accuracy of better than 25% ((Ruser 1999), Butterbach-Bahl, pers. comm.). Chamber techniques are well established, cheap and easily applicable but labour-intensive unless sampling is automated. Automated chambers are increasingly used. Automated systems often turned out too expensive to allow a sufficient number of replicates so that they are used in combination with manual chambers to address spatial variability. Chambers are useful in experiments with different treatments (fertilizer, crop type), as in agricultural systems. For monitoring, an adequate number of replicates and a minimum surface area of 1 m² covered by the chambers, depending on the spatial variability of a site, is recommended (Heinemeyer et al. 1995). Tunable diode lasers (TDL) and most recently quantum cascade lasers (QCL) offer a new methodology for N2O flux measurements by eddy covariance (Kroon et al. 2007). Chamber and micrometeorological techniques were shown to produce comparable results (Christensen et al. 1996; Smith et al. 1994). Well-aerated soils have been widely covered in experiments, but there are clear gaps in knowledge about soils with high organic contents, gleyic and stagnic properties and under alternating-wet conditions, as well as in Mediterranean and boreal climate. The European research projects NOFRETETE and NitroEurope are providing the first multi-year, quasi-continuous N2O flux measurements in ecosystems across Europe with automated static chambers.

### Observations at regional level

Micrometeorological measurements have successfully been applied in campaign mode using flux gradient methods, eddy covariance, mass balance (outgoing versus incoming winds), and nocturnal boundary layer approaches (Denmead et al. 2000; Griffith et al. 2002; Hensen et al. 1999). While results generally confirm the plot scale measurements, operability of FTIR and TDL sensors and sometimes inadequate sensitivity of the approaches to detect small fluxes still limit their use for monitoring purposes.

### Atmospheric observations at continental to global level

The first global atmospheric N2O monitoring networks started around 1980 (ALE/GAGE/AGAGE, NOAH-CMDL). In Europe, further national stations came into operation in the mid 1990s. In the frame of CarboEurope, N2O was included in the emerging multi-species observation network in Europe. N2O concentration measurements are, however, less standardized than those of CO2. The CarboEurope network comprises 9 continuous stations (2 ground monitoring stations, 7 tall towers), 21 flask sampling sites, and 5 (+3 in Russia) vertical aircraft profile sites at which N2O concentrations have been or are measured on a voluntary basis. Due to the small concentration gradients of N2O, detecting an atmospheric signal remains difficult. On the other hand, the information contained in existing N2O concentration measurements has not been systematically exploited. A major limitation in these data is the lack of an international primary scale standard for N2O that would allow to intercalibrate the various existing N2O observational sites and networks. The small concentration gradients are therefore further perturbed by likely calibration offsets. The isotopic information in atmospheric N2O has rarely been measured and has not been used until now to attribute regional N2O sources. (Chedin et al. 2002) propose retrieving nighttime atmospheric N2O concentrations from existing satellite observations in a latitudinal band relevant for Europe. It is unclear whether this satellite product will become operational and useful for top-down modelling.

### Bottom-up modelling

Literature provides numerous plot-scale studies, from which empirical emission factors or models for inventory estimates were derived. Examples for global sectoral inventories are (Bouwman et al. 2002b), and those summarized in Table 4.4 of IPCC (2001). Inventories apparently give a surprisingly robust estimate of fluxes when aggregated over larger areas (national to continental) within large uncertainties, but fail to depict the variation of N2O emissions with interannual variability of climate and crop rotations, or regional patterns. Empirical models depict some of the regional patterns of soil and climate with large uncertainties, but still have a limited capacity to explain the variation in local N2O fluxes. In Europe and EU Member States, inventories of N2O agricultural emissions were calculated by regional emission factors based on IPCC (national communications, (Boeckx and Van Cleemput 2001), (Brown et al. 2001)), based on measurements (Lilly et al. 2003), by empirical regressions (Bouwman et al. 2002b; Conen et al. 2000; Freibauer and Kaltschmitt 2003; Sozanska et al. 2002; Stehfest and Bouwman 2006), regionalization of farm types and agro-pedological regions (Boeckx et al. 2001).

Mechanistic process models have so far been applied at local, national and regional levels only. They provide good diagnostics of temporal patterns of N2O fluxes. The models are sensitive to soil temperature, moisture, texture, and organic carbon content. N2O flux maps have been presented for agriculture and/or forest in selected regions of Europe based on DNDC (agriculture, (Brown et al. 2002) (Butterbach-Bahl et al. 2001)), pNET-DNDC and regional versions of Expert-N (forest, (Butterbach-Bahl et al. 2001) (Butterbach-Bahl et al. 2004; Kesik et al. 2005; Schulte-Bisping et al. 2003). The modelled spatial patterns of N2O fluxes seem to adequately represent the expected magnitude of N2O emissions in standard situations of well aerated soils with low to medium organic carbon content, but uncertain in undersampled areas with likely high emissions (gleyic, stagnic soil properties and high soil organic carbon). Nevertheless, the explanatory power of the models for site-level N2O fluxes is often low. Obviously, not only the magnitude of drivers, e.g., fertilizer application rates, but also the intensity of gradients at small spatial scales matter. This goes beyond the feasible level of detail in existing models.

The comparison between approaches cited above reveals differences in national N2O budgets of up to a factor of 2, which can be partly explained by different scope or omissions of sources in older inventories. At sub-continental aggregation, a better agreement is achieved (e.g. (Freibauer 2003; Li et al. 2001).

### Top-down modelling

N2O fluxes have some principal differences to CO2 that complicate observations and spatial scaling of fluxes. The earth surface almost entirely acts as a source of N2O, small sinks were only found in some wet situations for a few days. N2O sources are very homogeneously spread in space, but extreme N2O emission events, e.g. during freeze-thaw periods, after fertilization or when rewetting dry soils, make them very variable at synoptical time scales in a presently unpredictable manner. This feature, together with the long atmospheric residence time of N2O, produces complex atmospheric N2O concentration patterns and very small atmospheric N2O concentration gradient signals. There are also N2O sources in the atmosphere, which complicate the top-down verification, e.g. review in (Bouwman et al. 2000).

Nevertheless, the global N2O emissions have been estimated by forward atmospheric transport modelling with very coarse resolution, but uncertainties are large (Bouwman et al. 2000) as a consequence of limited concentration data and small gradients. Using a 222Rn tracer approach, (Biraud et al. 2002; Schmidt et al. 2001) demonstrated that high-precision continuous multi-species monitoring can derive top-down estimates of N2O fluxes at landscape to continental scale. Top-down estimates are particularly useful to constrain regional budgets of the large-area, episodic N2O sources from soils.

We are not aware of any comprehensive co-ordinated effort to observe natural and anthropogenic sources (and small sinks) of N2O at global scale. Hence, the experimental community and data-model integration of N2O fluxes are less organized than for CO2. There is less, and less accessible data, and if so, measurements have often been performed discontinuously at plot scale. There are few, mainly discontinuous, atmosphere data with under-representation of continental stations. Atmospheric data have been little exploited.

## 15.5 Strategy towards a pan-European GHG (continental scale) monitoring network

This section sets forth a framework for integrated European N2O monitoring based on observation-driven modelling. This coordinated system has three main objectives:

- To provide the long-term observations and census information required to improve understanding of the present state and future behaviour of European N2O emissions, particularly the interaction between human and natural factors responsible for major sources.

- To integrate observations and models of the terrestrial C-, N, and water cycle.

- To monitor and assess the effectiveness of N2O emission reduction activities in Europe, including attribution of sources by region and sector.

The system will meet those objectives by routinely quantifying the regional distribution of N2O fluxes exchanged between the European land surface and the atmosphere via measurements and models, along with observations and census information of drivers of nitrification and denitrification. Table 1 identifies soils as the dominant source of N2O, and of uncertainty. Therefore, we will focus the strategy on the N2O exchange between soils and the atmosphere.

N2O can in principle be addressed by a monitoring system which has many analogies with CO2 monitoring, i.e. a multiple-constraint framework in which measurements at local scale feed into mechanistic process-oriented soil models with high time-space resolution. The results will be tested against atmospheric estimates based on atmospheric N2O concentration measurements and, if feasible, remotely sensed N2O columns. As the correlation between environmental and management factors with observed N2O fluxes is often weak mechanistic models may need some more development than for CO2. Even more than in the case of CO2, observation-driven, empirical models can provide equally accurate means of calculating contemporate N2O fluxes as biogeochemical models. It is expected that it will take longer to develop fully operational observational and data assimilation schemes for N2O than for CO2. Methodological synergies with CO2 and CH4 monitoring will facilitate progress.

The long-term vision is to fully couple the carbon-, nitrogen- and water-cycle in ecosystem models. This will allow to predict the response of the full GHG balance of ecosystems to climate variability and changing human management, and to simultaneously trace the fate of carbon and nitrogen in ecosystems. N2O is a small constituent (typically ~1 %) of the terrestrial N cycle. Consequently, even large errors and uncertainties in modelled N2O only have a small effect on the overall predictive capacity of the models for other elements of the N cycle, but significantly affect the capacity of models for quantifying the feedback between climate change, human activities and ecosystem GHG fluxes.

## 15.6 Future directions for enhanced observational strategies

To maximize synergy with existing observations of ecosystem GHG fluxes, systematic long-term in situ N2O observations at ecosystem level should best be linked with the existing CO2 observation network. The N2O observation network should be balanced to cover all ecosystem types according to the emission patterns of Figure 1, with a higher density in ecosystems that contribute most to European emissions, i.e. croplands, grasslands and nitrogen rich wetlands. A nested design in representative test farms and catchments appears most efficient for deriving adequate process information and to discriminate between anthropogenic and natural sources of N2O emissions. To some extent, the European research project NitroEurope (http://www.nitroeurope.eu/) has already adopted this strategy.

### Directions for enhanced measurements

Up to date, most N2O measurements have focused on agricultural upland ecosystems. Measurements should aim at filling observational gaps in (partially) moist ecosystems and in compacted, nitrogen-rich farm areas that are potentially high N2O emitters.

Unlike in the case of ecosystem carbon fluxes, N2O measurements have not yet been systematically collected in a central manner. An international initiative to enhance data harmonization and intercomparability, archiving, and distribution of compiled observational data to the modelling community is needed. Existing international programmes in the field of reactive nitrogen could integrate N2O into their activities, such as the International Nitrogen Initiative (INI), the COST Action 729 and the ESF programme NiNE.

Chamber measurements are best performed automated, quasi-continuous, or at least weekly plus at least daily in periods of likely high emission events, covering adequate surface area (> 1 m²) and adequate number of replicates (~8). Collocated data on meteorology, soil moisture, soil C and N, detailed management activities and, if possible, mineral N in soil, are required.

N2O chamber measurements should be performed in a factorial design to cover the full range of possible emission events, local hotspots and different intensity levels of test farms and landscapes (e.g. variation in crop type, fertilizer type, tree type (coniferous/broadleaved), management, soil type, soil water status, compacted areas, areas where livestock preferably rests). Additional low-intensity N2O monitoring sites may suit as control sites for spatial upscaling of annual ecosystem-level N2O budgets. For this purpose, cost-effective, low maintenance, in situ sensors for terrestrial ecosystem N2O fluxes need to be developed. Such sensors could be constructed based on the knowledge for N2O monitoring systems in hospitals but require a higher precision and sensitivity for N2O at ambient concentration. NitroEurope explores cumulative approaches based on the well-established chamber technique.

New observational approaches should be tested to integrate N2O emissions at farm to regional scale and to verify the highly uncertain upscaled local fluxes. Combinations of chamber measurements with mobile plume measurements appear promising (Hensen et al. 2006) as well as nighttime accumulation of N2O in the tropospheric inversion layer.

Isotope tracer methods should be further investigated to attribute atmospheric N2O to different sources. In a similar way, the existing long-term continuous atmospheric monitoring stations should be extended by high-precision, intercalibrated N2O and 222Rn to multi-species sampling where missing. Further inner-continental stations will reduce uncertainties over Eastern Europe where in-situ measurements and information about drivers are particularly scarce. The intercalibration of N2O concentration measurements and exhaustive use of existing and upcoming N2O concentration data from multi-species monitoring stations and flasks is essential to formulate the exact requirements for atmospheric N2O observations. The use of remotely sensed N2O column data should be further investigated. The production of an international primary standard scale for atmospheric N2O concentrations, and best also for 15N and 18O in N2O, is the precondition for systematic atmospheric observations. There may be an important prospect for combining the roles of N2O as a GHG gas and as a tracer gas in inversion studies. Developing this aspect of is likely to improve both N2O emission estimates and specification of CO2 budget components but the small atmospheric N2O gradients remain a challenge.

### Requirements for ancillary information for scaling

The major drivers for N2O emissions have been identified and need to be monitored together with N2O fluxes: Soil temperature and soil moisture status and their variability, as well as land use and management, in particular N-fertilization.

Data sources for meteorological information are operational. Soil moisture can be calculated by the models. However, it would be worth testing to what extent remote sensing products for topsoil moisture agree with these calculations and whether they provide additional constraints. Remote sensing products for soil moisture only exist in research mode.

Valuable census data at farm level are presently collected as a prerequisite for obtaining subsidies under the European Common Agricultural Policy, but these data are not public. At least in the context of the European network of test farms, plot-level and farm-gate C- and N-balances should be made available to the research and monitoring community. Regional C- and N-models require georeferenced information about the distribution of agricultural crop types and management dates. In this field, remote sensing methods may provide useful information. Crop types and field sizes are already being monitored from space in a statistical sampling mode in order to verify the farmers´ submissions for subsidies under the Common Agricultural Policy. Developing operational remote sensing products for agricultural crops and land management dates (harvest, ploughing) would significantly reduce the uncertainty in bottom-up modelling of N2O emissions.

For top-down estimates, it remains to be tested whether atmospheric N2O concentration columns can be measured in adequate resolution to constrain the terrestrial N2O budget at large scale.

### Stepwise implementation of the modelling strategy

At present, bottom-up modelling of N2O fluxes is immature as compared to CO2. Due to the difficulty in modelling the temporal N2O emission patterns and the large uncertainty in annual N2O fluxes a robust N2O modelling strategy must rely on a suite of different models. Before N2O modelling can become operational several steps need to be taken.

An obvious first step is the choice of promising models for continental scale N2O modelling. Existing process-oriented and empirical models need to be validated against independent observations from a wide range of ecosystems and further improved. Modules for important N2O sources other than soil need to be added. Sensitivity tests shall identify the most important input parameters that need to be constrained by observations.

One major source of uncertainty is the inadequate spatio-temporal representation of soil moisture and of land management. It must be a priority to produce maps of farming patterns, fertilization rates and crop types. It has been proven very useful to couple N2O models to farm-economic models, which allow spatializing the management patterns and farming intensity (Neufeldt et al. 2006). This approach needs to be further developed and expanded to European and global level.

Ultimately, as for CO2, a suite of different biogeochemical models with fully coupled C-N-water cycles should be developed that can deliver routine diagnostics of the European N2O emissions from soils. This effort should include bottom-up data assimilation schemes for nitrogen species.

For the top-down estimates, the suitability of various atmospheric models and approaches should be tested to constrain regional and continental N2O budgets.

## 15.7 Conclusion

The key to improving N2O emission estimates lies in the soil-borne fluxes. This requires the scientific development of cheap and operational flux measurement techniques to densify obervations in critical ecosystems and seasons prone to high N2O emissions, as well as improved process models. The ecosystem level N2O observation network in Europe has emerged in an opportunistic way around interested research groups. The NitroEurope project is starting to co-ordinate and harmonise the ecosystem level observations. However, important potential emission hotspots are likely to be yet uncovered by observations. Atmospheric N2O concentration measurements are not yet co-ordinated in Europe. Observations of N2O have proved more demanding than those of CO2. The continental co-ordination of observations and a systematic strategy for addressing the challenges of temporal and spatial scale transitions are still in their infancy.

## References

Biraud, S. et al. 2002. Quantification of carbon dioxide, methane, nitrous oxide and chloroform emissions over Ireland from atmospheric observations at Mace Head. Tellus Series B-Chemical & Physical Meteorology, 54(1): 41-60.

Boeckx, P. and Van Cleemput, O. 2001. Estimates of N2O and CH4 fluxes from agricultural lands in various regions in Europe. Nutrient Cycling in Agroecosystems, 60(1-3): 35-47.

Boeckx, P., Van Moortel, E. and Van Cleemput, O. 2001. Spatial and sectorial disaggregation of N2O emissions from agriculture in Belgium. Nutrient Cycling in Agroecosystems, 60(1-3): 197-208.

Bouwman, A.F., Boumans, L.J.M. and Batjes, N.H. 2002a. Emissions of N2O and NO from fertilized fields: Summary of available measurement data. Global Biogeochemical Cycles, 16(18 October 2002).

Bouwman, A.F., Boumans, L.J.M. and Batjes, N.H. 2002b. Modeling global annual N2O and NO emissions from fertilized fields. Global Biogeochemical Cycles, 16(4): 1080, doi:10.1029/2001BG001812.

Bouwman, A.F., Taylor, J.A. and Kroeze, C. 2000. Testing hypotheses on global emissions of nitrous oxide using atmospheric models. Chemosphere - Global Change Science, 2(3-4): 475-492.

Brown, L. et al. 2001. An inventory of nitrous oxide emissions from agriculture in the UK using the IPCC methodology: emission estimate, uncertainty and sensitivity analysis. Atmospheric Environment, 35(8): 1439-1449.

Brown, L. et al. 2002. Development and application of a mechanistic model to estimate emission of nitrous oxide from UK agriculture. Atmospheric Environment, 36(6): 917-928.

Brumme, R., Borken, W. and Finke, S. 1999. Hierarchical control on nitrous oxide emission in forest ecosystems. Global Biogeochemical Cycles, 13(4): 1137-1148.

Butterbach-Bahl, K., Kesik, M., Miehle, P., Papen, H. and Li, C. 2004. Quantifying the regional source strength of N-trace gases across agricultural and forest ecosystems with process based models. Plant & Soil, 260(1-2): 311-329.

Butterbach-Bahl, K., Stange, F., Papen, H. and Li, C.S. 2001. Regional inventory of nitric oxide and nitrous oxide emissions for forest soils of southeast Germany using the biogeochemical model PnET-N-DNDC. Journal of Geophysical Research-Atmospheres, 106(D24): 34155-34166, doi 2000JD000173.

Chedin, A. et al. 2002. Annual and seasonal variations of atmospheric CO2, N2O and CO concentrations retrieved from NOAA/TOVS satellite observations. Geophysical Research Letters, 29(8): doi 10.1029/2001BL014082.

Christensen, S. et al. 1996. Nitrous oxide emission from an agricultural field - comparison between measurements by flux chamber and micrometerological techniques. Atmospheric Environment, 30(24): 4183-4190.

Conen, F., Dobbie, K.E. and Smith, K.A. 2000. Predicting N2O emissions from agricultural land through related soil parameters. Global Change Biology, 6(4): 417-426.

Denmead, O.T., Leuning, R., Jamie, I. and Griffith, D.W.T. 2000. Nitrous oxide emissions from grazed pastures: measurements at different scales. Chemosphere - Global Change Science, 2(3-4): 301-312, doi:10.1016/S1465-9972(00)00035-0.

ETC. 2007. National greenhouse gases inventories (IPCC Common Reporting Format sector classification). The European Topic Centre on Air and Climate Change (Database version 1.1, 13 Feb 2007), national submissions to UNFCCC or to EU Monitoring Mechanism of CO2 and other greenhouse emissions. http://dataservice.eea.europa.eu/dataservice/metadetails.asp?id=971, downloaded on 10 April 2007.

Firestone, M.K. and Davidson, E.A. 1989. Microbiological basis of NO and N2O production and consumption in soil. In: M.O. Andreae and D.S. Schimel (Editors), Exchange of Trace Gases between Terrestrial Ecosystems and the Atmosphere. Dahlem Konferenzen. John Wiley & Sons, Chichester, pp. 7-21.

Freibauer, A. 2003. Regionalised inventory of biogenic greenhouse gas emissions from European agriculture. European Journal of Agronomy, 19(2): 135-160.

Freibauer, A. and Kaltschmitt, M. 2003. Controls and models for estimating direct nitrous oxide emissions from temperate and boreal agricultural mineral soils in Europe. Biogeochemistry, 63(1): 93-115.

Griffith, D.W.T., Leuning, R., Denmead, O.T. and Jamie, I.M. 2002. Air-land exchanges of CO2, CH4 and N2O measured by FTIR spectrometry and micrometeorological techniques. Atmospheric Environment, 36(11): 1833-1842.

Heinemeyer, O., Munch, J.C. and Kaiser, E.-A. 1995. Variabilität von N2O-Emissionen - Bedeutung der Gasauffangsysteme. Mitt Dtsch Bodenkundl Ges, 76(543-546).

Hensen, A., Dieguez Villar, A. and Vermeulen, A.T. 1999. Emission estimates based on ambient N2O concentrations measured at a 200 m tower in the Netherlands 1995-1997. In: A.a.K. Freibauer, M. (Editor), Approaches to Greenhouse Gas Inventories of Biogenic Sources in Agriculture, Stuttgart, pp. 215-228.

Hensen, A. et al. 2006. Dairy farm CH4 and N2O emissions, from one square metre to the full farm scale. Agriculture Ecosystems & Environment, 112(2-3 Special Issue SI): 146-152.

IPCC. 1996. Global Change 1995 - The Science of Climate Change. Contribution of Working Group I to the Second Assessment Report of the Intergovernmental Panel on Cliamte Change, Intergovernmental Panel on Climate Change.

IPCC. 2000. Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. IPCC National Greenhouse Gas Inventories Programme. Published for the IPCC by the Institute for Global Environmental Strategies, Japan.

IPCC. 2001. Climate Change 2001: The Scientific Basis. IPCC Third Assessment Report. http://www.ipcc.ch/.

Kaiser, J., Rockmann, T. and Brenninkmeijer, C.A.M. 2004. Contribution of mass-dependent fractionation to the oxygen isotope anomaly of atmospheric nitrous oxide -. Journal of Geophysical Research-Atmospheres, 109(D3): art. no. D03305.

Kesik, M. et al. 2005. Inventories of N2O and NO emissions from European forest soils. Biogeosciences, 2(4): 353-375.

Kim, K.R. and Craig, H. 1993. Nitrogen-15 and oxygen-18 characteristics of nitrous oxide - a global perspective. Science, 262(5141): 1855-1857.

Kroon, P.S. Hensen, A., Jonker, H.J.J., Zahniser, M.S., van’t Veen, W.H. and Vermeulen, A.T. 2007. Suitability of quantum cascade laser spectrometry for CH4 and N2O eddy covariance measurements. Biogeosciences Discuss., 4, 1137–1165.

Li, C.S. et al. 2001. Comparing a process-based agro-ecosystem model to the IPCC methodology for developing a national inventory of N2O emissions from arable lands in China. Nutrient Cycling in Agroecosystems, 60(1-3): 159-175.

Lilly, A., Ball, B.C., McTaggart, I.P. and Horne, P.L. 2003. Spatial and temporal scaling of nitrous oxide emissions from the field to the regional scale in Scotland. Nutrient Cycling in Agroecosystems, 66(3): 241-257.

Mosier, A.R., Duxbury, J.M., Freney, J.R., Heinemeyer, O. and Minami, K. 1996. Nitrous oxide emissions from agricultural fields - assessment, measurement and mitigation. Plant & Soil, 181(1): 95-108.

Neufeldt, H., Schäfer, M. and Angenendt, E. 2004. Modelling regional GHG emissions from farm production and agricultural soils with EFEM-DNDC. In: L. Institute for Energy and Environment (Editor), International Conference Greenhouse Gas Emissions from Agriculture - Mitigation Options and Strategies. Institute for Energy and Environment, Leipzig, Leipzig, Germany, pp. 165-171.

Neufeldt, H. et al. 2006. Disaggregated greenhouse gas emission inventories from agriculture via a coupled economic-ecosystem model. Agriculture Ecosystems & Environment, 112(2-3 Special Issue SI): 233-240.

Nevison, C. 2000. Review of the IPCC methodology for estimating nitrous oxide emissions associated with agricultural leaching and runoff. Chemosphere - Global Change Science, 2(3-4): 493-500.

Nevison, C. and Holland, E. 1997. A reexamination of the impact of anthropogenically fixed nitrogen on atmospheric N2O and the stratospheric ozone layer. J. Geophys. Res., 102: 25519-25536.

Olivier, J.G.J., Bouwman, A.F., Van der Hoek, K.W. and Berdowski, J.J.M. 1998. Global air emission inventories for anthropogenic sources of NOx, NH3 and N2O in 1990. Environmental Pollution, 102(Suppl 1): 135-148.

Perez, T. et al. 2001. Identifying the agricultural imprint on the global N2O budget using stable isotopes. Journal of Geophysical Research-Atmospheres, 106(D9): 9869-9878, doi 2000JD900809.

Rockmann, T., Kaiser, J. and Brenninkmeijer, C.A.M. 2003. The isotopic fingerprint of the pre-industrial and the anthropogenic N2O source. Atmospheric Chemistry and Physics, 3: 315-323.

Ruser, R. 1999. Freisetzung und Verbrauch der klimarelevanten Spurengase N2O und CH4 eines landwirtschaftlich genutzten Bodens in Abhängigkeit von Kultur und N-Duengung, unter besonderer Berücksichtigung des Kartoffelbaus. FAM-Bericht 36, Hieronymus-Verlag, München.

Schmidt, M., Glatzel-Mattheier, H., Sartorius, H., Worthy, D. and Levin, I. 2001. Western European N2O emissions: A top-down approach based on atmospheric

observations. Journal of Geophysical Research-Atmospheres, 106(D6): 5507-5516.

Schulte-Bisping, H., Brumme, R. and Priesack, E. 2003. Nitrous oxide emission inventory of German forest soils. Journal of Geophysical Research-Atmospheres, 108(D4): doi 10.1029/2002JD002292.

Skiba, U.M., Sheppard, L.J., Macdonald, J. and Fowler, D. 1998. Some key environmental variables controlling nitrous oxide emissions from agricultural and semi-natural soils in Scotland. Atmospheric Environment, 32(19): 3311-3320.

Smith, K.A. et al. 1994. Micrometeorological and chamber methods for measurement of nitrous oxide fluxes between soils and the atmosphere: Overview and conclusions. Journal of Geophysical Research, 99(D8): 16541-16548.

Smith, K.A., Thomson, P.E., Clayton, H., Mctaggart, I.P. and Conen, F. 1998. Effects of temperature, water content and nitrogen fertilisation on emissions of nitrous oxide by soils. Atmospheric Environment, 32(19): 3301-3309.

Sozanska, M., Skiba, U. and Metcalfe, S. 2002. Developing an inventory of N2O emissions from British soils. Atmospheric Environment, 36(6): 987-998.

Stehfest, E. and Bouwman, L. 2006. N2O and NO emission from agricultural fields and soils under natural vegetation: summarizing available measurement data and modeling of global annual emissions. Nutrient Cycling in Agroecosystems, 74(3): 207-228.

Tilsner, J., Wrage, N., Lauf, J. and Gebauer, G. 2003. Emission of gaseous nitrogen oxides from an extensively managed grassland in NE Bavaria, Germany. Biogeochemistry, 63(3): 249-267.

Yamulki, S. et al. 2001. Diurnal fluxes and the isotopomer ratios of N2O in a temperate grassland following urine amendment. Rapid Communications in Mass Spectrometry, 15(15): 1263-1269.

Figure captions

Figure 1:  Schematic dependence of N2O production by nitrification and denitrification on soil temperature and moisture. Graphs for soil moisture are redrawn from (Firestone and Davidson 1989).

Figure 2: Hierarchical control of annual N2O fluxes and resulting emission patterns in ecosystems, generalised and extended from (Brumme et al. 1999).

## Table

Table 1: Estimated budget of anthropogenic N2O for the European Community (ETC 2007). Emissions are derived from the national communications (http://ghg.unfccc.int/index.html) and uncertainties are based on (IPCC 2000; Olivier et al. 1998).

| EU-25 in 2004 |  | N2O emissions | Uncertainty |  |
| --- | --- | --- | --- | --- |
|  |  | Gg N yr-1 | Gg N yr-1 | % |
| Anthropogenic N2O emissions from national communications |  |  |  |  |
| Agricultural soils in EU-25 in EU-15 |  | 485 412 | 388 | 80 % |
| Manure management |  | 64 | 48 | 75 % |
| Land-Use‚ Land-Use Change and Forestry |  | 7 | 6 | 80 % |
| Industrial Processes |  | 118 | 12 | 10 % |
| Transport |  | 50 | 25 | 50 % |
| Other energy |  | 74 | 22 | 30 % |
| Waste |  | 24 | 24 | 100 % |
| Other sources |  | 8 | 8 | 100 % |
| Anthropogenic N2O [Gg N yr-1] in EU-25 in EU-15 |  | 830 699 | 532 448 | 64 % 64 % |
| Anthropogenic N2O in EU-25 [Tg Cequ yr-1] |  | 110 | 71 |  |
| Soilborne N2O emissions in EU-15 |  |  |  |  |
| Agricultural soils in EU-15 | 442 a |  | 116 a | 26 % |
| Forest soils in EU-15 | 78-87 b |  |  |  |
| Soil N2O emissions in EU-15 [Gg N yr-1] | 525 |  |  |  |

a  (Freibauer 2003), b (Kesik et al. 2005)

Figure 1

Figure 2

ALE, Atmospheric Lifetime Experiment; GAGE, Global Atmospheric Gases Experiment; AGAGE, Advanced Global Atmospheric Gases Experiment.

N2O and Halocarbons Group, Climate Monitoring and Diagnostics Laboratory, NOAA.