Chapter 3

Quantifying fossil fuel CO2 over Europe

Ingeborg Levin 1 and Ute Karstens 2

1 Institut für Umweltphysik, University of Heidelberg, Heidelberg, Germany

2 Max-Planck-Institut für Biogeochemie, Jena, Germany

3.1.  Introduction

Europe is responsible for more than 25% of global fossil fuel CO2 emissions (Marland, Boden and Andres 2006), and these emissions account for about 30-50% of the observed CO2 variability in this region (see Section 2.1). To balance greenhouse gases over Europe, therefore, also requires quantification of CO2 emissions from fossil fuel (i.e. coal, oil and natural gas) burning. Reliable continuous observations of the fossil fuel CO2 component are needed in order to validate emission-based model simulations and finally allow for robust estimates of the biogenic part in the observed atmospheric CO2 variations. Fossil fuel emissions in Europe are very heterogeneously distributed in space with hot spots in highly industrialised and populated regions. The temporal variability comprises seasonal as well as diurnal cycles with a strong coupling to ambient temperature variations (i.e. domestic heating), the current economic situation (i.e. industry) and other factors such as the general meteorological conditions or holiday periods (i.e. traffic and industry). All these parameters need to be accurately modelled if fossil fuel emissions shall be estimated in a realistic and quantitative way from bottom-up information on the respective sources.

“Monitoring” fossil fuel CO2 in the atmosphere is in principle possible via Radiocarbon (14C) observations in atmospheric CO2. 14C is the radioactive carbon isotope which is naturally produced in the atmosphere via cosmic ray induced reactions of neutrons with atmospheric nitrogen. The radioactive half life of 14C is 5730 years. The natural equilibrium level of atmospheric 14CO2 is established by production in the lower and middle stratosphere and subsequent decay in all carbon exchanging reservoirs, i.e. the atmosphere, the biosphere and the oceans. This natural level has been disturbed in the last century by man’s activities, via the ongoing input of fossil fuel CO2 into the atmosphere known as Suess effect  (Suess 1955), but also by artificially produced 14C during nuclear detonations in the atmosphere, mainly in the 1950s and early 1960s (14C bomb effect). CO2 from burning of fossil fuels, due to its age of several hundred million years, is free of 14C; adding fossil fuel CO2 to the atmosphere, therefore, not only leads to an increase of the CO2 mixing ratio but also to a decrease of the 14C/12C ratio in atmospheric CO2. From a 14CO2 measurement at a polluted sampling site, e.g. in the boundary layer on the European continent, we can therefore directly calculate the regional fossil fuel CO2 surplus if the undisturbed background 14CO2 level is known.

Particularly in Europe long-term quasi-continuous 14CO2 measurements exist at a number of marine, mountain as well as urban and rural sites (Levin, Münnich and Weiss 1980; Levin et al. 1985; 1989; 2003; Kuc 1986; Meijer, van der Plicht and Gislefoss 1995). These measurements allow determining the regional fossil fuel CO2 offset at the stations when using the observations at e.g. Mace Head (Western Ireland, clean marine air) or Jungfraujoch (Swiss high altitude “free troposphere” site) as background reference level. However, the 14CO2 measurements need to be made quasi-continuously at very high precision to yield accurate estimates of fossil fuel CO2. This is a very expensive undertaking and, thus, not yet realised at many sites.

Due to the lack of such precise and frequent 14CO2 observations other tracers that could possibly serve as surrogates for fossil fuel CO2 have been discussed in the literature. Among these are carbon monoxide (CO) and acetylene (C2H2), common by-products of (fossil fuel) combustion processes, sulphur hexafluoride (SF6), a purely artificial and very inert gas, which has a similar spatial source distribution as fossil fuel CO2 (associated with energy distribution), and others. In particular the use of CO as a quantitative tracer of fossil fuel CO2 has been investigated in recent years in a number of experimental and modelling studies (Bakwin et al. 1997; Potosnak et al. 1999; Gamnitzer et al. 2006; Rivier et al. 2006; Turnbull et al. 2006).

The aim of this chapter is to review the state of the art of quantifying fossil fuel CO2 over Europe. We will first provide an estimate of the relative signal of fossil fuel emissions over Europe using emissions inventories and atmospheric transport modelling and compare them to the respective signals from biogenic (and oceanic) sources and sinks. Then we will give an overview on the existing 14CO2 measurements and respective 14C-based fossil fuel CO2 estimates including their seasonal variability and temporal trends. We will further discuss the quality of the surrogate tracer CO, and finally present a sensitivity test of a proposed method, which allows to estimate the uncertainty of using continuous CO measurements together with 14C-calibrated CO/CO2(foss) ratios to estimate fossil fuel CO2 at high temporal resolution (Levin and Karstens 2007). Finally, we will make a recommendation for a fossil fuel CO2 monitoring network for the European continent.

3.2. State of the Art of quantifying fossil fuel CO2 over Europe

3.2.1 The atmospheric CO2 signals as derived from bottom-up emissions inventories and an atmospheric transport model

In order to estimate biogenic sources and sinks of CO2 over Europe from atmospheric observations and inverse modelling, the fossil fuel CO2 component is normally calculated with the same model from bottom-up emissions and subtracted from the respective atmospheric observations to obtain the pure biogenic (and oceanic) signal. This requires precise emissions inventories. For Western Europe two spatially disaggregated emissions inventories for fossil fuel CO2 (and CO) are available today. (1) The Emission Database for Global Atmospheric Research (EDGAR), which provides annual mean emissions for the base years 1990, 1995 and 2000 on a global 1°x1° grid (Olivier et al. 2005). (2) At the Institute of Energy Economics and Rational Use of Energy (IER, University of Stuttgart, Germany) a new high-resolution emissions inventory on a 50km x 50km grid for the year 2000 and for the greater part of Europe was constructed based on UNFCCC (United Nations Framework Convention on Climatic Change) statistics (IER, pers. comm. 2004). Here national totals were disaggregated in time and space, separately for each sector, according to typical diurnal, weekly and seasonal cycles and spatial distributions of point, line and area sources, respectively. Especially the high temporal resolution of this IER data set is a substantial improvement compared to EDGAR because it allows a more realistic simulation of the temporal characteristics in the atmospheric tracer mixing ratios. In the modelling studies presented here the IER emissions inventories were complemented by the EDGAR emissions inventories in countries where IER data were missing to cover the entire model domain.

In order to estimate atmospheric mixing ratios over Europe for 2002 at hourly resolution as caused by the different European CO2 sources and sinks we used here the regional atmospheric transport model REMO (Chevillard et al. 2002). The same model is later on applied to simulate the temporal and spatial distribution of CO as well as for the sensitivity tests in Section 2.6 of this Chapter. In this set up of REMO the horizontal grid resolution is 55 km x 55 km and the model domain covers the area north of 30ºN. For the simulation of the total atmospheric CO2 mixing ratio the biosphere-atmosphere exchange as described by the terrestrial biosphere model BIOME-BGC (Churkina et al. 2003) as well as ocean fluxes (Takahashi et al. 1999) were included. For CO surface emissions from sources other than fossil fuel burning, i.e. biomass burning, soil and ocean emissions, as well as oxidation of methane and volatile organic compounds (VOC), photochemical processes for CO destruction in the atmosphere, and soil uptake were taken into account. For the meteorological part analysis data from the European Centre for Medium-Range Weather Forecasts (ECMWF) were used for initialisation and as lateral boundary information with a time resolution of six hours. For more details of the modelling parameters and source/sink distributions see Gamnitzer et al. (2006).

Figure X.1 (upper panels) shows the comparison of CO2 observations made by the German Umweltbundesamt at the Schauinsland observatory located on a mountain ridge at about 1205 m above sea level (asl) in the Black Forest in Germany (47°55’N, 7°54’E) (Schmidt et al. 2003) with REMO model estimates for a typical winter (February) and a typical summer (July) month in 2002. The second row of panels in Figure X.1 shows the REMO simulated CO2 offsets compared to Jungfraujoch values originating from biogenic sources and sinks, fossil fuel emissions (EDGAR inventory) and ocean fluxes. It is immediately obvious from these model estimates that even at this quite remote mountain site the regional fossil fuel CO2 signal is of similar magnitude as that from biospheric sources. This is the case not only in winter but also in summer where the biospheric signal is dominated by the net continental sink. Note that in particular in winter the observed CO2 variability seems to be mainly driven by transport of CO2 to the station from essentially co-located biogenic and fossil fuel sources. The third row of panels in Figure X.1 shows the comparison of the regional fossil fuel component when using EDGAR respectively IER emissions and the lowest panels the differences between EDGAR and IER regional fossil fuel CO2 simulated in REMO. These results show clearly that just simply using the different emissions inventories already causes CO2(foss) differences up to 4 ppm or about 30% of the signal itself.

Figure X.2 shows respective comparisons for the regional polluted Heidelberg site located in the Western outskirts of Heidelberg (49°25’N, 8°41’E, 115m asl), a medium sized city in the highly populated and polluted upper Rhine valley. Quasi-continuous CO2 (and other trace gas) measurements in Heidelberg are made on air collected from the roof of the institute’s building at about 30m above local ground. As expected, in Heidelberg the fossil fuel CO2 offset in winter is higher than the biospheric one, however still of comparable magnitude. Differences in CO2(foss) from REMO simulations, when based on different inventories (lowest panels in Figure X.2) can be as large as 20 ppm, particularly during strong night-time inversion situations in summer. Here the influence of the diurnal cycle in the IER emissions is particularly strong.

Altogether the examples presented here for Schauinsland and Heidelberg illustrate the importance of the task to determine fossil fuel CO2 over Europe correctly, as it contributes about 50% to the total observed continental CO2 signal. Only using emissions from different emissions inventories (but the same transport model) can cause differences of more than 30% in the simulated CO2(foss) component, and we are not yet able to decide which of the inventories is correct.

3.2.2 Long-term observations of 14CO2 in Europe

3.2.2.1 The European 14CO2 network and measurement techniques applied

Two-weekly integrated 14CO2 measurements in continental Europe have started already in 1959 at the station Vermunt in the Austrian Alps (Levin et al. 1985). These measurements mainly focussed on monitoring the transient of bomb 14CO2 in the northern hemisphere troposphere as a basis to study the dynamics of the global carbon cycle (Naegler and Levin 2006). In the 1980s and 1990s 14CO2 measurements were extended towards more polluted or semi-polluted regions (e.g. Levin, Münnich and Weiss 1980; Kuc 1986; Levin et al. 1989; Meijer, van der Plicht and Gislefoss 1995) with an additional focus on determining fossil fuel CO2 in these areas. Measurements are made by conventional counting technique (Kromer and Münnich 1992) which requires CO2 from large samples of 10-20 cubic meters of air. In order to obtain these large CO2 amounts some enrichment process is necessary, i.e. chemical absorption of CO2 in basic solution (Levin, Münnich and Weiss 1980; Meijer, van der Plicht and Gislefoss 1995) or, alternatively, physical adsorption on molecular sieve (Kuc 1986).

Figure X.3 shows the REMO-simulated monthly mean excess fossil fuel CO2 level over Europe (using Jungfraujoch as the background reference) at about 130m above ground in February 2002. The distribution of fossil fuel CO2 was estimated using the fossil fuel CO2 emissions from the EDGAR inventory for the year 2000. There is a general increase of fossil fuel CO2 over Europe from West to East of about 4-6 ppm with hot spots of up to 15 ppm in highly populated and industrialized regions. The locations of the European 14CO2 monitoring stations are also shown in Figure X.3. The current network consists of two marine Atlantic clean air sites (Mace Head, Ireland and Izaña, Canary Islands (not shown)), one moderately polluted coastal site in the Netherlands (Lutjewad), three urban sites (Orléans, Heidelberg and Krakow), two mountain stations (Schauinsland, Black Forest, Germany and Kasprowy Wierch, High Tatra, Poland), and one high-altitude Alpine site (Jungfraujoch, Swiss Alps).

2.2.2	Measurements of fossil fuel CO2 over Europe

The two-weekly integrated long-term observations of 14CO2, which are available at some of the European sites, provide an excellent measure of the seasonal cycle and the long-term trend of fossil fuel CO2 over Europe. As an example, Figure X.4 (upper panel) shows the monthly mean 14CO2 measurements at Heidelberg and Schauinsland stations (Levin et al. 2003; Gamnitzer et al. 2006) together with the continental 14CO2 reference level. To estimate this reference level we have used the quasi-continuous 14CO2 observations at Jungfraujoch (Levin and Kromer 2004), assuming that this site at an elevation of 3450m in the Swiss Alps most of the time monitors the undisturbed 14CO2 level in the free troposphere over Europe. We observe a small seasonal variation of 14CO2 at this background site which, to a large extent, is caused by the seasonal input of (natural) 14CO2 from the stratosphere to the troposphere, by interhemispheric exchange, but also by 14CO2 disequilibrium fluxes between the atmosphere and the biosphere (Hesshaimer 1997; Randerson et al. 2002; Naegler 2005). A small addition to the seasonal amplitude is caused by variations of the fossil fuel CO2 component in background air at mid northern latitudes (Naegler 2005). A harmonic fit curve (Nakazawa et al. 1997) was calculated through the Jungfraujoch observations, and assumed to represent the continental reference level over Europe.

Compared to the continental reference level, 14CO2 at Schauinsland and Heidelberg is almost always lower due to dilution of the 14C/12C ratio with 14C-free fossil fuel CO2 at these sites (Figure X.4, upper panel). In Heidelberg mean negative departures of 14C values from the background are about 30-60‰ in winter and between 10‰ and 30‰ in summer. At the mountain site Schauinsland, the corresponding departures in summer are only between 2 and 6 ‰ while in winter the fossil fuel effect is between 10 and 15‰. The measured CO2 mixing ratio CO2meas at the sites consists of three components, a background component CO2bg, a biospheric component CO2bio, and a fossil fuel component CO2foss. The 14C of these components are resp., 14Cmeas, 14Cbg, 14Cbio and 14Cfoss. To calculate the regional fossil fuel CO2 component, we can formulate two balance equations, one for the CO2 mixing ratio and one for the 14C/12C ratios (proportional to (14Ci + 1000)) (Levin et al. 2003)

CO2meas = CO2bg + CO2bio + CO2foss							(1)

CO2meas (14Cmeas+1000) = CO2bg (14Cbg+1000) + CO2bio (14Cbio+1000)

+ CO2foss (14Cfoss+1000) . 				(2)

The fossil fuel term in Eq. (2) is zero as 14Cfoss = -1000 ‰. Setting 14Cbio equal to 14Cbg, and combining both equations leads to the fossil fuel CO2 component to be calculated from Eq. (3):

CO2foss=CO2measΔCbg− ΔCmeasΔCbg+1000						(3)

(Note that the 14C difference of the total CO2 respiration flux (autotrophic and heterotrophic) compared to recent atmospheric 14C levels can be estimated to lie between +15‰ and +25‰ (Naegler 2005) which, if neglected as was done here, causes a possible mean bias in CO2(foss) of less than 0.2 ppm). At Schauinsland, the CO2 mixing ratio is measured continuously since 1972 by the German Umweltbundesamt (Schmidt et al. 2003). For Heidelberg, CO2 mixing ratios are available only from 1995 to 1998 and from 2002 onwards. For the periods without CO2 observations we approximate mixing ratios in Heidelberg using the mean seasonal offset from background air measured for 1995-1998 and 2002-2004 and adding the actual background mixing ratio for 48°N from GLOBALVIEW-CO2 (2005). Fossil fuel CO2 is then calculated from Eq. (3) for Schauinsland and Heidelberg as displayed in Figure X.4. The long-term mean fossil fuel CO2 component at Schauinsland is 1.4 ppm with slightly higher values in the winter half year (1.7 ppm) than in summer (1.0 ppm). In Heidelberg the long-term mean fossil fuel CO2 component is almost one order of magnitude larger than at Schauinsland (10.9 ppm). The very pronounced seasonality of fossil fuel CO2 in Heidelberg (winter: 14 ppm, summer: 6.5 ppm) is caused by a significant seasonality of the emissions, but mainly by seasonally varying atmospheric dilution of the emissions. The Rhine valley, particularly during winter, often experiences strong inversion situations causing large pile-ups of ground level emissions in the atmospheric surface layer. It is interesting, however, to note here, that even in winter about half of the total CO2 pile-up is due to biogenic emissions (Figure X.4, lowest panel). This was also estimated in the REMO modelling study. Similar to the Heidelberg and Schauinsland stations we are able to estimate (monthly mean) fossil fuel CO2 levels for other 14CO2 stations in Central Europe (not shown).

In summary: The current network of 14C-based fossil fuel CO2 observations is very sparse and unevenly distributed with important sites missing in South-Western, North-Eastern and Southern Europe. Mean observed CO2(foss) levels are on the order of a few up to a few tens of ppm, comparable to the REMO model simulations (Figure X.3).

3.2.3	How well do model estimates of the fossil fuel CO2 mixing ratio over Europe compare to observations ?

The regional fossil fuel CO2 mixing ratios derived from 14CO2 observations at Jungfraujoch, Schauinsland and Heidelberg have been compared to estimates from different models in an intercomparison study by Geels et al. (2006), and the results for July and December 1998 are displayed in Figure X.5. Also included are pure model comparisons for a number of sites where no 14CO2 observations exist. Plotted is the fossil fuel CO2 offset relative to the results for Mace Head, a marine background station at the Western coast of Ireland. In all model simulations the same emissions inventory (EDGAR3.0 for the year 1990) was used.

Except for the Heidelberg and Tver site, in July 1998 all model estimates agree within better than 1 ppm. At Schauinsland, the models generally underestimate CO2(foss) by about a factor of two while at Heidelberg all models except LMDZ overestimate fossil fuel CO2 by up to a factor of 2.5. In December 1998, the differences in the fossil fuel CO2 offset between models are very large (note the different scale compared to July), in particular at the ground level sites Cabauw, Heidelberg, Hungary and Tver. In Heidelberg, all models now underestimate fossil fuel CO2, in some cases (REMO, LMDZ and TM3) by up to a factor of 3. It is also important to note that even at the remote Russian site Tver with little direct influence from fossil fuel CO2 emissions, large differences between models occur (up to a factor of 3). This is worrying because a station such as Tver may be one of the key sites in Europe for regional inversions of CO2 observations and respective quantification of regional biogenic fluxes.

Figure X.6 shows a more detailed comparison of fossil fuel CO2 mixing ratios simulated with REMO and observed for individual two-weekly integrated samples collected in Heidelberg and at the Schauinsland mountain station in 2002. Again REMO was run with the two different emissions inventories from EDGAR and IER. While for Schauinsland in 2002 REMO results compare within 30% with the observations (for both emissions inventories), in Heidelberg in 2002 REMO generally overestimates regional CO2(foss), sometimes by more than a factor of two. This overestimation may be partly explained by transport deficiencies of the model (see Section 2.5) but Gamnitzer et al. (2006) could show that also the inventories must be wrong in the Heidelberg catchment area, particularly in summer.

These model-data comparisons of fossil fuel CO2 clearly show that there are large discrepancies between different models using the same emissions inventory and between models and observations, which can be partly explained by model transport deficiencies but also point to deficiencies in the emissions inventories. This underlines the importance of model validation and improvement of emissions inventories (as well as model transport) and the necessity of ongoing verification with 14CO2 observations. Such verification is necessary in particular at those sites that provide key CO2 concentration records for European CO2 inversions.

3.2.4	Which precision and temporal resolution of 14CO2 measurements is required to reliably monitor fossil fuel CO2 over Europe ?

Our current network of two-weekly integrated 14CO2 observations in Europe has shown to be very sparse (compare Figure X.3) and provides mean fossil fuel CO2 concentrations only on a monthly or at best on a weekly time resolution. Higher temporal resolution (i.e. hourly) is in principle possible if 14C analyses are made by Accelerator Mass Spectrometry (AMS) technique, which allows 14C analyses on air samples of less than 1 litre volume. However, for large (conventional counting) but also small volume samples (AMS analysis), the costs of 14CO2 measurements are high so that - at least for continuous long-term monitoring programs - higher than weekly resolution is normally not affordable.

The second important point to mention here is the significance of 14CO2 measurements to determine fossil fuel CO2: With the existing technology of high-precision conventional counting the precision of 14C measurements is limited to 14C =1-2 ‰ (Kromer and Münnich 1992). For AMS analyses (where much smaller sample sizes are measured), the standard precision of single analyses is normally on the order of ± 4 ‰. However, some laboratories recently reached a maximum precision of ±1.6 ‰ by analysis of multiple aliquots of the same sample (Turnbull et al. 2006). This does, however, require a large analytical effort and further increases costs of 14CO2 analyses. Still, a better precision than ±1.5 ‰ is probably not realistic on a routine basis. If we further assume that the background 14CO2 level (i.e. at the Jungfraujoch or at Mace Head, if continental European observations are concerned) can be determined also to about ±1 ‰ on a monthly mean basis (by fitting/averaging two-weekly samples), the fossil fuel CO2 component on a weekly integrated sample can be determined at best to ±0.7 ppm. This is about half of the mean fossil fuel CO2 component at a remote continental mountain site such as Schauinsland in summer (Figure X.4).

However, although the significance and temporal resolution of 14CO2 observations to determine fossil fuel CO2 are limited, 14CO2 measurements have the unique advantage to allow direct estimates of fossil fuel CO2 which is not possible with any other tracer (see Section 2.5). A dense network of (integrated) 14CO2 observations is, therefore, indispensable if fossil fuel CO2 shall be reliably monitored over Europe.

3.2.5	Surrogate tracers to quantify fossil fuel CO2 on a continuous basis

The limited temporal resolution, the costs, and the rather low sensitivity of 14CO2 measurements were the main reasons why a number of proxies for fossil fuel CO2 have been suggested and used in the past to indirectly derive fossil fuel CO2 mixing ratios in the atmosphere. The demands to a good proxy for fossil fuel CO2 are: (1) The sources of the tracer should be related in a unique way to those of fossil fuel CO2, (2) the tracer should be easily and precisely measured continuously, and (3) it should behave conservatively in the atmosphere or its sink processes should be well understood so that it is possible to model them accurately. None of the tracers mentioned above fulfils all these demands: While CFCs and SF6 are chemically stable in the troposphere, their emissions are not directly co-located with those of fossil fuel CO2, in particular on the local and regional scale. On a larger continental or hemispheric scale, SF6 may be better suited (Rivier et al. 2006). In the case of CO, its major sources are very closely linked to those of fossil fuel CO2: Any fossil fuel combustion process with CO2 as an end product is, to some extent, associated with the production of CO. However, the chemical sink of CO in the atmosphere has a strong diurnal and seasonal cycle, and CO has also an atmospheric source from oxidation of volatile organic compounds (VOCs). Similarly important, the CO/CO2 emission ratio of different fossil fuel sources, such as domestic heating or emissions from traffic, can vary by orders of magnitude (UNFCCC 2005; Olivier et al. 2005) questioning the use of this tracer for quantitative estimates, in particular in polluted or semi-polluted areas where the source distribution is heterogeneous and the source mix can be quite variable.

At least two experimental (Gamnitzer et al. 2006; Turnbull et al. 2006) and one modelling study  (Rivier et al. 2006) have been pursued recently to investigate the suitability of different surrogate tracers. However, in the study by Rivier et al. (2006) no 14CO2 measurements were available providing reliable validation of the simulated fossil fuel CO2 component. Turnbull et al. (2006) compare 14C-, CO- and SF6-based fossil fuel CO2 mixing ratios on atmospheric samples collected in New England and Colorado. They find implausibly large differences between SF6-based and 14C-based estimates which disqualify SF6 as a quantitative fossil fuel surrogate tracer. Their CO-based estimates agree better with 14C-based fossil fuel CO2 estimates but, as mentioned above, seasonally coherent biases point to other source and sink processes influencing regional CO mixing ratios which need to be taken into account.

A very detailed study using three years of quasi-continuous (two-)weekly integrated 14CO2 measurements as well as measured diurnal cycles of 14CO2 in combination with continuous CO2 and CO measurements and the REMO model to simulate CO and CO2 mixing ratios from emissions inventories was performed by Gamnitzer et al. (2006). The observational data used by Gamnitzer et al. (2006) are presented in Figure X.7 as monthly means and extended up to the end of 2004.

The upper two panels of Figure X.7 again show 14CO2 at Heidelberg and Jungfraujoch (background) as well as the 14C-based regional fossil fuel CO2 offset in Heidelberg (compare Figure X.4). The third panel shows monthly mean regional CO offsets relative to the marine background estimated from Atlantic stations in mid northern latitudes (Masarie, pers. comm., extrapolated to 2004), while the lowest panel gives the ratios of CO/CO2(foss) calculated from the CO2(foss) and CO data in the two middle panels. On a monthly mean basis the locally observed CO/CO2(foss) ratio in Heidelberg varies by only ± 21% and shows no systematic seasonal cycle. This indeed suggests that CO may be well suited as a surrogate tracer for fossil fuel CO2.

A more detailed comparison by Gamnitzer et al. (2006) of the Heidelberg observations for 2002 with REMO-simulated CO and CO2(foss) offsets does, however, give a somewhat different picture: While the ratios of CO/CO2(foss) show good agreement between observed (13.5±2.5 ppb CO/ppm CO2(foss)) and modelled values (EDGAR inventory: 13.0±0.9 ppb CO/ppm CO2(foss), IER inventory: 11.5±1.1 ppb CO/ppm CO2(foss)) individual offsets of CO2(foss) (see Figure X.6) but also of CO (not shown) differ by up to a factor of two between model and observations. This may be either due to errors in the model transport or due to errors in respective emissions inventories or both. Gamnitzer et al. (2006) have, therefore, corrected the model results for transport deficiencies by means of continuous 222Radon observations and respective model results. But even after this 222Radon correction, differences between observations and REMO simulations are as large as 100% for both components, CO2(foss) and CO. For CO2 these remaining differences are probably due to deficiencies in the emissions inventories, but for CO also the non-fossil sources and sinks of CO have large uncertainties. Gamnitzer et al. (2006) have estimated the uncertainty of the CO production via VOC oxidation to more than 50%, and that of the soil sink of CO, which is not well quantified at all in the literature (Sanhueza et al. 1998), to up to 100%. They have performed a number of model scenarios quantifying possible biases in the CO model estimates associated with VOC oxidation and soil uptake which yield a total range of almost 100% in the modelled CO offset in Heidelberg and respective systematic changes of 50% in the modelled CO/CO2(foss) ratio. The good agreement between modelled and observed CO/CO2(foss) ratio in Heidelberg could, thus, be purely accidental.

Still the good agreement between modelled and observed CO/CO2(foss), whether accidental or real, allowed Gamnitzer et al. (2006) to correct modelled CO2(foss) with modelled and measured CO in a similar way as the 222Radon correction. With this correction, agreement of better than 20% root mean square (RMS) error for EDGAR emissions and better than 35% RMS error for IER emissions between observed (14C-based) and simulated REMO plus emissions inventory derived CO2(foss) in Heidelberg could be achieved. The CO-correction or normalisation obviously takes care of some deficits of simulated transport in REMO but also of other CO modelling deficits. This example shows that in a situation where the CO/CO2(foss) ratio of emissions is more or less correct continuous CO measurements (in combination with CO model results) can help to correct simulated CO2(foss) mixing ratios for model and partly also inventory deficiencies.

The implications of these CO studies are twofold: (1) When CO alone shall be used as a quantitative tracer for fossil fuel CO2, the CO/CO2(foss) ratio of emissions as well as its temporal changes need to be known accurately, and (2) reliable estimates of the non-fossil sources and the sinks of CO are required as these may change atmospheric CO/CO2(foss) ratios by up to 50%.

3.2.6	Can we combine integrated 14CO2 and continuous CO observations to improve estimates of continuous regional CO2(foss) ?

In the preceding section we have seen that CO can be a valuable tracer for fossil fuel CO2 if the CO/CO2(foss) ratios of emissions from inventories in the catchment area of the site are more or less correct. Besides continuous CO observations this requires also accurate modelling of atmospheric CO, and the reliability of the CO/CO2(foss) ratio of the emissions must still be confirmed e.g. by comparison with (integrated) 14CO2 measurements at the sites in question. But what is really needed to properly determine fossil fuel CO2 over Europe is a simple observational approach not biased by large uncertainties of CO modelling and/or inventory data. Such an approach has been recently suggested by Levin and Karstens (2007): It is based on integrated 14CO2 and continuous CO observations where CO2(foss) mixing ratios could then be estimated from the measured hourly CO mixing ratios and a weekly mean CO to CO2(foss)14C-based ratio according to

ΔCO2(foss)hourly =ΔCOhourlymeas⋅ΔCO2(foss)weeklyC−basedΔCOhourlymeasweekly			(4)

Levin and Karstens (2007) used REMO simulated CO2(foss) and CO records for the year 2002 to determine the uncertainty of this approach for different sites in Europe by assuming that these modelled mixing ratios represent the correct hourly mixing ratios of both gases. From the continuous hourly modelled CO2(foss) and CO values weekly means and subsequent weekly mean CO/CO2(foss) ratios were calculated and these mean ratios were then applied to the original high-resolution modelled CO record to re-calculate a high-resolution CO2(foss) record

ΔCO2(foss)hourlyre−calculated=ΔCOhourlymodel⋅ΔCO2(foss)hourlymodelweeklyΔCOhourlymodelweekly		(5)

The differences between the original high-resolution CO2(foss) record (i.e. the true record) and the CO-based re-calculated CO2(foss) record (Eq. 5) provide a measure of the minimum uncertainty of any attempt to use hourly CO measurements to derive an hourly CO2(foss) record according to Eq. (4). Figure X.8 shows the results of this test for Heidelberg, separately with REMO simulations based on IER and those based on EDGAR emissions for February and July 2002. The CO2(foss) REMO records named “original” are the originally calculated values assumed to represent the truth.

In addition to estimates from Eq. (5), we made here a second test where CO2(foss) was calculated in a similar way, but replacing the (REMO simulated “observed”) mean CO to mean CO2(foss) ratio by the mean ratio of the respective emissions in the catchment area (which we call “CO2(foss) inventory”):

ΔCO2(foss)hourlyinventory =ΔCOhourlymodel⋅CO2(foss)inventoryCOinventory			(6)

For these inventory-based estimates the mean ratios of emissions were calculated assuming a catchment area corresponding to eleven 1° x 1° EDGAR grid boxes around Heidelberg (the same catchment area as in the Gamnitzer et al. (2006) study). The CO emissions included also non-fossil sources and biogenic emissions.

The root mean square (RMS) deviations of CO2(foss) from the true values when using the weekly mean modelled ratios and Eq. (5) for Heidelberg are about 18% in the case of IER emissions and about 14% in the case of EDGAR emissions. Deviations for EDGAR are smaller because they are caused only by differences of the CO/CO2(foss) emissions ratios in the different parts of the catchment area (which the original REMO calculations are taking care of) and the processes influencing the chemistry of CO (production by VOC oxidation and destruction by OH). The larger deviations in the IER results are caused by both, spatial and also temporal (i.e. diurnal) variations of the CO/CO2(foss) ratio of emissions (these vary by about 10% in winter and up to 20% in summer), which again are taken care of in the original REMO results. If only the pure inventory ratios are used to estimate CO-based CO2(foss) (Eq. 6) the RMS errors increase to about 30% in the case of IER emissions and to about 20% in the case of EDGAR emissions. This illustrates the importance of atmospheric chemistry processes as well as soil uptake influencing CO mixing ratios in the continental troposphere. At other European sites the relative RMS error for the application of atmospheric ratios (according to Eq. 5) increased with decreasing fossil fuel CO2 offset showing maximum uncertainties at Schauinsland station of about 40% for the IER and about 35% for EDGAR emissions inventories. This is due to the fact that the non-fossil sources of CO become increasingly important with decreasing fossil fuel component.

In summary, and in the real world, if continuous CO measurements are available at a CO2 monitoring site as well as e.g. weekly integrated 14CO2 observations to obtain a good estimate of regional CO2(foss) and of the mean CO/CO2(foss) ratio, we will be able to estimate regional fossil fuel CO2 at hourly resolution with an uncertainty of about 20-40% (the uncertainty of the 14C-based determination of fossil fuel CO2 of about ±1 ppm (see Section 2.4) must be added to this number). Note, however, that this estimate would be purely based on observations and would not be biased by uncertainties of emissions inventories or in modelling of atmospheric CO!

3.3	How should an observational network for continuous monitoring of the fossil fuel CO2 component over Europe look like?

From the studies by Turnbull et al. (2006) and in particular by Gamnitzer et al. (2006) we have learned that fossil fuel CO2 estimates based solely on emissions inventories can be largely biased, in the case of the Heidelberg measurement site by up to a factor of two. Moreover, the model comparison study by Geels et al. (2006) showed that large differences of up to a factor of two or even more exist in monthly estimates of the fossil fuel component in atmospheric CO2 between different models, even when using the same fossil fuel emissions inventories as input data. With a fossil fuel CO2 signal over Europe being of the same magnitude as the respective biospheric signals, this is clearly a non-satisfactory situation if we try to understand causes and trends in the biosphere-atmosphere CO2 exchange in this region.

Continuous CO observations together with CO model estimates as applied by Gamnitzer et al. (2006) have the potential to correct model-derived CO2(foss) mixing ratios yielding final uncertainties of 20-35% if the CO/CO2(foss) ratios of the emissions inventory used are reliable. Such a CO correction does, however, require not only modelling of the fossil fuel CO2 component but also modelling of CO with all its uncertainties in non-fossil CO sources including oxidation of VOC, CO sinks in the atmosphere, as well as soil deposition. For such an approach, also errors associated with uncertainties in model transport have to be taken into account which will additionally complicate the interpretation and application of the CO correction method. Taking advantage of CO observations can then correct for some shortcomings related to transport and other unaccounted-for processes and may be one step forward towards improvement of estimating fossil fuel CO2 based on modelling studies. The success of such an approach does, however, fully depend on the reliability of the CO/CO2(foss) ratios available from emissions inventories.

The most reliable approach towards a better quantification of high-resolution fossil fuel CO2 over Europe is, however, purely observation-based, namely a combination of e.g. weekly integrated high precision 14CO2 measurements with precise continuous CO observations  (Levin and Karstens 2007). This approach will be accurate to within 20-40%, depending on the site, with additional random weekly mean biases of about 1 ppm caused by the 14CO2 measurement error. This approach seems to be a fair compromise of costs and affordable observational effort.

We, therefore, recommend high-precision weekly integrated 14CO2 observations as well as precise continuous CO measurements at any continental CO2 measurement site which shall be used for inversions of CO2 sources and sinks.

Acknowledgements

We wish to thank the personnel at the Schauinsland and Jungfraujoch stations for collecting the atmospheric 14CO2 samples, as well as Frank Meinhardt at UBA Schauinsland for making available the continuous atmospheric CO2 measurements from this site. The members of the Heidelberg Radiocarbon Laboratory are gratefully acknowledged for their ongoing effort to analyse all 14CO2 samples presented here. Camilla Geels made available to us Figure X.5. Dietmar Wagenbach (IUP) and Tobias Naegler (IUP) have helped with fruitful discussions. This work was funded by the European Union under contracts No. EVK2-CT-1999-00013 (AEROCARB) and GOCE-CT2003-505572 (CarboEurope-IP).

References:

Bakwin, P. S., Hurst, D. F., Tans, P. P., and Elkins, J. W. 1997. Anthropogenic sources of halocarbons, sulfur hexafluoride, carbon monoxide, and methane in the south-eastern United States. J. Geophys. Res. 102:15,915-15,925.

Chevillard, A., Karstens, U., Ciais, P., Lafont, S., and Heimann, M. 2002. Simulation of atmospheric CO2 over Europe and western Siberia using the regional scale model REMO. Tellus 54B:872-894.

Churkina, G., Tenhunen, J., Thornton, P., Falge, E. M., Elbers, J. A., Erhard, M., Grünwald, T., Kowalski, A. S., Rannik, Ü., and Sprinz, D. 2003. Analyzing the ecosystem carbon dynamics of four European coniferous forests using a biogeochemistry model. Ecosystems 6:168-184.

Gamnitzer, U., Karstens, U., Kromer, B., Neubert, R. E. M., Meijer, H. A. J., Schroeder, H., and Levin, I. 2006. Carbon Monoxide: A quantitative tracer for fossil fuel CO2 ? J. Geophys. Res. 111:D22302 doi:10.1029/2005JD006966.

Geels, C., Gloor, M., Ciais, P., Bousquet, P., Peylin, P., Vermeulen, A. T., Dargaville, R., Aalto, T., Brandt, J., Christensen, J. H., Frohn, L. M., Haszpra, L., Karstens, U., Rödenbeck, C., Ramonet, M., Carboni, G., Santaguida, R. 2006. Comparing atmospheric transport models for future regional inversions over Europe. Part 1: Mapping the CO2 atmospheric signals over Europe. Atmos. Chem. Phys. Discuss. 6:3709-3756.

GLOBALVIEW-CO2 2005. Cooperative Atmospheric Data Integration Project - Carbon Dioxide. CD-ROM, NOAA CMDL, Boulder, Colorado (Also available on Internet via anonymous FTP to ftp.cmdl.noaa.gov, Path: ccg/co2/GLOBALVIEW ).

Hesshaimer, V. 1997. Tracing the global carbon cycle with bomb radiocarbon, Ph.D. thesis, University of Heidelberg.

Kromer, B. and Münnich, K. O. 1992. CO2 gas proportional counting in Radiocarbon dating – review and perspective. In Radiocarbon after four decades, eds. R. E. Taylor, A. Long and R.S. Kra pp.184-197. New York: Springer-Verlag.

Kuc, T. 1986. Carbon isotopes in atmospheric CO2 of the Krakow region: A two-year record. Radiocarbon 28:649-654.

Levin, I., Münnich, K. O., and Weiss, W. 1980. The effect of anthropogenic CO2 and 14C sources on the distribution of 14CO2 in the atmosphere. Radiocarbon 22:379-391.

Levin, I., Kromer, B., Schoch-Fischer, H., Bruns, M., Münnich, M., Berdau, D., Vogel, J. C., and Münnich, K.O. 1985. 25 Years of tropospheric 14C observations in Central Europe. Radiocarbon 27:1-19.

Levin, I., Schuchard, J., Kromer, B., and Münnich, K. O. 1989. The continental European Suess effect. Radiocarbon 31:431-440.

Levin, I., Kromer, B., Schmidt, M., and Sartorius, H. 2003. A novel approach for independent budgeting of fossil fuels CO2 over Europe by 14CO2 observations. Geophys. Res. Lett., 30(23):2194, doi:10.1029/2003GL018477.

Levin, I., and Kromer, B. 2004. The tropospheric 14CO2 level in mid-latitudes of the Northern Hemisphere (1959-2003). Radiocarbon 46:1261-1272.

Levin, I., and Karstens, U. 2007. Inferring high-resolution fossil fuel CO2 records at continental sites from combined 14CO2 and CO observations. Tellus B, doi:10.1111.j1600.0889.2006.00224.x.

Marland, G., Boden, T.A., and Andres, R. J. 2006. Global, Regional, and National CO2 Emissions. In Trends: A Compendium of Data on Global Change. Carbon Dioxide Information Analysis Center, Oak Ridge National Laboratory, U.S. Department of Energy, Oak Ridge, Tenn., U.S.A.

Meijer, H. A. J., van der Plicht, J., and Gislefoss, J. S. 1995. Comparing long-term atmospheric 14C and 3H records near Groningen, the Netherlands with Fruholmen, Norway and Izaña, Canary Islands 14C stations. Radiocarbon 37:39.

Naegler, T., and Levin, I. 2006. Closing the global bomb radiocarbon budget. J. Geophys. Res.111, D12311, doi: 10.10292005JD006758.

Naegler, T. 2005. Simulating bomb Radiocarbon: Implications for the Global Carbon Cycle. Ph.D. thesis, University of Heidelberg.

Nakazawa, T., Ishizawa, M., Higuchi, K., and Trivett, N. B. A. 1997. Two curve fitting methods applied to CO2 flask data. EnvironMetrics 8:197-218.

Olivier, J. G. J., van Aardenne, J. A., Dentener, F., Ganzeveld, L., and Peters, J. A. H. W. 2005. Recent trends in global greenhouse gas emissions: regional trends and spatial distribution of key sources. In Non-CO2 Greenhouse Gases (NCGG-4), ed. A. van Amstel (coord.), pp. 325-330. Millpress, Rotterdam, ISBN 9059660439. Information available online at www.rivm.nl.

Potosnak, M. J., Wofsy, S. C., Denning, A. S., Conway, T. J., Munger, J. W., and Barnes, D. H. 1999. Influence of biotic exchange and combustion sources on atmospheric CO2 concentrations in New England from observations at a forest flux tower. J. Geophys. Res. 104:9561-9569.

Randerson, J.T., Enting, I.G., Schuur, E.A.G., Caldeira, K., and Fung, I.Y. 2002. Seasonal and latitudinal variability of troposphere 14CO2: Post bomb contributions from fossil fuels, oceans, the stratosphere, and the terrestrial biosphere. Global Biogeochem. Cycles 16, doi:10.1029/2002GB001876.

Rivier, L., Ciais, P., Hauglustaine, D.A., Bakwin, P., Bousquet, P., Peylin, P., and Klonecki, A. 2006. Evaluation of SF6, C2Cl4 and CO as surrogate tracers for fossil fuel CO2 in the United States using models and continuous atmospheric observations on high towers. J. Geophys. Res. 111, D16311, doi:10.1029/2005JD006725.

Sanhueza, E., Dong, Y., Scharffe, D., Lobert, J. M., and Crutzen, P. J. 1998. Carbon monoxide uptake by temperate forest soils: the effects of leaves and humus layers. Tellus 50B:51-58.

Schmidt, M., Graul, R., Sartorius, H., and Levin, I. 2003. The Schauinsland CO2 record: 30 years of continental observations and their implications for the variability of the European CO2 budget. J. Geophys. Res. 108 (D19), 4619, doi:10.1029/2002JD003085.

Stuiver, M., and Polach, H. A. 1977. Discussion: Reporting of 14C data. Radiocarbon 19:355-363.

Suess, H.E. 1955. Radiocarbon concentration in modern wood. Science 122:415.

Takahashi, T., et al. 1999. Net sea-air CO2 flux over the global oceans: An improved estimate based on the sea-air pCO2 difference. Proceedings of the 2nd CO2 in Oceans Symposium, Tsukuba, JAPAN, January 18-23.

Turnbull, J. C., Miller, J. B., Lehman, S. J., Tans, P. P., Sparks, R. J., and Southon, J. 2006. Comparison of 14CO2, CO, and SF6 as tracers for recently added fossil fuel CO2 in the atmosphere and implications for biological CO2 exchange. Geophys. Res. Lett. 33, L01817, doi: 10.1029/2005GL024213.

UNFCCC (United Nations Framework Convention on Climate Change) 2005. Greenhouse gases database. Bonn, Germany. Available online at ghg.unfccc.int, Download on 10. Feb. 2005.

Figure Captions:

Figure X.1:

Comparison of continuous hourly CO2 observations at the mountain station Schauinsland with respective REMO model estimates for February and July 2002. The upper panels show the observations in comparison with the model results including all CO2 flux components, the second panels show separately simulated components of the total CO2 signal (ocean, terrestrial biosphere, and EDGAR fossil fuel emissions). The third rows compare the simulated fossil fuel CO2 components if based on the two different inventories used in this study, IER and EDGAR, while the bottom panels show the differences between these two.

Figure X.2:

Same as Figure X.1 but for Heidelberg

Figure X.3:

Monthly mean fossil fuel CO2 over Europe at 130 m above ground in February 2002 as simulated with REMO using the EDGAR emissions inventory for 2000. The stations where quasi-continuous 14CO2 measurements are performed are marked.

Figure X.4:

Monthly mean 14CO2 measurements in Heidelberg and at Schauinsland station in comparison to the continental reference level over Europe as derived from observations at Jungfraujoch (upper panel) 14C concentrations are per mil deviations of the 14C/12C ratio from NBS oxalic acid standard activity (Stuiver and Polach 1977). Fossil fuel CO2 component at Schauinsland (middle panel) and Heidelberg (lower panel) as calculated from the respective 14C difference to the continental reference level according to Eq. (3). In the case of Heidelberg also the total CO2 offset compared to marine background air (GLOBALVIEW-CO2 2005) is plotted (grey shaded). Note that the biospheric contribution in summer (minimum offsets) is larger than that from fossil fuels while it is of the same magnitude as the fossil fuel component during winter.

Figure X.5

(adopted from Figure 4 of Geels et al. (2006)): East-West transect of the fossil fuel CO2 component over Europe in July and December 1998 as estimated with five different transport models with the same underlying emissions. Note that TM3, LMDZ and HANK are coarser resolution models with grids of approximately 1° x 1° or larger (TM3) while DEHM and REMO have an approximate resolution of 0.5° x 0.5° over Europe  (obs: Observations; mhd: Mace Head, 53°19’N, 9°53’W, 26m asl; cbw: Cabauw tower, 51°58’N, 4°55’E, 213m above ground; prs: Plateau Rosa, 45°56’N, 7°42’E, 3480m asl, sch: Schauinsland, 47°55’N, 7°55’E, 1205 m asl, jfj: Jungfraujoch, 46°33’N, 7°59’E, 3580m asl; hei: Heidelberg, 49°24’N, 8°42’E, 115m asl, 30m above ground; hun: Hegyhatsal tower 46°57’N, 16°39’E, 115m above ground; pal: Pallas, 67°58’N, 24°07’E, 560m asl; tvr: Tver tower, 56°57’N, 32°57’E, 29m above ground)

Figure X.6:

Comparison of 14C-based fossil fuel CO2 observations (two weekly integrated samples from the year 2002) with respective REMO simulation results for the same time intervals at Schauinsland (upper panel) and Heidelberg (lower panel).

Figure X.7:

Monthly mean values of 14CO2 at Heidelberg and Jungfraujoch (upper panel), 14C-based estimates of CO2(foss) and CO in Heidelberg (middle panels). The lowest panel shows the ratios of regional CO/CO2(foss).

Figure X.8:

Sensitivity tests of the uncertainty of CO-based CO2(foss) estimates: Upper two panels: CO2(foss) estimated from “atmospheric” CO records and weekly mean CO/CO2(foss) ratios calculated from original “atmospheric” results according to Eq. (5) and from CO/CO2(foss) emissions ratios directly, according to Eq. (6) in comparison to original “atmospheric” CO2(foss) records (plotted for IER and EDGAR inventories separately). Lower two panels: Respective differences.

Figure X.1

Figure X.2

Figure X.3

Figure X.4

Figure X.5

Figure X.6

Figure X.7

Figure X.8