# Chapter 18

# A roadmap for a continental scale greenhouse gas observing system

Han Dolman, Philippe Ciais, Riccardo Valentini, Detlef Schulze and Annette Freibauer.

The growth rate of CO2 emissions from fossil fuel burning and other industrial processes has been accelerating from 1.1% per year in 1990-1999 to more than 3% per year in the period 2000-2004. (Raupach et al., 2007). This increase may have contributed to an observed increase in the atmospheric growth rate of CO2;  during 1990-1999 the atmospheric CO2 growth rate was estimated at 3.1 Gton C per year; for the period of 2000-2005 this is estimated at 4.2 Gton C per year (Denman et al., 2007). The dominant factor causing this imbalance appears to be a faster increase in fossil fuel emissions relative to the sink capacity of the land and oceans.  The increase in emissions has put the growth rates of atmospheric carbon in recent years at the top end of the emission scenarios of the IPCC that are used in climate models to predict the effects of greenhouse gasses on the earth’s climate. Rapauch’s et al (2007) analysis points clearly to the need to continue observing the carbon cycle as a means both to verify and fingerprint emissions. They furthermore suggest that increased understanding of where the emissions are accelerating, and where decarbonization of energy use takes place, has important implications for global equity and burden sharing in the costs of global change.  This makes understanding the global carbon balance an issue of direct and fundamental relevance to society.

Under the Kyoto protocol, the majority of industrial countries have agreed to cut emissions and take other measures to reduce the amount of greenhouse gases in the atmosphere. The effectiveness of greenhouse gas emission reductions or lack of it, as shown by Raupach et al. (2007) analysis, needs to be monitored through observations. Denman et al. (2007) noted for the IPCC’s Fourth Assessment Report that the budget of anthropogenic CO2 can now be calculated with improved accuracy. On land, they suggest that there is a better understanding of the contribution to the build up of CO2 in the atmosphere since 1750 associated with land use and of how the land surface and the terrestrial biosphere interact with a changing climate. At the global scale, inverse techniques (Chapter 5) used to infer the magnitude and location of major fluxes in the global carbon cycle have continued to mature, reflecting both the refinement of the techniques and the availability of new observations.

Monitoring the carbon cycle and understanding the underlying carbon sequestration and emission mechanisms thus serves essentially two goals (e.g. Chapter 2): increased understanding of the carbon cycle in the context of the earth system and its future evolution and provision of the data and analysis required to inform policy decisions about management of the global carbon cycle. The chapters of this book have shown how precise and carefully executed observations can help us to continue improving our continental scale estimates and improve our process understanding. However, it must also be said that our current scientific and technological capacity is not yet up to the task of providing sufficiently accurate regional and national budgets of greenhouse gas sources and sinks and of ecosystem response to climate change. Thus for instance, the precise geographic distribution of the land and ocean sources and sinks of CO2 has remained elusive and this uncertainty hinders future CO2 and climate change projections and verification of approaches to curb emissions.

As the chapters in this book show, approaches to establish greenhouse budgets so far have relied on a combination of various methods to assess the individual budget components and this is likely to remain the case in the near future.  This book has attempted to show both the “top down” and “bottom up” approaches, and more in particular how these need to be merged into a single multiple constraint estimate to provide realistic estimates of the continental scale greenhouse gas budget. We have deliberately chosen to show that a combination of process understanding and data integration is required to achieve the ambitious goal of a multiple constraint methodology.

The atmosphere efficiently mixes and integrates surface fluxes that vary both spatially and temporally. This distribution of regional fluxes over land and oceans can be retrieved using observations of atmospheric CO2 and related tracers within models of atmospheric transport. Atmospheric inversions belong to this approach, and they determine an optimal set of fluxes that minimise the mismatch between modelled and observed concentrations, accounting for measurement and model errors (Chapter…). Chapters …. provide examples of this approach for both CO2 and CH4  at the continental to regional scale, although the latter is still very much in development (Chapter..).  This is what is generally referred to as the called the ‘top-down’ approach to estimating fluxes.

These techniques using the current set of atmospheric observation sites (Chapter 2, 18) strongly suggests the existence of a significant carbon sink over northern ecosystems. As mentioned, the atmospheric network however is still too sparse over most of the continents to fully use the potential of this technique. This sparseness of the observation network severely limits adding more spatial resolution to this continental or even latitudinal result. There is some hope that tall tower networks (reference?) may eventually fill in this gap, but their implementation is still at early stages. At the regional scale level more dense observation networks are required to achieve the desired improved spatial and temporal resolution (e.g. Chapter..).

In contrast to the inversion estimates, the fluxes can also be determined by measuring carbon stock changes at repeated intervals, from which time-integrated fluxes can be deduced, or by direct observations of the fluxes. This is known as the ‘bottom-up’ approach. Ideally the top down and bottom up estimates should agree within error bounds.

It is worth considering two recent papers that tried to compare various bottom-up and top-down estimates of the carbon balance of the European and North American continents (Janssens et al., 2003, Pacala et al., 2001). Table 2 compares the bottom up estimates of the two papers.

|  | Area US (Mha) | Area EU (Mha) | Rate US (g C m –2 yr-1) | Rate EU (g C m –2 yr-1) |
| --- | --- | --- | --- | --- |
| Forest trees | 247 | 339 | 44.5-60.7 | 107 |
| Other wooded land | 247 | 50 | 12.1-60.7 | 28.0 |
| Croplands | 184 | 326 | 21.5 | -92 |
| Non-forest, non cropland (including grassland and woody encroachment) | 335 | 151 | 35.8-38.8 | 67 |
| Total area (Mha) and total sink (PgC a-1) | 1013 | 866 | 0.26-0.47 | 0.11 |

Table 1. Comparison of bottom-up estimates of European and North American carbon balance based on Pacala et al. (2001) and Janssens et al. (2003).

When biomass inventories (e.g. Chapter….) are used to report on the sink strength of forest, lower values are found than those obtained by inverse models. However, the table also highlights important differences between the inventories of both continents. These become apparent when we look at the rates of sequestration used in the two studies, rather than their overall total sink estimates. The first difference is the rate of forest C-sequestration. Paccala et al. using forest inventory data calculate a value that is two times as low as the value used by Janssens et al. (2003), which is based on eddy correlation (direct) flux measurements. The latter are downscaled using a fixed NEP/NBP ratio of 0.47,  taken from inventory studies. Pacala et al. (2001) in contrast, use the estimates from the forest inventory (Chapter …) and model estimates to account for losses litter, slash, woody debris, fires and harvest. Rates of carbon uptake by eddy correlation in the US and EU made by eddy correlation tend to be similar, even higher in the US (Chapter …). There is thus still considerable uncertainty attached to these numbers and their correct use and interpretation in carbon balance estimates. The most apparent difference is the estimate for croplands. In the EU these are estimated to be sources of carbon, whereas in the US they are estimated as sinks. There is, as Janssens  et al. (2003) argue, considerable uncertainty surrounding this number, as it is based on a single model study, that can hardly be validated with existing empirical data. Chapter .. shows how progress in this area can be achieved by modelling and observation sof the full greenhouse gas balance at the farm and regional level. Importantly, data on land use management are required to achieve improved realism in these estimates.

Eddy covariance techniques allow continuous monitoring of CO2, H2O, heat fluxes over vegetation canopies (Chapter…). They are also increasingly used to measure the fluxes of trace gasses of CH4 and N2O. The temporal resolution of a day or so for eddy flux towers is sufficient to capture the variability in terrestrial fluxes as it is driven by changing weather patterns (e.g. the effect of frost or drought on forests). There is a need to transform these largely research based networks of towers into a routinely operating system. However, terrestrial ecosystem carbon fluxes are so heterogeneous and variable that it will be virtually impossible to measure fluxes over all kinds of ecosystems continually over Europe and adjacent regions (e.g. Chapter …).  As it is impossible to sample all the possible permutations of land use (history) and climate with eddy covariance towers, a limited number of ecosystem observation sites may need to be selected where continuous and intense measurements of the CO2, H2O, and heat fluxes are made. Flux data at these sites can then be complemented by carbon pool and biophysical parameters measurements to gain full understanding of the processes which control the greenhouse balance of ecosystems. These long-term ecosystem observation sites can be the basis for upscaling, using advanced models which can integrate the terrestrial greenhouse gas fluxes.

In general, our analysis shows that forest are relatively well sampled in eddy covariance networks, but that uncertainties in both the CH4 and N2O (chapter…) budgets would require increased focus on peatland (chapter…) and grassland ecosystems (chapter …). Redesigning the networks to move from a carbon observing system to a full greenhouse gas observing system may require major relocations and improvements to the current networks.

A full greenhouse monitoring network should ideally be planned as a long-term network designed from the very beginning to measure both the atmosphere and representative ecosystems in an integrated manner ( e.g chapter….).  As chapter 12 shows, ther is also a need to determine the precise magnitude and variability of the lateral fluxes in trade and river transport that affect continental scale averaged budgets. Regional integrated carbon measurement networks already exist in the United States (for the atmosphere), and in Japan and Australia. Yet, a unified European long-term carbon observing network is still lacking. The state of the art is that research groups are currently bearing the burden of running the flux towers and the atmospheric observatories, with these activities being combined in research projects (e.g. chapter…). Although, measuring sites are numerous in Europe (see Figure 1), the funding is short-term, many sites have opened or closed, and instrumentation is not always traceable on uniform scale. Such measurement variations make it difficult to detect the changes in trends that reflect the efficiency of management strategies or reveal surprises.. The CARBOEUROPE series of projects essentially led to a research network that explored the feasibility and the methodologies of the integrated top-down and bottom-up process, but this is not an operational observation network. Nevertheless, a lot of work still needs to be done

Operational networks ideally should adhere to the monitoring principles of the Global Climate and Terrestrial Observing Systems GCOS and GTOS. These consist of an established set of principles (http://www.wmo.int/pages/prog/gcos/Publications/GCOS_Climate_Monitoring_Principles.pdf) that are far from trivial in its practical execution. They concern detailed measurement protocols, quality control, data management plans for secure long term operation.   For Europe plans for an Integrated Carbon Observing System (ICOS; not yet a full Greenhouse Gas system) exist.  They involve the establishment of about 30 atmospheric main sites with co-located aircraft observation, and 30 ecosystem main sites over Europe. A permanent quality control system would be established to ensure the long term integrity of these main sites. The main sites, once selected in this project using optimal network design modeling (Chapter …) and logistical adequacy criteria, would form a backbone observing network, guaranteed to become the future operational network. Determining accurately the time and space distribution of greenhouse fluxes at a high resolution requires however a much larger number of associated regional sites (see also Figure 1). This implies that strategic choice need to be made of having either a few main sites at which the largest number of core parameters will be measured operationally with the highest precision or a larger number of associated regional sites, at which measuring the full list of parameters at the highest precision will not be mandatory. The associated sites would then form the basis of the Regional Observing Network. The ICOS could support the Regional Observing Network through common data management systems, powerful outreach tools, and access to cheap and low maintenance sensors and standards to ensure that these sites can be maintained with a minimum baseline funding.

The impending launch of satellites specially targeted at the global carbon cycle, especially the atmospheric reservoir, could provide a quantum leap in our ability to diagnose the behaviour of this cycle at global and regional scales.  The two currently proposed missions, OCO and GOSAT, are experimental missions designed to test the technology.  These tests will require the closest possible integration with ground-based remote sensing and in-situ measurements.  This requires both sets of data to be “integration ready”.

As is the case for ground-based flask data, satellite measurements of atmospheric gas concentrations provide no direct information on carbon exchange processes, but can constrain atmospheric transport models whose boundary conditions are the surface fluxes we wish to know. In contrast, other satellite measurements can provide information on relevant surface properties (albedo, LAI )and processes (fPAR), but the connection to carbon exchange estimates is normally indirect, and can only be achieved by interfacing them with an ecosystem model. Effective use of the data requires that this interface be well-founded. Chapter 11explores how this can be achieved through increased use of dat assimilation techniques..

The successful deployment and long-term operational viability of such an ICOS also requires central facilities for data acquisition and quality control. Integration with other data sources such as those from forest inventory, soil carbon inventories, agricultural data, land use history and accurate regional fossil fuel estimates is necessary to make such a system work to its full capacity. Again, this is far from trivial. The data streams of the various components, that show great variability temporal and spatial resolution in such a system need to me merged in a single or multiple coherent data assimilation network. So far, only prototypes of such systems exist (Chapter.., e.g., Peeters et al, 2006).

Figure 1. Existing European research network of ecosystem observation sites (left) and atmospheric concentration (right) among which the ICOS main sites and associated sites will be selected and essential new sites implemented.

Left: white triangles are CARBOEUROPE flux sites and black triangles flux sites from other projects. Right: Stars are regular lower troposphere profiling by aircraft, diamonds the continuous CO2, CH4, CO, SF6, N2O measurements on tall towers. Diamonds in grey: continuous measurements on intermediate size towers. Circles: continuous surface stations. Triangles: flask sampling sites.

References

Denman, K.L., G. Brasseur, A. Chidthaisong, P. Ciais, P.M. Cox, R.E. Dickinson, D. Hauglustaine, C. Heinze, E. Holland, D. Jacob, U. Lohmann, S Ramachandran, P.L. da Silva Dias, S.C. Wofsy and X. Zhang, 2007: Couplings Between Changes in the Climate System and Biogeochemistry. In: Climate Change 2007: The Physical Science Basis.  Contribution of Working Group I to the Fourth Assessment Report of the Intergovernmental Panel on Climate Change [Solomon, S., D. Qin, M. Manning, Z. Chen, M. Marquis, K.B. Averyt, M.Tignor and H.L. Miller (eds.)]. Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA.

Peters, W. et al. (2005), An ensemble data assimilation system to estimate CO

surface fluxes from atmospheric trace gas observations, J. Geophys. Res.,

110(D24304), doi:10.1029/2005JD006157.

Raupach, et al., 2007. Global and regional drivers of accelerating CO2 emissions. PNAS doi:10.1073/pnas.0700609104

Janssens, I.A. et al. 2003. Europe's Terrestrial Biosphere Absorbs 7 to 12% of European Anthropogenic CO2 Emissions. Science, 300(5625): 1538-1542.

Pacala, S.W. et al. 2001. Consistent Land- and Atmosphere-Based U.S. Carbon Sink Estimates. Science, 292(5525): 2316-2320.